3GPP TSG-RAN WG4 Meeting #112	R4-2412643

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on RRM requirements for SL positioning and CPP

Source: 	Huawei, HiSilicon

Agenda item:	6.1.1.2

Document for:	Discussion

Introduction

Maintenance of RRM core requirements for SL positioning and CPP are discussed in RAN4#111, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- SL positioning

- CPP

In this paper, we will provide our views on remaining issues in core requirements for SL positioning and CPP.

Discussion

SL positioning

Issue 1-1-3: End point of SL-PRS based RSTD measurement period requirements Proposals: - Option 1: - Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

In our understanding, the TSL RSTD,Total in current requirement is for per TX UE or per resource, and it is same as other SL PRS measurement e.g. SL Rx-Tx that is performed over only one TX UE. Note that this is different from Uu, where TRSTD,Total is the measurement period for all configured resources.

The SL RSTD measurement shall be performed during the measurement period TSL RSTD,Total is defined as: TSL RSTD,Total=s=1STSL RSTD,effect,s , where S is the number of samples per measured link, defined below: S = 1 for SL-PRS bandwidth>48 PRBs, S = 4 for SL-PRS bandwidth≤48 PRBs, For each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities [Components 2 and 3 of FG 41-1-1], TSL RSTD,effect,s is defined as:

Therefore, we suggest to update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs. A TP is provided in our draftCR.

Proposal 1: Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

Issue 1-1-4: Impact of Uu link connect Proposals: - Option 1: - RAN4 not to define any impact of Uu link connection on the measurement period. - Option 2: - It is clarified in TS 38.133 for the SL-PRS based measurements in the introduction section 12A.1: - NOTE 3: When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all the requirements specified in Clause 6, assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrupt the SL positioning measurements or SL-PRS transmissions in order to meet the measurement requirements specified in Clause 6.

For impact of Uu link, we do not support option 2. Option 2 is different from the existing note as it is accounting clause 6 while existing note Clause 9. Clause 9 is for measurement which is periodic, and we understand this is the reason behind the existing note. However, clause 6 is for mobility which is one shot, and there are other one shot Uu procedures that may impact SL, e.g. SCell addition/release, DRX ON/OFF, BWP switch, etc. It is unreasonable to only account the mobility procedure in clause 6 as this may cause confusion about other one shot procedures.

In our view, RAN4 should either define generic rule regarding the impact of all one shot procedures in Uu, or not define anything as for SL Communication. We slightly prefer the latter because the impact of Uu should be same for SL Communication and SL Positioning. Defining an extra rule for SL Positioning may cause confusion about the impact of one shot procedures in Uu on SL Communication.

Proposal 2: RAN4 not to define any impact of Uu link connection on the measurement period.

In RAN4#110bis meeting, RAN4 discussed whether to keep dedicated requirements for SL PRS-RSRP(P). There was no consensus and RAN4 sent LS [2] to clarify with RAN1/2.

In May meeting, RAN1 provided the following response [3].

RAN1 confirms requesting and/or reporting only SL PRS-RSRP or SL PRS-RSRPP without any other SL positioning measurements is not supported. In addition, RAN1 would like to inform RAN4 and RAN2 of the following with regards to the mapping between the mandatory measurement and the targeted SL positioning method, ● SL-RSTD is mandatory for SL-TDOA ● SL RTOA is mandatory for SL-TOA ● SL UE Rx - Tx time difference is mandatory for SL-RTT ● at least one of SL AoA/ZoA should be reported for SL-AoA

In May meeting, RAN2 provided the following response [4].

RAN2 thanks RAN4 for the LS regarding SLPP measurement for SL-AOA. During the discussion in RAN2#126, the following agreement was reached in RAN2: Agreements: At least one angular measurement (AoA/ZoA) is mandatory in the measurement report for SL-AoA positioning. Hence, regarding the following question from RAN4, RAN4 respectfully asks RAN2 to clarify: - whether SL PRS-RSRP or SL PRS-RSRPP can be requested and/or reported standalone specifically for SL-AoA positioning.. RAN2 agreed that SL PRS-RSRP or SL PRS-RSRPP cannot be requested and/or reported standalone for SL-AoA positioning.

From both RAN1 and RAN2 responses, it can be seen that SL PRS-RSRP(P) cannot be used as stand-alone measurement for SL positioning, and they must be reported together with other SL PRS measurement which are mandatory measurement for the corresponding to the SL positioning method.

We therefore suggest to remove the dedicated measurement period requirements for SL PRS-RSRP(P), as the measurement of SL PRS-RSRP(P) should be same as the mandatory measurement for the corresponding to the SL positioning method.

Proposal 3: Remove dedicated measurement period requirements for SL PRS-RSRP(P).

CPP

Issue 2-1-1: Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD Agreements: For UE configured for CPP measurement with legacy measurement with multiple PFLs, when LMF does not configure measurement time window(s) for a PFL - UE performs DL RSCPD/RSCP measurement on a single PFL that is common between the reference TRP and the target TRP. - Existing requirements for RSTD/UE Rx-Tx without time window apply. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and does not support FG 41-2-3: - UE performs DL RSCPD/RSCP measurement on the indicated PFL by the network. - Existing legacy requirements without time window apply for both CP measurements and legacy measurements. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3, but does not support FG 41-2-8/41-2-9: - Existing legacy requirements without time window apply for legacy measurements. - CPP measurement requirements apply for CP measurements for the PRS resources associated to the time window have occasions only within the measurement time window. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3 and FG 41-2-8/41-2-9: - CPP measurement requirements apply for both legacy measurements and CP measurements. FFS: Whether and/or how to define the measurement period requirements for DL RSCPD/RSCP measurements when aperiodic time window is configured.

RAN4 has settled down the UE requirements for different UE capabilities related to the time window, and one remaining issue is the aperiodic time window.

In last meeting, some companies suggested that UE is assumed to suspend all PRS measurements until it has completed the measurement of PRS resources in the time window. In our view, this proposal is reasonable as it avoids the impact between periodic and aperiodic measurements, and effectively prioritizes the aperiodic measurement which is critical for CPP. Otherwise, UE may need to interrupt ongoing periodic measurement to perform the aperiodic measurement, or UE may miss the aperiodic time window due to ongoing periodic measurement, both of which are not desirable.

Following the proposal, the requirements for CPP measurement should be the sum of the time from the start of measurement period to the time window and the processing time. After UE finishes processing resources in the time window for CPP measurement, it should start RSTD or Rx-Tx measurement as in legacy way. Note that above only applies when UE supports CPP measurement in the time window.

Proposal 4: When aperiodic time window is configured and UE support FG 41-2-3,

- the measurement period for CPP is Tuncertainty+Ti,

- the measurement period for RSTD/Rx-Tx is Tuncertainty+maxTeffective,i+Tlegacy,

- Tuncertainty is the time from the start of the measurement to the start of the time window,

- Ti is the processing time for PFL i,

- Tlegacy is the legacy requirements for RSTD and Rx-Tx.

Issue 2-1-2: Measurement period requirements for DL RSCP/DL RSCPD with multiple PFLs configured Proposals: - Option 1: - When multiple PFLs are configured in the assistance data and DL RSCPD is requested with RSTD, the measurement period is given by TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation. - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window. - When multiple PFLs are configured in the assistance data and DL RSCP is requested with UE Rx-Tx, the measurement period is given by TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation. - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window. - Option 2: - When multiple PFLs are configured where RSCPD is supposed to be measured in PFL j, support option 1 with the following updates (taking RSCPD measurement as the example) - For UE supporting both FG 41-2-3 and FG 42-2-8，TRSCPD with RSTD, j is the measurement period for both RSCPD and RSTD in PFL j by taking time window into account. - For UE supporting FG 41-2-3 only, TRSCPD with RSTD, j=max(TRSCPD, j,TRSTD, j), where TRSCPD, j is the measurement period for RSCPD in PFL j by taking time window into account, and T RSTD, j is the legacy measurement period for RSTD in PFL j without time window. - For UE not supporting FG 41-2-2 or FG 41-2-8, TRSCPD with RSTD, j=TRSTD, j.

We do not see option 1 as necessary since the PRS measurement requirements are already defined per PFL, and RAN4 only needs to clarify in the requirements that the periodic time window only impacts the measurement of the PFL associated with the time window. Option 2 was already addressed last meeting.

Proposal 5: RAN4 to clarify that the application of the periodic time window is limited to the PFL indicated for CPP measurement.

Issue 2-1-3: The impact of carrier frequency offset Proposals: - Option 1: - The accuracy requirements for CP measurement apply provided that the two PRS resources for calculating RSCPD or relative RSCP are located in the same set of symbols after accounting for expected RSTD. - Option 2: - The carrier phase definition should be clarified to indicate the carrier phase is defined at a specific location within the slot. - Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time for the carrier phase measurement. - Define the referred carrier phase difference as the difference between the referred carrier phase measurements. - Define the same common reference time for the UE and the PRU. - The UE and the PRU report either the referred carrier phase measurements or the carrier phase difference measurements computed using the referred carrier phase measurements. - Option 3: - RAN4 not to define enhancements to mitigate impact of carrier frequency offset on CPP measurements in Rel-18. - RAN4 to define an applicability condition for DL RSCPD accuracy requirements based on the time proximity between the target and reference PRS resources used to calculate the DL RSCPD measurement. - DL RSCPD accuracy requirements are defined and apply when the target and reference PRS resources are measured in the same time slot. FFS the maximum time separation between the resources within a slot. - RSCPD accuracy derived from RAN4 simulations applies assuming zero carrier frequency error at the UE and TRPs.

We do not support RAN4 to introduce enhancements related to carrier frequency offset as option 2.

- The proposed definition requires UE to estimate the carrier frequency offset between UE and a TRP. However, we understand UE does not have an accurate estimate of this frequency error, as long as it meets the 0.1ppm requirements defined in 38.101.

- Each carrier phase measurement is associated with a time stamp which can be down to OFDM symbol level. It means UE can already reference a carrier phase measurement to an OFMD symbol and report this information to LMF.

- The proposed enhancement is in RAN1 scope rather than RAN4, and the proposed solution does not impact RAN4 core requirements. Besides, the core part of the WI has been closed for half a year.

We support RAN4 to discuss how to account for carrier frequency offset in the accuracy requirements as in option 1 or option 3. More detailed proposal can be found in our companion paper for the Perf part.

Proposal 6: RAN4 not to introduce enhancements related to carrier frequency offset. Instead, RAN4 to account for the impact of carrier frequency offset in the CPP accuracy requirements.

Conclusions

In this paper we provided our views on remaining issues in core requirements for SL positioning and CPP.

Proposal 1: Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

Proposal 2: RAN4 not to define any impact of Uu link connection on the measurement period.

Proposal 3: Remove dedicated measurement period requirements for SL PRS-RSRP(P).

Proposal 4: When aperiodic time window is configured and UE support FG 41-2-3,

- the measurement period for CPP is Tuncertainty+Ti,

- the measurement period for RSTD/Rx-Tx is Tuncertainty+maxTeffective,i+Tlegacy,

- Tuncertainty is the time from the start of the measurement to the start of the time window,

- Ti is the processing time for PFL i,

- Tlegacy is the legacy requirements for RSTD and Rx-Tx.

Proposal 5: RAN4 to clarify that the application of the periodic time window is limited to the PFL indicated for CPP measurement.

Proposal 6: RAN4 not to introduce enhancements related to carrier frequency offset. Instead, RAN4 to account for the impact of carrier frequency offset in the CPP accuracy requirements.

Reference

- R4-2410193, WF on SL positioning and carrier phase positioning, CATT

- R4-2406386, Reply LS on SL positioning measurement, RAN4

- R1-2405511, Reply LS on SL positioning measurement, RAN1

- R2-2405872, Reply LS on SL positioning measurement, RAN2