3GPP TSG-RAN4 Meeting #112	R4-2412644

# Maastricht, The Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM requirements for SL positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - SL RSTD requirements should reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs. - There is no need to define dedicated measurement period for SL PRS-RSRP(P) as they are not stand-alone measurement. - In RAN4#111, it is agreed that for synchronization reference source change occurring at Tx side, measurement accuracy requirements do not apply, so the corresponding note should be removed to accuracy clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs. - Void the dedicated measurement period for SL PRS-RSRP(P), and add in the requirements for other SL PRS measurements that SL PRS-RSRP(P) are measured over same measurement period. - Remove the note related to the synchronization reference source change occurring at Tx side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirements for SL PRS measurement are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12A.2.5, 12A.3, 12A.4.5, 12A.5, 12A.6.5, 12A.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 12A.2.5	Measurements Period Requirements

When the physical layer receives the last of SL-TDOA-ProvideAssistanceData and SL-TDOA-RequestLocationInformation from LMF or another UE via SLPP [37], the UE shall be able to perform multiple SL RSTD measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in Clause 12A.2.3), with each SL RSTD measurement based on  SL-PRS from the reference UE and SL-PRS from another anchor UE, as defined in TS 38.215 [4]. The SL RSTD measurement shall be performed during the measurement period TSL RSTD,Total.

The starting point of the measurement period is the earlier between starting points for TSL RSTD,effect,s,ref and TSL RSTD,effect,s,tar, and the ending point of the measurement period is the later between the ending points for TSL RSTD,effect,s,ref and TSL RSTD,effect,s,tar, where TSL RSTD,effect,s,ref and TSL RSTD,effect,s,tar are the measurement periods for the SL-PRS from the SL RSTD reference UE, which is selected by the UE, and SL-PRS from another anchor UE, respectively.

For each measured UE, the measurement period TSL RSTD,Link is defined as:

TSL RSTD,LinkTotal=s=1STSL RSTD,effect,s  ,

where

S is the number of samples per measured link, defined below:

S = 1 for SL-PRS bandwidth>48 PRBs,

S = 4 for SL-PRS bandwidth≤48 PRBs, and

for each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities [Components 2 and 3 of FG 41-1-1], TSL RSTD,effect,s is defined as:

TSL RSTD,effect,s=ts+1-ts , for s<S, where ts+1 and ts are the beginning of the first slot  of SL-PRS sample s+1 and SL-PRS sample s, respectively,

TSL RSTD,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RSTD measurement,

ΔSLproc is the processing time given by the UE capability in [Components 4 of FG 41-1-1].

[A UE may drop one or more SL PRS measurement samples if the number of active slots and number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities in [FG 41-1-1]. For a single-sample measurement, the whole measurement may not be performed.]

If the synchronization reference source changes during TSL RSTD,Total at the measuring UE, while the UE is performing the SL RSTD measurement, then the UE shall restart the SL RSTD measurement after the synchronization reference source change and shall send the measurement report during a measurement period, which can be longer than TSL RSTD,Total.

TSL RSTD,Total

The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL RSTD,Total. Otherwise, the measurement period can be longer.

The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL RSTD,Total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-TDOA, SL RSTD and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

<End of Change 1>

<Start of Change 2>

## 12A.3	Void

TSL PRS-RSRP,Total

TSL PRS-RSRP,Total=s=1STSL PRS-RSRP,effect,s

S

S

TSL PRS-RSRP,effect,s

TSL PRS-RSRP,effect,s=ts+1-tsts+1ts

TSL PRS-RSRP,effect,s=Tdur,s+ΔSLproc ,

Tdur,s

ΔSLproc

TSL PRS-RSRP,TotalTSL PRS-RSRP,Total

TSL PRS-RSRP, Total

TSL PRS-RSRP,Total

TSL PRS-RSRP,Total

<End of Change 2>

<Start of Change 3>

### 12A.4.5	Measurement Period Requirements

When the physical layer receives SL-RTT-ProvideAssistanceData message from SL-RTT-RequestLocationInformation message from LMF or another UE via SLPP [37], the UE shall be able to perform multiple SL Rx-Tx time difference measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in Clause 12A.4.3), as defined in TS 38.215 [4]. For each individual SL-PRS resource measured by a UE, the SL Rx-Tx time difference measurement is performed during TSL Rx-Tx,total defined as:

TSL Rx-Tx,total=s=1STSL Rx-Tx,effect,s+Tuncertain ,

where,

S is the number of samples for a single SL Rx-Tx measurement defined below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth≤48 PRBs.

For SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities [Components 2 and 3 of FG 41-1-1], TSL Rx-Tx,effect,s is defined as:

TSL Rx-Tx,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the start of the s-th and (s+1)-th slot of SL-PRS samples s and SL-PRS samples s+1, respectively.

TSL Rx-Tx,effect,s=Tdur,s+ΔSLproc , for s = S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of SL Rx-Tx measurement.

ΔSLprocis the processing time as indicated via capability [component 4 of FG 41-1-1] of the UE performing the SL Rx-Tx time difference measurement.

Tuncertain is defined as below:

If the UE reports the transmission timestamp of a SL PRS as defined in TS 38.215 [4], and the SL PRS transmission occurs after the SL PRS reception used to derive the measurement, Tuncertain is the additional time delay from the SL PRS reception until the actual SL PRS transmission.

Otherwise, Tuncertain=0.

[A UE may drop one or more SL PRS measurement samples if the number of active slots and number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities in [FG 41-1-1].

- For a single-sample measurement, the whole measurement may not be performed.]

If the synchronization reference source changes during TSL Rx-Tx,total at the measuring UE, while the measuring UE is performing the SL Rx-Tx time difference measurement, then the measuring UE shall restart the SL Rx-Tx time difference measurement and shall send the measurement report during a measurement period, which can be longer than TSL Rx-Tx,total.

TSL Rx-Tx,total

The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL Rx-Tx,total. Otherwise, the measurement period can be longer.

The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL Rx-Tx,total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-RTT, SL Rx-Tx time difference and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

<End of Change 3>

<Start of Change 4>

12A.5	Void

<End of Change 4>

<Start of Change 5>

### 12A.6.5	Measurement Period Requirements

When the physical layer receives the last of SL-AoA-ProvideAssistanceData message from SL-AoA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure multiple SL AoA measurements based on SL-PRS from one or more other SL UEs (up to the UE capapbility specified in 12A.6.3), as defined in TS 38.215 [4]. The SL AoA measurement shall be performedduring the measurement period TSL AoA,total defined as:

TSL AoA,total=s=1STSL AoA, effect,s ,

where,

S is the number of samples for the SL AoA measurement, defined as below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth ≤ 48 PRBs,

For each SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities [Components 2 and 3 of FG 41-1-1]. TSL AoA,effect,s is defined as below,

TSL AoA,effect,s=ts+1-ts  for s < S, where ts and ts+1 are the beginning of the slots of SL-PRS sample s and SL-PRS sample s+1, respectively.

TSL AoA,effect,s=Tdur,s+ΔSLproc for s = S,

Tdur,s is the duration of slot carrying SL-PRS sample s of the SL AoA measurement,

ΔSLproc is the processing time indicated via UE capability in [component 4 of FG 41-1-1] of the UE performing SL AoA measurement.

[A UE may drop one or more SL PRS measurement samples if the number of active slots and number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities in [FG 41-1-1].

- For a single-sample measurement, the whole measurement may not be performed.]

If the synchronization reference source of the measuring UE changes during TSL AoA,total, while the UE is performing the SL AoA measurements, then the measuring UE shall continue performing the SL AoA measurement after the synchronization reference source change, while meeting the requirements in this clause.

TSL AoA,total

The requirements in this clause apply provided that no SL-PRS symbols for the SL AoA measurement are dropped due to e.g., the selection or reselection of synchronization reference source according to clause 12.4 during the measurement period. Otherwise, the measurement period can be extended.

The requirements in this clause, apply provided that reception of slots containing SL-PRS for the SL AoA measurement is not interrupted e.g., due to network coverage change. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be extended.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-AoA, SL AoA and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

<End of Change 5>

<Start of Change 6>

12A.7.5	Measurement Period Requirements

When the physical layer receives the last of SL-RTOA-ProvideAssistanceData message from SL-RTOA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure multiple SL RTOA measurements based on SL-PRS from one or more other SL UEs (up to the UE capapbility specified in 12A.7.3), as defined in TS 38.215 [4]. The SL RTOA measurement shall be performed during TSL RTOA,total defined as:

TSL RTOA,total=s=1STSL RTOA, effect,s ,

where,

S is the number of samples for the SL RTOA measurement, defined as below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth ≤ 48 PRBs,

For SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities [Components 2 and 3 of FG 41-1-1]. TSL RTOA,effect,s is defined as below,

TSL RTOA,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the beginning of the slots of SL-PRS sample s and SL-PRS sample s+1, respectively,

TSL RTOA,effect,s=Tdur,s+ΔSLproc for s = S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RTOA measurement.

ΔSLprocis the processing time indicated via capability in [component 4 of FG 41-1-1] of the UE performing SL RTOA measurement.

If the synchronization reference source of the measuring UE changes during TSL RTOA,total, while the UE is performing the SL RTOA measurements, then the measuring UE shall restart the SL RTOA measurement after the synchronization reference source change and shall send the measurement report during a measurement period, which can be longer than TSL RTOA,total.

TSL RTOA,Total

The requirements in this clause apply provided that no SL-PRS symbols for the SL RTOA measurement that are dropped due to e.g., the selection or reselection of synchronization reference source according to clause 12.4 during the measurement period. Otherwise, the measurement period can be extended.

The requirements in this clause apply, provided that reception of slots containing SL-PRS for the SL RTOA measurement is not interrupted due to network coverage change. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

If SL PRS-RSRP and/or SL PRS-RSRPP are configured for SL-TOA, SL RTOA and SL PRS-RSRP and/or SL PRS-RSRPP are performed over the same measurement period.

<End of Change 6>