3GPP TSG-RAN WG4 Meeting #112	R4-2412647

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on RedCap positioning and PRS/SRS BWA

Source: 	Huawei, HiSilicon

Agenda item:	6.1.1.4

Document for:	Discussion

Introduction

Maintenance of RRM core requirements for RedCap positioning and PRS/SRS BWA are discussed in RAN4#111, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- RedCap positioning

- PRS/SRS CA

In this paper, we will provide our views on remaining issues in core requirements for RedCap positioning and PRS/SRS BWA.

Discussion

RedCap positioning

In current spec, the number of hops that UE is required to perform within an MG occasion is defined based on number of PRS repetitions.

The number of hops within a single MG occasion Nhop is defined as Nhop=minNhops,effect,Nhop,max where - Nhop,max is the maximum number of Rx hops signaled in the UE capability (FG 41-5-1) - Nhops,effect is the effective number of Rx hops within a MG instance, - Nhops,effect=2*NrepPRS, if Nhopsslot = 2, - Nhops,effect=NrepPRS, if Nhopsslot = 1, - Nhops,effect=NrepPRS, if Nhopsslot = 1/2 and MrepPRS >1, - Nhops,effect=NrepPRS-12+1, if Nhopsslot = 1/2 and MrepPRS =1, - where NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times, MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap and Nhopsslot is the applicable number of hops per slot as defined in Table 9.9A.4.8-1.

This is however incorrect because in 38.133 PRS repetition is defined on comb pattern basis.

A PRS resource is considered to be fully (partially) overlapped with measurement gaps if all (some) of its instances are overlapped with a measurement gap occasion. A PRS resource instance is considered to be overlapped with measurement gap occasion if the minimum number of unmuted repetitions of the instance taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD is fully covered by the MGL excluding RF switching time, where the minimum number is given in the accuracy requirements in clause 10.1.23, 10.1.x1, 10.1.24, 10.1.25, 10.1.x2 and 10.1.38 for RSTD, RSTD based on aggregated PRS resources from multiple PFLs, PRS-RSRP, UE Rx-Tx time difference and UE Rx-Tx time difference based on aggregated PRS resources and aggregated SRS resources from multiple PFLs, PRS-RSRPP, respectively.

For example, assuming there are 4 slots with PRS repetitions within an MG occasion, and the PRS resource is configured with comb size 2 and 4 symbols. In this case there are 2 PRS repetitions per slot and in total 8 repetitions within the MG occasion (NrepPRS=8). If the RF retuning time is 3 symbols which leads to Nhopsslot=1, UE is assumed to measure 1 repetition per slot and the effective number of hops within the MG occasion should be 4. However, based on current spec it is calculated as 8.

To resolve the issue, one way is to define an upper bound on number of repetitions per hop, i.e.

- if Nhopsslot = 2, the upper bound is 2

- if Nhopsslot = 1, the upper bound is 1

- if Nhopsslot = 1/2, the upper bound is 1

It is noted that the upper bound is for number of repetitions per hop instead of number of repetitions per slot because the starting point of a hop is not necessarily aligned with slot boundary.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 9.9A.4.8-1, and RRTFH is the retuning time between Rx hops as reported by UE in dl-PRS-MeasurementWithRxFH-RRC-Connected via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

The other way to resolve the issue is to define a new term for slot level PRS repetition. We do not have strong view but slightly prefer the first approach considering another muting issue raised up last meeting [2].

Issue 2-1-1-1: Clarification of parameters for number of RX FH within MG in RRC_CONNECTED state. - Proposals - Option 1: OPPO - Clarify that NrepPRS is the number of unmuted PRS repetitions (i.e. bit 1 signalled by dl-PRS-MutingOption2) within the MG occasion excluding the gap retuning times. - If Nhopsslot = 1/2 and MrepPRS = 1, the effective number of Rx hops within a MG instance is defined as the number of PRS repetitions within MG occasion after applying the logic AND operation between bitmap-MutingOption2 and bitmap-default, where: - bitmap-MutingOption2 is the bitmap given by higher-layer parameter dl-PRS-MutingOption2. - bitmap-default is “101010…” with the same length as bitmap-MutingOption2. - Recommended WF - Discuss the option(s).

We understand the issue is valid, and a generic approach to resolve both issues (definition of PRS repetition and consideration of option 2 muting) is to simply count the total number of unmuted PRS repetitions with upper bound that overlap with the sampling window of each hop. In this way, we can not only re-use the existing definition of PRS repetition, but also deal with different option 2 muting patterns.

Proposal 1: Update the requirement on Nhops,effect as

Nhops,effect=hminNrep,hPRS,Nrepmax

where

- h is the index of hops within an MG instance

- Nrep,hPRS is the number of unmuted PRS repetitions that overlaps with the sampling window of hop h,

- Nrepmax=2 if Nhopsslot = 2, otherwise Nrepmax=1.

Another issue in requirements for FH that has been missed in earlier discussions is the application of N3 (component 4 of FG 41-5-1). In our understanding, it is the maximum duration for which UE can buffer PRS samples across multiple hops within an MG occasion, and it will limit the number of hops UE can actually take (Nhop). For example, consider one PRS resource has 6 slot level repetitions within an MG occasion. Without considering N3, the number of hops UE can take is 6. However, if N3=4ms, UE would only be able to take 4 hops over the first 4 repetitions. Note that for one PRS resource, UE cannot take the first 4 hops within one MG occasion and the other 2 hops in the next MG occasion, so scaling the measurement period with Lprs/N3 should not apply for a single resource, and the number of hops within an MG occasion should be limited by N3/Lprs,per-hop.

Proposal 2: Update the requirements on Nhop by adding an upper bound for N3 capability

Nhop=minNhops,effect,N3Lper-hop,Nhop,max

Finally, the definition of sampling duration Lprs also needs update. In non-FH requirements, Lprs is the PRS resource duration across multiple MG occasions within Tavailable, e.g. UE may measure PRS resource #1 in the first MG occasion and PRS resource #2 in the second MG occasion, and Lprs is the total duration of both PRS resources #1 and #2. However, this is not supported for FH in the current requirements since Nhop * Lprs,per-hop is the PRS resource occasion for only one MG occasion. To reflect that UE should measure different PRS resources in different MG occasions within Tavailable, the definition of Lprs should be updated.

Proposal 3: Update the requirements on Lprs by considering measurement of different PRS resources in different MG occasions

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,j

PRS/SRS CA

Interruption delay requirement for SRS aggregation Agreement - Define interruption requirements for SRS transmission with BW aggregation on CC without PUSCH/PUCCH. - Reuse the framework of SRS carrier and/or antenna switching based interruption requirements as the starting point - The interruption length is FFS

RAN4 agreed to define interruption requirements for SRS BWA on CC without PUCCH/PUSCH, and to re-use the framework from SRS carrier and/or antenna switching.

For both SRS carrier and/or antenna switching, RAN4 has assumed that SRS is transmitted only if the SRS switching does not collide with other UL transmission or DL reception with higher priority as defined by RAN1 on victim cells, and does not collide with any NR L3 or L1 measurement on victim cells. We suggest to re-use the principle for SRS BWA. Here, we understand in time domain the “SRS switching” means SRS duration plus guard period before and after.

RAN1 has defined 41-1-9 for UE to indicate which bands are affected by the SRS BWA. This is similar to SRS antenna switching, and the victim cells of SRS BWA should be defined based on this capability.

On the interruption length, for both SRS carrier switching and antenna switching, the interruption length is defined based on the SRS duration and 2 times the RF switching time (for SRS carrier switching interruption is defined separately for switching to and switching from, technically it is same as 2 times the RF switching time). We suggest to re-use the same approach for BWA, where the RF switching time is already defined as guard period by RAN1. The length of guard period is indicated by component 9 of 41-4-7, and the candidate values are {0, 30, 100, 140, 200}us.

The next issue is whether to define interruption on symbol or slot level. We suggest to define it on symbol level because it is a UE requirement, and how it impacts the scheduling e.g. whether to stop scheduling on a few symbols, a whole slot containing the SRS, or even multiple slots considering RTD, can be left to NW implementation. From UE perspective, we do not see any restriction to define symbol level interruption.

Proposal 4: For interruption requirements for SRS BWA on CC without PUCCH/PUSCH,

- SRS is transmitted only if SRS duration plus guard period before and after does not collide with other UL transmission or DL reception with higher priority as defined by RAN1 on victim cells

- SRS is transmitted only if SRS duration plus guard period before and after does not collide with any NR L3 or L1 measurement on victim cells

- The victim cells are given by 41-4-9

- The interruption length is defined on symbol level as 2*guard period + SRS duration, where the guard period is indicated via component 9 of 41-4-7

In requirements for PRS BWA, Taggregate_RSTD is defined as follows. There is a margin for UE to switch across different PFL combinations, which is defined as (G-1)*maxTeffect_agg,i. However, using maxTeffect_agg,i means UE has to measure all PFL combinations for aggregated measurement before it can measure single PFLs for non-aggregated measurement (or vice versa), since the switching margin as highlighted in yellow does not allow UE to switch between PFL combinations and single PFLs.

Taggregate_RSTD, total measurement period for RSTD measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as: Taggregate_RSTD=i=1GTRSTD_aggregate,i +(G-1)*maxTeffect_agg,i,

In our view, such restriction is unreasonable, and it should be up to UE implementation to decide the order to measure each PFL combination and each PFL. To allow this flexibility, we suggest to replace maxTeffect_agg,i by Tmargin in current requirements which is for switching between any PFL combination and any single PFL.

Tmargin is delay margin to account for delay between RSTD measurement performed by UE on PFLs that contain PRS resources for aggregation and RSTD measurement performed by UE on PFLs that do not contain PRS resources for aggregation. Tmargin is calculated as max(Teffect,i), by considering both aggregated PFLs and non-aggregated PFLs configured for positioning measurement. Tmargin is only applicable when UE is configured to perform measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation.

Proposal 5: In PRS BWA requirements for Taggregate_RSTD, replace the margin (G-1)*maxTeffect_agg,i as (G-1)*Tmargin.

Conclusions

In this paper we provided our views on remaining issues in core requirements for RedCap positioning and PRS/SRS BWA.

Proposal 1: Update the requirement on Nhops,effect as

Nhops,effect=hminNrep,hPRS,Nrepmax

where

- h is the index of hops within an MG instance

- Nrep,hPRS is the number of unmuted PRS repetitions that overlaps with the sampling window of hop h,

- Nrepmax=2 if Nhopsslot = 2, otherwise Nrepmax=1.

Proposal 2: Update the requirements on Nhop by adding an upper bound for N3 capability

Nhop=minNhops,effect,N3Lper-hop,Nhop,max

Proposal 3: Update the requirements on Lprs by considering measurement of different PRS resources in different MG occasions

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,j

Proposal 4: For interruption requirements for SRS BWA on CC without PUCCH/PUSCH,

- SRS is transmitted only if SRS duration plus guard period before and after does not collide with other UL transmission or DL reception with higher priority as defined by RAN1 on victim cells

- SRS is transmitted only if SRS duration plus guard period before and after does not collide with any NR L3 or L1 measurement on victim cells

- The victim cells are given by 41-4-9

- The interruption length is defined on symbol level as 2*guard period + SRS duration, where the guard period is indicated via component 9 of 41-4-7

Proposal 5: In PRS BWA requirements for Taggregate_RSTD, replace the margin (G-1)*maxTeffect_agg,i as (G-1)*Tmargin.

Reference

- R4-2410353, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson

- R4-2407878, Discission on core requirements for RedCap positioning and PRS bandwidth aggregation, OPPO