3GPP TSG-RAN WG4 Meeting #112	R4-2412651

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	On performance requirements for SL positioning

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2.2

Document for:	Discussion

Introduction

RRM performance requirements for SL positioning are discussed in RAN4#111, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- Accuracy requirements

- Test cases

In this paper, we will provide our views on RRM performance requirements for SL positioning.

Discussion

Accuracy requirements

Issue 1-2-2: Margin for RF calibration Agreements: - Define the RF calibration margin for SL RSTD measurements in FR1 using the following structure: PRS BW (RB number); Margin (Tc). SCS=15kHz; SCS=30kHz; SCS=60kHz. ≥ 48; ≥ 24; N/A; Z1. ≥ 96; ≥ 48; ≥ 24; Z2. - Define the RF calibration margin for SL Rx-Tx measurements in FR1 using the following structure: [Min(SL PRS Rx BW, SL PRS Tx BW) (RB)]; Margin (Tc). SCS = 15 kHz; SCS = 30 kHz; SCS = 60 kHz. ≥ 48; ≥ 24; N/A; 1. ≥ 96; ≥ 48; ≥ 24; 2. - FFS the detailed values in the above tables. Margins due to frequency drift (Y): - For SL RSTD, the existing margins due to frequency drift for DL PRS can be reused for SL PRS. - Consider time margins due to frequency drift to Rx-Tx time difference accuracy requirement if the UE reports the transmission timestamp of a SL PRS measurement - FFS the exact time margin values, e.g., the ones for RSTD could be reused.

In our view, RF calibration margin for SL RSTD (Z) and SL Rx-Tx (δ) can be re-used from corresponding requirements for Uu positioning. We do not see particular difference in SL positioning related to group delay calibration compared to Uu positioning.

Proposal 1: The RF calibration margin for SL RSTD (Z) and SL Rx-Tx (δ) can be re-used from corresponding requirements for Uu positioning.

On the frequency drift margin for SL Rx-Tx with time stamp, we also believe the Uu margin value for RSTD of 32 Tc corresponding to 160ms time separation can be re-sed. The margin is derived from a 0.1ppm frequency error which is a common requirement for both SL and Uu.

Proposal 2: The frequency drift margin for SL Rx-Tx is defined as Y=32 Tc, provided that the time offset between the SL PRS transmission and reception, which are used for a single SL Rx-Tx estimate, is no greater than 160 ms.

Test cases

Issue 1-2-6: Test case configurations Agreements: - RAN4 will not specify SL positioning test cases with additional path reporting - RAN4 will not specify SL positioning test cases with LoS/NLoS reporting (no corresponding RAN4 requirements to test) - RAN4 will not specify SL positioning test cases with Tx/Rx ARP-ID reporting (no corresponding RAN4 requirements to test) - NR Uu configurations NR Uu configuration; Description. 1; NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode. 2; NR Uu: 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode. 3; NR Uu: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. - SL PRS configurations NR SL configuration; Description. 1; SL: 15 kHz, 10 MHz bandwidth, HD duplex mode. 2; SL: 30 kHz, 10 MHz bandwidth, HD duplex mode. 3; SL: 30 kHz, 20 MHz bandwidth, HD duplex mode. - The number of SL UEs in test cases - The number of SL UEs in the SL measurement delay test cases for SL positioning: - SL RSTD: 4 (1 target rx UE, 3 anchor tx UEs [1 reference anchor UE and 2 other anchor tx UEs)]) - SL Rx-Tx, SL PRS-RSRP/RSRPP, SL AoA: 3 (1 target rx UE, 2 anchor tx UEs), - SL RTOA: [2] (1 target tx UE, 1 anchor rx UE). - The number of SL UEs in the SL measurement accuracy test cases for SL positioning: - SL RSTD: 3 (1 target rx UE, 2 anchor tx UEs [1 reference anchor UE and 1 other anchor tx UEs)]) - SL Rx-Tx, SL PRS-RSRP/RSRPP: 3 (1 target rx UE, 2 anchor tx UEs). - All TX and RX UEs are in coverage and using gNB as sync reference.

In last meeting RAN4 agreed on some test configurations for SL PRS measurement, especially the number of UEs. There are also some other test configurations that need to finalized.

On resource pool types, RAN4 agreed to define TCs for dedicated resource pool and shared resource pool, but it is unclear from the TC list how the resource pool types are considered. To not increase the number of TCs, the pool type can be randomly distributed among different TCs.

For the Es/Iot, we suggest to use 3dB for the test although we suggest -3dB for the accuracy requirements. The reason is that based on demodulation requirements in 38.101, the PSCCH performance is defined at Es/Iot of 4.7dB. If we define the SL PRS test at -6dB, UE may miss the SCI thus not measure SL PRS. Since the test is to verify SL PRS performance, a better Es/Iot should be used to ensure 100% PSCCH decoding.

Proposal 3: For the test configuration,

- Shared and dedicated resource pools are randomly used among different TCs

- Es/Iot: 3dB (to ensure 100% PSCCH decoding)

For SL PRS, RAN4 has simulated the performance for (symbol num, comb size) = (4, 4) and (2, 4). The partial staggering pattern is new for SL PRS and it is meaningful to test it together with full staggering.

For the BW, we suggest to choose one at least for the delay test. We believe 48 RB, which corresponds to 10/20/40MHz for 15/30/60kHz SCS, is a typical configuration. Also, it is associated with one sample, which is more stringent requirement for the delay test. For accuracy tests, we suggest to follow the Uu principle to test two BWs, one small BW of 24 RB and one large BW of 48 RB.

To verify the performance in multiple TX UE scenario (which is typical in real deployment), we suggest o model 2 TX UEs at least in the delay test. Since the UE may process multiple active resources in multiple slots, it is suggested to have two TX UEs transmitted in consecutive slots, and the UE that can process multiple active resources in multiple slots should take both SL PRS TX. To account for UE that cannot process multiple active resources in multiple slots, one TX UE should TX after the UE processing time which can be up to 100ms, and in that case the UE should always take the second SL PRS TX.

Proposal 4: RAN4 to consider the following SL PRS related parameters for the test.

- (symbol num, comb size): (4, 4) and (2, 4)

- BW: 48 RB for delay TCs, 24 and 48 RB for accuracy TCs

- MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 100ms

Issue 1-2-7: Whether to verify accuracy of legacy measurements in SL PRS-RSRP(P) TCs Proposals: - Option 1: - Consider the following rules: - If RAN1/2 confirms that the SL PRS-RSRP(P) is possible to be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy is verified alone in separate section. - If RAN1/2 confirms that the SL PRS-RSRP(P) cannot be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy should be verified with RSTD/RX-TX accuracy together in one section. - Option 2: - Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP. - Define test case to verify accuracy requirements for SL PRS-RSRPP in a separate section. - Option 3: - Support option 1a to define separate sections for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively.

For accuracy TC, we support option 3, i.e. SL PRS-RSRP(P) should have its dedicated accuracy TC. In the TC, UE is requested to report SL PRS-RSRP(P) together with other measurement, but only SL PRS-RSRP(P) accuracy is verified to verdict pass or fail. The reason is that if accuracy for SL PRS-RSRP(P) and other measurement are verified together, it would impose higher requirements to the UE (UE needs to meet both accuracy with 90% rate) compared to how accuracy requirements are derived based on simulation.

Proposal 5: For accuracy TCs for SL PRS-RSRP(P), separate section for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively.

Conclusions

In this paper we provided our views on RRM performance requirements for SL positioning.

Proposal 1: The RF calibration margin for SL RSTD (Z) and SL Rx-Tx (δ) can be re-used from corresponding requirements for Uu positioning.

Proposal 2: The frequency drift margin for SL Rx-Tx is defined as Y=32 Tc, provided that the time offset between the SL PRS transmission and reception, which are used for a single SL Rx-Tx estimate, is no greater than 160 ms.

Proposal 3: For the test configuration,

- Shared and dedicated resource pools are randomly used among different TCs

- Es/Iot: 3dB (to ensure 100% PSCCH decoding)

Proposal 4: RAN4 to consider the following SL PRS related parameters for the test.

- (symbol num, comb size): (4, 4) and (2, 4)

- BW: 48 RB for delay TCs, 24 and 48 RB for accuracy TCs

- MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 100ms

Proposal 5: For accuracy TCs for SL PRS-RSRP(P), separate section for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively.

Reference

- R4-2410193, WF on SL positioning and carrier phase positioning, CATT