3GPP TSG-RAN WG4 Meeting #112	R4-2412653

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	On performance requirements for LPHAP

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2.3

Document for:	Discussion

Introduction

RRM performance requirements for LPHAP are discussed in RAN4#111, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- Configuration of PRS measurement reporting periodicity in TCs

In this paper, we will provide our views on remaining issues in performance requirements for LPHAP.

Discussion

Issue 1-2-2: whether to configure PRS measurement reporting periodicity in the test cases Wayforwards - Option 1: No - Option 2: Yes, configured as 20s (smaller than eDRX cycle)

We can understand the motivation of option 2, i.e. to verify that UE performs periodic reporting during RRC_INACTIVE, which fits use case 6 of LPHAP. However,

- Using periodic reporting will lead to very long test time. To verify the UE reporting behavior, at least 2 reports need to be observed by the TE, and this will cause the test time longer than 40s.

- Using periodic reporting will require change of test procedure. To perform the reporting, UE needs to get into RRC_CONNECTED, and to be released to RRC_INACTIVE after sending the report. This will lead to quite some change to the test procedure in the spec and also the TE implementation.

- UE requirements are same with the two options, i.e. in both cases UE will measure based on DRX cycle. If UE can pass the test with option 1, it is safe to assume the same UE can pass the test with option 2 since the measurement performance is already verified and reporting itself is a functionality.

Based on above considerations, we suggest to go with option 1.

Proposal: Do not configure PRS measurement reporting periodicity in the test cases for LPHAP.

Conclusions

In this paper we provided our views on remaining issues in performance requirements for LPHAP.

Proposal: Do not configure PRS measurement reporting periodicity in the test cases for LPHAP.

Reference

- R4-2410194, WF on LPHAP, Huawei, HiSilicon