3GPP TSG-RAN WG4 Meeting #112	R4-2412655

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	On performance requirements for RedCap positioning

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2.4

Document for:	Discussion

Introduction

RRM performance requirements for RedCap positioning are discussed in RAN4#111, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- Accuracy requirements

- Test cases

In this paper, we will provide our views on remaining issues in performance requirements for RedCap positioning.

Discussion

Accuracy requirements

Bandwidth configuration for accuracy requirement for positioning measurement with RX FH for RedCap positioning Agreement: - Define accuracy requirements for FH case based on at least following BW groups: - 15kHz: per-hop BW ≥ 52 RB, min total BW = [268] RB - 30kHz: per-hop BW ≥ 52 RB, min total BW = [272] RB - 60kHz (FR1): per-hop BW ≥ 24 RB, min total BW = [132] RB - 60kHz (FR2): per-hop BW ≥ 68 RB, min total BW = [264] RB - 120kHz: per-hop BW ≥ 68 RB, min total BW = [264] RB - FFS: additional BW configurations for accuracy requirements, e.g., smaller total BW. Note: per-hop BW is to be converted in the accuracy requirements to BW in MHz and aligned with RAN2 signaling. - For accuracy requirements for FH case, derive accuracy numbers from simulation results. - The requirements apply, provided that the BWtotal defined in clause core requirements is no less than the total BW above.

We do not support to define accuracy requirements for additional BW configurations. On one hand, the performance part of the WI is to be closed in this meeting. Defining additional requirements would require extra time and efforts for additional simulations and calibrations. On the other hand, the motivation of FH is to enlarge the measurement BW, and the most relevant scenario is the one that was agreed last meeting, with largest BW for a single PFL. We acknowledge that in real deployment UE may not be always able to reach the largest BW, but to us the scenarios with smaller total BW are less interesting.

Proposal 1: RAN4 not to define FH accuracy requirements for additional BW configurations.

In last meeting, it was agreed that per-hop BW is to be converted in the accuracy requirements to BW in MHz and aligned with RAN2 signaling. After further checking, we do not think this conversion is needed. Per-hop BW is re-used from R16 FG 13-1, i.e. in R16 the UE capability on PRS BW is indicated in MHz, but the accuracy requirements were defined based on PRS BW in number of RBs and separately for different SCS. We suggest to re-use the same approach for per-hop BW to keep consistent with R16 requirements.

Proposal 2: Per-hop BW is defined in RB number instead of MHz.

Accuracy requirement for RedCap positioning Agreement: - Accuracy requirements with Rx FH are different (more accurate than) from accuracy requirements without Rx FH for the scenario where the total PRS BW after all hops is larger than the PRS BW without Rx FH. - FFS: With some BW configurations, accuracy requirements can be same as legacy for the cases with and without Rx FH for the scenario where the total PRS BW after all hops is equal to the PRS BW without Rx FH, if such exist.

In last meeting, RAN4 agreed to derive accuracy numbers from simulation results for accuracy requirements for FH case, so we do not think there is a need to re-use any existing accuracy requirements for FH.

Technically, the non-FH accuracy requirements are defined up to 20MHz in FR1 and 100MHz in FR2. The FH accuracy requirements will be defined for 50/100MHz in FR1 and 200/400MHz for FR2. It can be seen that there is no such case where the total PRS BW after all hops is equal to the PRS BW without Rx FH.

If we consider accuracy requirements for non-RedCap UE and assume accuracy requirements for FH will be defined for a smaller total BW, there might be a case where total PRS BW after all hops is equal to the PRS BW for non-RedCap UE, but we do not think the accuracy requirements for non-RedCap UE can be re-used for RedCap UE with FH because the phase offset between two hops will degrade the performance. The overlapping RB between hops is introduced to help UE to do phase compensation, but it does not mean the phase offset can be perfectly compensated like a single PFL. Deriving the accuracy numbers from simulation is the most reasonable way to define requirements.

Proposal 3: RAN4 not to re-use non-FH accuracy requirements for FH cases.

Another issue RAN4 needs to discuss for accuracy requirements is the group delay calibration margin. For example, below is the requirement on margin Z for RSTD accuracy for non-RedCap UE.

Table 10.1.23.2-5: Margin for RSTD measurement accuracy in FR1 PRS BW (RB number); Margin (Tc). SCS=15kHz; SCS=30kHz; SCS=60kHz. ≥ 24; N/A; N/A; 120. ≥ 52; ≥ 24; N/A; 72. ≥ 104; ≥ 48; ≥ 24; 36. N/A; ≥ 132; ≥ 64; 16. N/A; N/A; ≥ 132; 12. Table 10.1.23.2-6: Margin for RSTD measurement accuracy in FR2 PRS BW (RB number); Margin (Tc). SCS=60kHz; SCS=120kHz. ≥ 24; N/A; 72. ≥ 64; ≥ 32; 32. ≥ 132; ≥ 64; 16. N/A; ≥ 128; 12.

For non-FH, we understand the existing values for non-RedCap can be re-used for applicable BW, including both 2RX and 1RX. For FH, the calibration error should be limited by the per-hop BW rather than the total BW after FH because the hardware capability, e.g. RF BW, is still limited. On top of this, some additional margin may be needed as there can be additional jitter in the calibration error due to Rx chain working on different frequencies during the measurement. This is similar to the case when reference and target resource for RSTD are in different PFLs. Of course, for FH case, different working frequencies are within a PFL, so the margin may not need to be as large margin for Δ RSTD. We are open to discussing the exact value.

Proposal 4: RAN4 to use the following assumptions for group delay calibration margin for RedCap

- Non-FH: existing values for non-RedCap for the applicable BW

- FH: existing values for non-RedCap for per-hop BW plus an extra margin (value TBD)

Test cases

PRS bandwidth for FH TCs for RedCap positioning Conclusion: Collect feedback from TE vendors.

In last meeting we proposed two options to configure PRS BW in TCs:

- Option 1: consider the BW in test configuration as UE BW and cell BW can be larger

- Option 2: consider PRS are transmitted from non-serving cells (the cell BW of the non-serving cells need to be defined), and BW in test configuration is only for serving cell.

Based offline discussions, we understand option 1 is not aligned with the current assumption from TE side for existing TCs, and there is a preference from TE side on option 2. However, after further checking our view is that option 2 may not work well.

- If the serving cell (20MHz) transmits PRS, then the neighbor cells cannot transmit PRS of 100MHz on the same PFL because PRS BW is common for a PFL. An alternative would be to have neighbor cells to transmit 100MHz PRS on a different PFL, but it means there must be 2 PFLs which will cause some problems. For example, 2-PFL may not be supported by all UEs, the measurement delay for single PFL cannot be verified, and the RSTD accuracy with FH cannot be verified as it limited by the reference PRS resource which is measured without FH.

- If the serving cell (20MHz) does not transmit PRS, then the reference PRS resource must be from a neighbor cell, and UE would need additional time to obtain the timing, including SFN, of the neighbor cell before it can start PRS measurement. There is however no requirement on neighbor cell SFN reading for such cases, so it is not possible to define tests for such a configuration.

We suggest to adopt option 1 although we understand it may cause some efforts at TE side. Technically, option 1 (a 100MHz cell hosting RedCap UE and transmitting PRS) is valid from specification perspective and typical from deployment perspective. It is thus better to support it for testing.

Proposal 5: For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.

PRS RMC for RedCap positioning TCs Agreement: - For small BWs, reuse the existing one. - For larger BWs non-FH: new PRS RMC I to be defined (to be used for measurement accuracy TCs), e.g.: - 104/48/64 RB for 15/30/120kHz, respectively

For RRM testing, RAN4 has defined two sets of PRS RMCs, one with small BW and one with large BW.

- 15kHz: 24 RB and 104 RB

- 30kHz: 24 RB and 132 RB

- 120kHz: 32 RB and 128 RB

RAN4 agreed on PRS RMC for non-FH TCs last meeting. For FH TCs,

- For measurement delay TCs, new RMC with large BW needs to be defined. Existing RMC with large BW are without repetition as existing accuracy requirements for large PRS BW do not require repetition, but repetition is needed for FH.

- For measurement accuracy TCs, in existing TCs both PRS BW are used (in different sub-tests) to verify UE performance with small and large PRS BWs. For FH case, we understand the focus should be on large BW, so only one RMC for the large BW is needed (same RMC as used for the delay TCs).

Proposal 6: For FH TCs, introduce a new PRS RMC with large BW and repetitions.

Conclusions

In this paper we provided our views on remaining issues in performance requirements for RedCap positioning.

Proposal 1: RAN4 not to define FH accuracy requirements for additional BW configurations.

Proposal 2: Per-hop BW is defined in RB number instead of MHz.

Proposal 3: RAN4 not to re-use non-FH accuracy requirements for FH cases.

Proposal 4: RAN4 to use the following assumptions for group delay calibration margin for RedCap

- Non-FH: existing values for non-RedCap for the applicable BW

- FH: existing values for non-RedCap for per-hop BW plus an extra margin (value TBD)

Proposal 5: For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.

Proposal 6: For FH TCs, introduce a new PRS RMC with large BW and repetitions.

Reference

- R4-2410353, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson