3GPP TSG-RAN WG4 Meeting #112	R4-241xxxx

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	On performance requirements for PRS BWA

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2.5

Document for:	Discussion

Introduction

RRM performance requirements for PRS BWA are discussed in RAN4#111, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- Accuracy requirements

- Test cases

In this paper, we will provide our views on remaining issues in performance requirements for PRS BWA.

Discussion

Accuracy requirements

Issue 5-1-1-9: RF calibration margin for positioning measurements based on bandwidth aggregation. - Proposals - Option 1: HW - RAN4 to further discuss the RF calibration margin for RSTD and UE Rx-Tx. - Recommended WF - Discuss the option(s).

The RF calibration margin Z (group delay calibration error for single PFL) for RSTD without PRS BWA are defined in the tables below. The last column is added showing the BW in MHz which is the determining factor of the calibration performance.

Table 10.1.23.2-5: Margin for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) | BW (MHz) |
| --- | --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |  |
| ≥ 24 | N/A | N/A | 120 | 5 |
| ≥ 52 | ≥ 24 | N/A | 72 | 10 |
| ≥ 104 | ≥ 48 | ≥ 24 | 36 | 20 |
| N/A | ≥ 132 | ≥ 64 | 16 | 50 |
| N/A | N/A | ≥ 132 | 12 | 100 |

Table 10.1.23.2-6: Margin for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) | BW (MHz) |
| --- | --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |  |
| ≥ 24 | N/A | 72 | 20 |
| ≥ 64 | ≥ 32 | 32 | 50 |
| ≥ 132 | ≥ 64 | 16 | 100 |
| N/A | ≥ 128 | 12 | 200 |

Table 1 and Table 2 show the per-PFL BW for which accuracy requirements are defined. The last two columns are showing the aggregated BW in MHz with 2-PFL and 3-PFL.

Table 1: BW in MHz for measurement accuracy requirements for PRS BWA in FR1

| Per-PFL PRS BW in RB number (and MHz) |  |  | Aggregated BW in MHz |  |
| --- | --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz | 2 PFL | 3 PFL |
| ≥ 104 (20M) | N/A | N/A | 40M | 60M |
| N/A | ≥ 132 (50M) | ≥ 64 (50M) | 100M | 150M |
| N/A | ≥ 272 (100M) | ≥ 132 (100M) | 200M | 300M |

Table 2: BW in MHz for measurement accuracy requirements for PRS BWA in FR2

| Per-PFL PRS BW in RB number (and MHz) |  | Aggregated BW in MHz |  |
| --- | --- | --- | --- |
| SCS=60kHz | SCS=120kHz | 2 PFL | 3 PFL |
| ≥ 64 (50M) | N/A | 100M | 150M |
| ≥ 132 (100M) | ≥ 64 (100M) | 200M | 300M |
| N/A | ≥ 128 (200M) | 400M | 600M |

In our view, the existing margin for single PFL can be re-used for the aggregated BW in green. For example, for aggregated BW of 40MHz and 60MHz, we can re-use the margin for single PFL for 50MHz. For the aggregated BW in yellow which are larger than the largest BW of single PFL, we suggest RAN4 to further discuss the calibration margin. It is noted that the margin cannot reduce proportionally when the aggregated BW goes up due to hardware limit.

Proposal 1: For group delay calibration margin for RSTD and UE Rx-Tx with PRS BWA, re-use the existing margin for single PFL when the aggregated BW is no larger than largest BW of single PFL. FFS margins for aggregated BW larger than largest BW of single PFL.

Test cases

Issue 5-1-2-2: PRS resource set up for accuracy test cases. - Proposals - Option 1: HW - For accuracy TCs, test one BW based on the largest per-PFL BW in accuracy requirements. Define new PRS RMC for 30kHz SCS with 272 RB. - Option 2: E/// - Test cases for PRS aggregation are defined for 2PFL case only. In the test cases, the existing PRS configuration is reused to configure PRS resources on a per PFL basis. - Recommended WF - Discuss the option(s).

In existing accuracy TCs, two BWs are used to verify the performance with small and large PRS BW. For PRs CA, since the target is to achieve high accuracy with large BW, we suggest to test one BW based on the largest per-PFL BW in accuracy requirements. Existing RMC with 104 RB, 132 RB and 128 RB can be used for 15kHz, 30kHz and 120kHz SCS, respectively.

Proposal 2: For accuracy TCs, test one BW based on the existing PRS RMC with larger BW.

Conclusions

In this paper we provided our views on remaining issues in performance requirements for PRS BWA.

Proposal 1: For group delay calibration margin for RSTD and UE Rx-Tx with PRS BWA, re-use the existing margin for single PFL when the aggregated BW is no larger than largest BW of single PFL. FFS margins for aggregated BW larger than largest BW of single PFL.

Proposal 2: For accuracy TCs, test one BW based on the existing PRS RMC with larger BW.

Reference

- R4-2410353, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson