3GPP TSG-RAN WG4 Meeting #112	R4-2412659

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	On performance requirements for CPP

Source: 	Huawei, HiSilicon

Agenda item:	6.1.2.6

Document for:	Discussion

Introduction

RRM performance requirements for CPP are discussed in RAN4#111, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- Accuracy requirements

- Test cases

In this paper, we will provide our views on RRM performance requirements for CPP.

Discussion

Accuracy requirements

Issue 2-2-2: Additional margins for frequency drift and RF calibration Proposals: - Option 1: Define extra margin in requirements - FFS: need for extra simulations. - Option 2: Do not define extra margin in requirements - Do not update simulation assumptions for carrier phase measurement. Further discuss in the next meeting, proponents are requested to bring results justifying the additional margin for Option 1.

Due to existence of carrier frequency offset, the error in measured RSCPD or relative RSCP will increase along with the time separation between the two resources for calculating RSCPD or relative RSCP. Assuming 4GHz and 0.1 ppm frequency error, the phase error will be 72 deg if the two resources are separated by one slot of 30kHz SCS (0.5ms). This error is much larger than the BB estimation error which is less than 20 deg based on companies’ results and will make the CP measurement useless.

Therefore, we suggest to define a side condition for the accuracy requirements for CP measurement, that the two PRS resources for calculating RSCPD or relative RSCP are located in the same set of symbols after accounting for expected RSTD.

Proposal 1: The accuracy requirements for CP measurement apply provided that the two PRS resources for calculating RSCPD or relative RSCP are located in the same set of symbols after accounting for expected RSTD.

On top of the applicability condition, some additional margin still needs to be added for the expected RSTD uncertainty. This is because even the two resources are in the same set of symbols after accounting for the expected RSTD, there can still be RTD at UE side due to expected RSTD uncertainty, and this RTD will cause error in RSCPD or relative RSCP if there exists frequency carrier frequency offset.

It is noted that in our submitted simulation results, we did not model the impact of residual frequency error, i.e. the results are based on zero frequency error. We do not see update of the simulation assumption needed or additional simulation needed to derive the additional margin. Instead, the additional margin can be derived based on the expected RSTD uncertainty and the assumed carrier frequency offset.

Proposal 2: For defining CP measurement accuracy requirements, add additional margin on top of simulation results to account for the impact of carrier frequency offset and RSTD uncertainty.

Test cases

Issue 2-2-1: Whether to verify the accuracy of legacy measurements in RSCPD/RSCP TCs Proposals: - Option 1: - Verify both the accuracies of legacy measurements and CPP measurements in one TC with a 90% success rate to reflect UE’s real positioning performance in the deployment. - Option 2: - Not verify the accuracy requirements for legacy RSTD/Rx-Tx measurement in the RSCPD/RSCP TC.

For accuracy TCs, our view is that the accuracy of the other measurement configured and reported together with RSCPD/RSCP should not be verified. In the accuracy TC for RSCPD/RSCP, UE is requested to report RSCPD/RSCP together with other measurement, but only RSCPD/RSCP accuracy should be verified to verdict pass or fail. The reason is that if accuracy of other measurement is also verified, it would impose higher requirements to the UE (UE needs to meet both accuracy with 90% rate) compared to how accuracy requirements are derived based on simulation.

Proposal 3: For accuracy TCs for RSCPD/RSCP, the accuracy of the other measurement configured and reported together with RSCPD/RSCP is not verified.

Conclusions

In this paper we provided our views on RRM performance requirements for CPP.

Proposal 1: The accuracy requirements for CP measurement apply provided that the two PRS resources for calculating RSCPD or relative RSCP are located in the same set of symbols after accounting for expected RSTD.

Proposal 2: For defining CP measurement accuracy requirements, add additional margin on top of simulation results to account for the impact of carrier frequency offset and RSTD uncertainty.

Proposal 3: For accuracy TCs for RSCPD/RSCP, the accuracy of the other measurement configured and reported together with RSCPD/RSCP is not verified.

Reference

- R4-2410193, WF on SL positioning and carrier phase positioning, CATT