3GPP TSG-RAN WG4 Meeting #112	R4-2412660

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on remaining issues in RRM requirements for MUSIM

Source: 	Huawei, HiSilicon

Agenda item:	5.25.1

Document for:	Discussion

Introduction

RRM core for R18 MUSIM are discussed in RAN4#111, and the outcomes are captured in [1]. In our view, some remaining issues need to be further discussed in maintenance phase.

In this paper, we will provide our views on remaining issues in R18 MUSIM.

Discussion

Issue 1-1-2: Scenarios for the case where the MO to be measured without MG have to be measured in the associated MG Agreement: - Case 1: For the scenario when measurement gap and MUSIM gaps are configured, where intra/inter- frequency measurement is measured with no measurement gap however part of the SMTC occasions of this intra/inter- frequency measurement object are overlapped with the measurement gap and all its SMTC are overlapping with the union of measurement gap and MUSIM gaps. Clarify the measurement will be performed within measurement gap. - Further discuss the CR wording - Case 2: For the scenario when concurrent gaps and MUSIM gaps are configured, where intra intra/inter- frequency measurement is measured with no measurement gap however part of the SMTC occasions of this intra/inter- frequency measurement object are overlapped with the associated measurement gap of concurrent gap and all its SMTC are overlapping with the union of concurrent gaps and MUSIM gaps. Clarify the measurement will be performed within the associated measurement gaps. - Further discuss the CR wording - Case 3: For the scenario when concurrent gaps and MUSIM gaps are configured, where intra/inter-frequency measurement is measured with no measurement gap and this measurement object is not associated with any concurrent gap, No requirement applies when all its SMTC are overlapping with the union of concurrent gaps and MUSIM gaps. - The proposal for case 3 is RAN4’s common understanding in principle, and further discuss whether/how to capture it in the spec.

We do not see the need to make further clarification for Case 3 in the requirements. Even without MUSIM gaps, no measurement requirement would apply for an MO when concurrent MG is configured and the MO is not associated with any MG. This is already captured in R17 as copied below.

Requirements for concurrent measurement gaps apply provided that each frequency layer is only associated with one concurrent measurement gap.

It is obvious that the applicability of the measurement requirements does not change with MUSIM gaps because MUSIM gaps are not expected to be used for measurement in NW A.

Proposal 1: RAN4 not to make further clarification for Case 3 in the requirements.

Issue 1-1-3: How to capture agreements on MUSIM and Type-1 collision handling when their MGRP is identical Agreement: Capture in the MUSIM requirements in an existing section that no requirements apply if collisions occur between a MUSIM gap and any measurement gap without assigned priority if the two gaps in a collision have the same MGRP.

The agreement for Issue 1-1-3 is already included in the spec as copied below. We do not see a need to make further clarification in the requirements.

Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps. Collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply. Any collisions between MUSIM gaps shall be addressed as specified in clause 9.1.10.3 and 9.1.10.4. When “keep solution” in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP.

Observation 1: Applicability of MUSIM gap and Type-1 MG collision handling is already in the spec.

Issue 1-1-4: Clarification on network schedule on dropped gaps Agreement: The network can schedule the UE in gaps (MUSIM and measurement gaps) which are dropped due to gap collision handling. - Capture the above understanding in RAN4 spec if it it not fully covered in the current spec. Further discuss the details in the CR.

The agreement for Issue 1-1-4 is already included in the spec as copied below. We do not see a need to make further clarification in the requirements. It is noted that UE may not be schedulable during dropped MUSIM gap occasions because other interruption or scheduling restriction may apply, so the current wording in the spec is more accurate as it states that a slot is not interrupted by a dropped MUSIM gap.

A slot is interrupted by a MUSIM gap if it overlaps with any occasion of the configured MUSIM gap, except for a dropped MUSIM gap occasion.

Observation 2: UE schedulability during dropped MUSIM gap occasions is already in the spec.

Issue 1-1-5: Clarification on MUSIM related operations within allocated MUSIM gaps - Proposals - P1: Clarify in section 9.1.10 that the operations listed concerning cell detection, measurements, paging reception and SI reception are for MUSIM operations and shall be performed within the allocated MUSIM gaps. (Nokia) Recommendations: Discuss during the meeting Issue 1-1-7: UE Rel-18 MUSIM operation using separate receiver - Proposals - P1: RAN4 need to discuss potential impact from UE autonomous MUSIM operation using a separate (MUSIM) receiver. RAN4 to provide a clarification within Rel-18 regarding Rel-18 MUSIM operation using separate receiver (Nokia)

Issue 1-1-5 and 1-1-7 are similar as they both relate to where UE would perform MUSIM operations. In our view, this is already clear with the following statement in the spec.

The UE is not required to perform cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network that is outside the MUSIM gaps.

RAN4 does not need to restrict UE to perform MUSIM operations only within MUSIM gaps. UE is allowed to perform MUSIM operations outside MUSIM gaps (autonomous MUSIM operation) provided it does not impact the NW A operations. This is no different from other operations UE may perform, e.g. GNSS, and we do not see the need to make additional clarification in the requirements.

Proposal 2: RAN4 not to make additional clarification on where UE to perform MUSIM operations in the requirements.

Issue 1-1-6: Clarification on MUSIM requirements - Proposals - P1: Clarify that the MUSIM requirements applies when UE is allocated one or more of the requested MUSIM gaps. (Nokia) Recommendations: Check whether the clarification is agreeable Issue 1-1-8: Clarify UE requirements when only part of the requested MUSIM gaps are allocated by the network. - Proposals: - Clarify that the MUSIM requirements applies when UE is allocated one or more of the requested MUSIM gaps. (Nokia)

Issue 1-1-6 and 1-1-8 are same, and we understand the intension is that UE may not be allocated with all of the requested MUSIM gaps, and even it is allocated part of the MUSIM gaps requested, it should still meet the MUSIM gap related requirements. In our view, whether UE gets all the requested MUSIM gaps would impact whether UE can complete MUSIM operations in NW B (for which RAN4 agreed not to define requirements in R18), but it does not change how the MUSIM gaps interact with other operations in NW A, so the MUSIM gap related requirements for NW A should still apply.

Proposal 3: RAN4 to clarify that the MUSIM gap related requirements for NW A apply when UE is allocated one or more of the requested MUSIM gaps.

Conclusions

In this paper we provided our views on remaining issues in R18 MUSIM.

Proposal 1: RAN4 not to make further clarification for Case 3 in the requirements.

Observation 1: Applicability of MUSIM gap and Type-1 MG collision handling is already in the spec.

Observation 2: UE schedulability during dropped MUSIM gap occasions is already in the spec.

Proposal 2: RAN4 not to make additional clarification on where UE to perform MUSIM operations in the requirements.

Proposal 3: RAN4 to clarify that the MUSIM gap related requirements for NW A apply when UE is allocated one or more of the requested MUSIM gaps.

Reference

- R4-2410412, WF on NR Dual TxRx Multi-SIM, Moderator (vivo)