3GPP TSG-RAN WG4 Meeting #112	R4-2412661

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on remaining issues in R18 NTN core requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.23.6

Document for:	Discussion

Introduction

RRM core requirements for R18 NTN are discussed in RAN4#111, and the outcomes are captured in [1]. In our view, the following issues need to be further discussed in maintenance phase.

- NW verified location

- Mobility enhancement

In this paper, we will provide our views on remaining issues in R18 NTN core requirements.

Discussion

NW verified location

RAN4 has defined requirements for PRS measurement for NW verified location. The requirements are mostly re-used from existing TN requirements, with some adaptations e.g. removing the factors related to multiple frequency layer, multiple TEGs, Rx beam sweeping, etc.

However, we believe one of the most outstanding difference compared to PRS measurement in TN has been missed in current requirements, i.e. UE is only required to measure PRS from the serving cell. In TN, UE is required to measure all PRS resources provided in the Assistance Data up to its capability, no matter if the PRS resource is transmitted by its serving cell or other non-serving cells. In NTN, we understand UE is only required to measure PRS from the serving cell when the Assistance Data contains configuration for PRS from other cells, and this should be clarified in the requirements.

Proposal 1: RAN4 to clarify that UE is only required to measure PRS from the serving cell for PRS measurement for NW verified location.

Mobility enhancement

### Issue 4-1: TN to NTN cell reselection Agreement: (online) - It is a common understanding that location-based measurement triggering parts are not applicable for cell reselection from TN to NTN. Whether/How to implement this in RAN4 spec is left to CR. - If both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode, - For NTN capable UE, the cell reselection requirements (i.e., the TN to NTN reselection requirement agreed in RAN4 #110) are applied to both TN and NTN target cells/carriers. - FFS: For NTN incapable UE, the existing TN-to-TN cell reselection requirements are applied. - Further check does UE know whether it is a TN carrier or NTN carrier

RAN4 has defined requirements for TN to NTN cell reselection. For NTN capable UE, both TN and NTN carriers are considered, and it is FFS whether existing TN to TN cell reselection requirements apply for NTN incapable UE.

In our view, if the UE is incapable of NTN, it is not expected to measure the NTN carriers even if NTN carriers are provided in SIB4. The UE should then meet the existing TN to TN cell reselection requirements where NTN carriers are not considered. The issue was left FFS because it was unclear if the UE can tell a carrier configured in SIB4 is TN or NTN carrier.

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

…

}

MultiFrequencyBandListNR-SIB ::=            SEQUENCE (SIZE (1.. maxNrofMultiBands)) OF NR-MultiBandInfo

NR-MultiBandInfo ::=                        SEQUENCE {

freqBandIndicatorNR                         FreqBandIndicatorNR         OPTIONAL,   -- Cond OptULNotSIB2

nr-NS-PmaxList                              NR-NS-PmaxList              OPTIONAL    -- Need S

}

In our understanding, UE is able to tell if a carrier configured in SIB4 is a TN or an NTN carrier, by the band information which is mandatory in SIB4. As shown above, for each carrier in SIB4, NW has to provide the band number, and the band numbers for TN and NTN are distinct. An NTN incapable UE will ignore carriers that are associated to NTN bands because they belong to bands the UE does not support. In other words, it will not measure the NTN carriers.

Therefore, we suggest RAN4 to confirm that for NTN incapable UE, the existing TN-to-TN cell reselection requirements apply even NTN carriers are configured for measurement.

Proposal 2: RAN4 to confirm that for NTN incapable UE, the existing TN-to-TN cell reselection requirements apply even NTN carriers are configured for measurement.

RAN4 has defined requirements for satellite switch in a similar way as HO. In particular, the ending point of the delay or interruption is the UL transmission in the new satellite. Also, the delay or interruption time includes the time uncertainty TIU from the complete of fine time tracking to the UL transmission.

##### 6.1C.3.2.2 Interruption time for hard satellite switch with re-sync The interruption time is the time between t-service and the time the UE starts transmission of the new PRACH for RACH-based case or first UL transmission on PUSCH for RACH-less case if the UE only supports the feature for hard satellite switch and the hardSatelliteSwitch-Resync-NTN-r18 is enabled. ##### 6.1C.3.2.3 Satellite switch delay for soft satellite switch with re-sync The Satellite switch delay is from t-serviceStart to the time instance for the first UL transmissionon of the new PRACH for RACH-based case if TA timer is not running and there is no PUCCH SR or configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH for RACH-less based case, if the UE supports the feature for soft satellite switch and the softSatelliteSwitch-Resync-NTN-r18 is enabled.

However, based on RAN2 procedure, UE is not expected to transmit UL after the satellite switch, and this is different from HO where UE has to send PRACH or PUSCH as part of the HO procedure. Without data or control message to transmit, UE will not transmit anything after the satellite switch. The RACH procedure will also be triggered as needed, i.e. UE will not necessarily transmit PRACH after the satellite switch.

Therefore, it is improper to use UL transmission as the ending point of satellite switch. We suggest to re-use what we have in BWP switch requirements, i.e. UE being ready to receive DL channels/signals or transmit UL channels/signals from/to the target satellite. Accordingly, TIU can be removed.

Proposal 3: RAN4 to clarify that the ending point of satellite switch with re-sync is the time point when UE is ready to receive DL channels/signals or transmit UL channels/signals from/to the target satellite, and to remove TIU in the delay/interruption time.

Conclusions

In this paper we provided our views on remaining issues in R18 NTN core requirements.

Proposal 1: RAN4 to clarify that UE is only required to measure PRS from the serving cell for PRS measurement for NW verified location.

Proposal 2: RAN4 to confirm that for NTN incapable UE, the existing TN-to-TN cell reselection requirements apply even NTN carriers are configured for measurement.

Proposal 3: RAN4 to clarify that the ending point of satellite switch with re-sync is the time point when UE is ready to receive DL channels/signals or transmit UL channels/signals from/to the target satellite, and to remove TIU in the delay/interruption time.

Reference

- R4-2410380, WF on RRM requirements for NR_NTN_enh, Moderator (Qualcomm Incorporated)