3GPP TSG-RAN WG4 Meeting #112	R4-2412664

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on remaining issues in R18 NTN perf requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.23.7

Document for:	Discussion

Introduction

RRM performance requirements for R18 NTN are discussed in RAN4#111, and the outcomes are captured in [1]. In our view, the following issues need to be further discussed in maintenance phase.

- Rx beam gain

- UL timing test

- RLM TC

In this paper, we will provide our views on remaining issues in R18 NTN performance requirements.

Discussion

Rx beam gain

- For the minimum SSB_RP condition, - The gain difference between fine and rough beams is YdB: - For mechanical steering antenna, Y=0 - For electronic steering antenna, FFS Y

In our view, the concept of rough beam is for beam sweeping, where UE may use a set of beams with lower Rx gain in order to reduce the number of beams (thus reducing the sweeping delay). Beam sweeping is used mainly for L1 and L3 measurement when UE needs to search for other Tx beams or cells with different Rx beams. However, beam sweeping is not assumed in the measurement and mobility requirements in R18, and all requirements except for HO are for intra-satellite scenario where UE in assumed to use same Rx beam as for the serving cell according to [2]. Even for inter-satellite HO, UE is only assumed to fine tune the beam steering rather than to perform beam sweeping.

Therefore, we understand the concept of rough beam is not applicable to FR2-NTN, not only for mechanical steering antenna, but also for electronic steering antenna. Although it is feasible for the latter to adjust the beam patterns (beam width/gain/number) to form rough beams, there is no clear use case (when UE needs to use rough beams), and defining rough beam will complicate the spec, e.g. the SSB_RP conditions needs to consider both beams. We suggest not to define rough beam for electronic steering antenna (Y=0)

Proposal 1: RAN4 to confirm Y=0 also for electronic steering antenna.

- FFS: Gmin and Gmax

In our view, Gmin should be derived from the EIS, which is the minimum gain that all UEs needs to support. This is aligned with the methodology recently used by RAN4 to derive the Gmin for PC1/5/6 [2].

- For minimum dBi: value from proposal 1 agreed with [ ]. - Note: Value derived based on minimum EIS delta compared to PC3) - For maximum dBi: - Equation: - Option 1: 20log (# antenna element) +5dBi +3 - FFS whether need to update the value for PC3 to align the equation - Option 2: 10log (# antenna element) +5dBi +3 (PC3 assumption) - Number of antenna elements - Option 1: 144 for PC1, 64 for PC5/6? - Option 2: aligned with the assumption from Y and Z - 64 for PC1, 36 for PC5/6 - Note: Test ability issue also need to be considered for feasible values of Gmin and Gmax

Of course, we cannot use ESI delta to PC3 to derive Gmin for VSAT because the NF and implementation margin may be different between TN UE and VSAT. In fact, in clause 7.4.3.3.2 of TR 38.863, it is captured how the EIS requirements for VSAT are derived.

- 50MHz minimum EIS for VSAT type 3 (fixed VSAT communicating with LEO only with electronical steering antenna) is -115.6dBm, based on G/T = -5dB

- 50MHz minimum EIS for other VSAT types is -122dBm, based on G/T = 6dB

It is noted that G/T also includes the impact of NF. Considering 6dB NF for VSAT type 3 and 2.5dB NF for other VSAT types, and assuming T value for 20 degree Celsius, we can get the Gmin for VSAT type 3 as 25dB and for other VSAT types as 33dB.

Gmax depends more on the implementation. We suggest to use 50dB for all VSAT types, but we are open to other views.

Proposal 2: RAN4 to define

- Gmin = 25dB for VSAT type 3, and Gmin = 33dB for other VSAT types

- Gmax = 50dB for all VSAT types

UL timing test

- The value for TGNSS_margin for mobile and fixed UEs shall be introduced for uplink timing error requirements for FR2 NTN (Ka band introduced in Rel-18 ) - Further discuss the exact values: - Option 1: X = [(Te_NTN-Te)/2] - Other options not precluded

TGNSS_margin is a negative margin used in the UL timing test, i.e. it is used to tighten the testing requirements compared to the core requirements due to the simplified test setup without GNSS emulation. In R17 TC, it is defined as 327.68 Tc corresponding to 25m distance error which is half of the assumed GNSS error in core requirements.

5) The test system shall verify that the UE transmit timing offset stays within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±(Te_NTN –TGNSS_margin) of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment. … e. The TGNSS_margin counts for the margin for the GNSS position definition error considered in the core requirement, which needs to be subtracted for the test requirement, due to the usage of AT commands in the test. TGNSS_margin=327,68×Tc

We suggest to re-use the same principle to define the margin for FR2-NTN. In this sense, we do not support option 1 because Te_NTN-Te includes not only the GNSS error but also the satellite positioning error, so the testing requirements are overly tightened.

Based on the discussions during the core part, we suggest to take 10m and 30m GNSS error for fixed and mobile VSAT, and the resulted TGNSS_margin is 65.536 Tc and 196.608 Tc. Note that the GNSS error was not separately agreed in core part, and the above numbers are derived based on our own understanding on how the final Te_NTN is made up. We are open to other views.

Proposal 3: RAN4 to define TGNSS_margin as 65.536 Tc and 196.608 Tc for fixed and mobile VSAT.

- In the test case of UE transmission timing accuracy for Case-3 (120kHz SCS) (Mobile UE for GSO): - UE mobility is not considered before the testability issue is resolved. And without consideration of UE mobility during test, UE test requirement for Case-3 will be further discussed considering necessary margin. - It’s not precluded to further update test case including test requirements for case -3 if testability issue on UE mobility resolved in future release.

As UE mobility is not modelled in the test, the test setup for Case 1 and Case 3 will be identical. In our view, the test requirements should be same for the two cases, and based on core requirements of Case 1. A mobile VSAT is a VSAT that is capable to move, but it does not necessarily have a worse GNSS performance as if it is moving when it is actually not. When it is absolutely fixed as in the test setup, we understand it should be able to achieve similar GNSS performance as fixed VSAT.

Proposal 4: For FR2-NTN UL timing test, the test requirement for Case 3 is same as that for Case 1.

Note that above does not mean we need to make any change to the core requirements because in real world it may be difficult to determine whether a VSAT is moving or fixed like in the test setup.

RLM TC

- Check if SNR values for RLM-RS test to NTN FR2 UE shall be updated from legacy FR2 and how to determine the values - Ericsson

In terms of RX number, a VSAT is similar to 1RX UE due to single polarization reception. In last meeting it is agreed to relax the measurement accuracy due to 1RX. 1RX will also impact the RLM.

In R17 RedCap WI, the impact of 1RX UE was discussed. PDCCH parameters are updated to make sure the Qout and Qin levels are same as 2RX UE. The evaluation period for the OOS is extended to make sure the measurement accuracy is same between 1RX and 2RX UEs around Qout. As a result, the SNR levels in the RLM TCs are same between 1RX and 2RX UEs. RAN4 should discuss whether to follow same approach as R17 RedCap to handle VSAT, and this means the core requirements for RLM need to be updated.

Another alternative is to follow the current core requirements. Qin and Qout levels would be different due to worse demodulation performance of 1RX. The measurement accuracy may also need to be evaluated to determine the margins between SNR levels. The SNR levels in the TCs likely need to be changed.

Proposal 5: For RLM for FR2-NTN, RAN4 to discuss the following options.

- Option 1: update core requirements (PDCCH parameters, evaluation period) as for R17 RedCap

- Option 2: update the SNR levels in TCs with new Qout/Qin and measurement accuracy

Conclusions

In this paper we provided our views on remaining issues in R18 NTN performance requirements.

Proposal 1: RAN4 to confirm Y=0 also for electronic steering antenna.

Proposal 2: RAN4 to define

- Gmin = 25dB for VSAT type 3, and Gmin = 33dB for other VSAT types

- Gmax = 50dB for all VSAT types

Proposal 3: RAN4 to define TGNSS_margin as 65.536 Tc and 196.608 Tc for fixed and mobile VSAT.

Proposal 4: For FR2-NTN UL timing test, the test requirement for Case 3 is same as that for Case 1.

Proposal 5: For RLM for FR2-NTN, RAN4 to discuss the following options.

- Option 1: update core requirements (PDCCH parameters, evaluation period) as for R17 RedCap

- Option 2: update the SNR levels in TCs with new Qout/Qin and measurement accuracy

Reference

- R4-2410380, WF on RRM requirements for NR_NTN_enh, Moderator (Qualcomm Incorporated)

- R4-2410294, WF on defining the missing testing parameter for PC1/5/6, Samsung