3GPP TSG-RAN WG4 Meeting #112	R4-2412667

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Initial discussion on RRM requirements for less than 5MHz Ph2

Source: 	Huawei, HiSilicon

Agenda item:	8.4.3

Document for:	Discussion

Introduction

In RAN#103, a new R19 WI on NR channel BW less than 5MHz for FR1 Phase 2 was approved, and the latest WID can be found in [1].

The objective of the WI is re-produced below.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In this paper, we will provide our initial views on RRM requirements for less than 5MHz Ph2.

Discussion

From the WID, the main enhancement in Ph2 is to support CA/DC with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band. In our view, RAN4 needs to analyze

- Whether and how to extend R18 requirements to SCell and PSCell

- Whether there are new impacts from CA/DC

Extending R18 requirements to SCell and PSCell

In R18, the impacts of less than 5MHz CBW to RRM requirements mainly include

- PBCH puncturing, and the impacted requirements are HO and SSB index reading in L3 measurement

- Reduced BW, and the impacted requirements are RLM and BFD (PDCCH parameter)

PBCH puncturing

PBCH puncturing will lead to longer delay for PBCH DMRS sequence detection and PBCH/MIB decoding, so the delay requirements for SSB index reading in L3 measurement and HO are extended.

Considering CA and DC, we understand at least the following requirements are subject to similar impacts.

- SSB index reading in L3 measurement for deactivated SCC

In R18, the extension of SSB index reading time is only applicable for intra-frequency measurement for active serving cell, and inter-frequency measurement. It is straightforward that the SSB index reading time for deactivated SCC (e.g. Table 9.2.5.1-6) is extended.

- PSCell change and addition

PSCell change and addition procedure is quite similar to HO requirements. As PBCH/MIB decoding is also needed during the procedure, the delay requirements should be extended in a similar way.

Reduced BW

Reduced BW means the PDCCH BW cannot be too large, so in R18 the PDCCH parameter for RLM and BFD are changed so that the BW can fit into the reduced BW of 12 RB and 15RB, and CCE number is reduced accordingly.

For PSCell, the R18 requirements directly applies (same RLM and BFD requirements apply to PCell and PSCell). For SCell one may consider to update the PDCCH parameter for SCell BFD. We do not have strong view, but SCell BFD may not be useful for less than 5MHz which is deployed in lower FDD bands.

Summary

Based on above, we suggest RAN4 to first focus on the impacts of PBCH puncturing, and to the most basic requirements. RAN4 can discuss if additional requirements, e.g. SCell BFD, HO with PSCell, CPAC, SCG activation, subsequent PSCell change, etc. needs to be updated to account for less than 5MHz.

Proposal 1: RAN4 to discuss the impacts of less than 5MHz, in particular PBCH puncturing, to at least the following requirements

- SSB index reading in deactivated SCC measurement

- PSCell change and addition

New impacts from CA/DC

Based on our analysis, less than 5MHz may not impact other requirements for SCell or PSCell, e.g. MTTD / MRTD, SCell addition/release, SCell activation, interruption related to SCell and PSCell operation, can be re-used from existing requirements.

One issue that RAN4 may need to discuss in the PBCH BW indication in MO. The issue was discussed in R18, and it was concluded that UE supporting less than 5MHz can determine the PBCH BW for an SSB configured in MO for measurement by checking whether the SSB frequency is on the new sync raster or not. The assumption in R18 was that PBCH puncturing is only applicable to SSB on the new sync raster because the cell with less than 5MHz can only be PCell when serving a UE.

In R19, the cell with less than 5MHz can be SCell, and it is not clear if the SSB of such a cell can be off the new sync rater. If it is possible, then UE may need additional indication from NW to determine if an SSB frequency in an MO is with punctured PBCH or not. Otherwise, such an indication is not needed and UE can follow R18 approach.

Proposal 2: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell.

Conclusions

In this paper we provided our initial views on RRM requirements for less than 5MHz Ph2.

Proposal 1: RAN4 to discuss the impacts of less than 5MHz, in particular PBCH puncturing, to at least the following requirements

- SSB index reading in deactivated SCC measurement

- PSCell change and addition

Proposal 2: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell.

Reference

- RP-240832, New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN, Moderator (vivo)