3GPP TSG-RAN WG4 Meeting #112	R4-2412668

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Initial discussion on RRM requirements for less than 5MHz for NTN

Source: 	Huawei, HiSilicon

Agenda item:	8.8.3.4

Document for:	Discussion

Introduction

In RAN#104, a new R19 WI on Enhanced requirements and test methodology for NR and IoT NTN was approved, and the latest WID can be found in [1].

One objective in the WI that has RRM impact is support for less than 5MHz for NTN.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In this paper, we will provide our initial views on RRM requirements for less than 5MHz for NTN.

Discussion

Based on the justification of the objective, which is reproduced below, the intension is to re-use the work done in the R18 WI “NR support for dedicated spectrum less than 5 MHz for FR1” for TN as much as possible.

Less than 5MHz for NTN In Rel-18, the feature to support less than 5MHz for NR is defined for TN to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID “NR support for dedicated spectrum less than 5 MHz for FR1” by extending support for NTN. This feature will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited. This feature should be built on the work done in the Rel-18, i.e., WI of NR support for dedicated spectrum less than 5 MHz for FR, as much as possible by extending support for NTN.

In R18 TN WI for less than 5MHz, the main impacted RRM requirements include

- HO interruption

- SSB index reading in L3 measurement

- Hypothetical PDCCH parameters for RLM/BFD

The first two requirements are impacted because of PBCH puncturing, where additional SSB samples are allowed for UE to read PBCH/MIB, and the delay requirements are extended depending on the related SINR conditions. The last requirements are impacted because of reduced Tx BW configuration, so the existing PDCCH parameters including the aggregation level, number of OFDM symbols, and CCE to REG mapping are modified such that the PDCCH can be accommodated by the reduced Tx BW configuration. It will result at different Qout/Qin levels compared to normal Tx BW configurations.

In our view, impacts to RRM requirements due to support of less than 5MHz are orthogonal to those due to NTN. PBCH/MIB decoding delay and PDCCH parameters for RLM/BFD are not impacted due to NTN, so we do not need to combine the impacts of two features. Instead, the changes to TN requirements for less than 5MHz can be directly re-used to update the requirements for NTN to support less than 5MHz.

Proposal: As starting point, the changes to the following TN requirements for less than 5MHz in R18 are re-used to update the requirements for NTN to support less than 5MHz.

- HO interruption

- SSB index reading in L3 measurement

- Hypothetical PDCCH parameters for RLM/BFD

Conclusions

In this paper we provided our initial views on RRM requirements for less than 5MHz for NTN.

Proposal: As starting point, the changes to TN requirements for less than 5MHz in R18 are re-used to update the requirements for NTN to support less than 5MHz.

- HO interruption

- SSB index reading in L3 measurement

- Hypothetical PDCCH parameters for RLM/BFD

Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)