3GPP TSG-RAN WG4 Meeting #112	R4-2412669

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Simulation assumption for LP-WUR measurement

Source: 	Huawei, HiSilicon

Agenda item:	8.22.4.1

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#111, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to evaluate the link level performance of LP-WUR measurement, and a simulation assumption is listed in [1] for information.

In this paper, we will provide our views on the link level simulation assumption for LP-WUR measurement.

Discussion

In the next, we will provide our suggested changes to the simulation assumption in [1]. The last column is added to provide our considerations behind the updates.

Table 1: General parameters

| Simulation parameters | Comments/values | Remarks |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | FR1: 2.6 GHz/700MHz |  |
| System bandwidth | 20MHz for 700MHz carrier frequency / 100 MHz for 2.6GHz carrier frequency; | Typical system BW for corresponding carrier frequency |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Yes / No | Cell 1 is the serving cell and should be known to UE; Cell 2 is interfering cell and not known to UE |
| DRX | No |  |
| BS transmit antennas for LP-SS blocks and SSB | 1 tx or single layer transmissions | Same Tx number for SSB and LP-SS |
| UE receive antennas | 1 rx |  |
| Data and control channel subcarrier spacing | Same SCS for LP-SS, SSB and PDCCH/PDSCH | SCS should not depend on Rx type, and for simplicity we assume same SCS for all BS transmissions |
| Measurement period (in number of measurement samples) | LP-SS: [5, other number of samples may also be studied upon a need] SSB: [5] | Measurement period should depend on RS but not Rx type |
| Subcarrier spacing | 2.6GHz: 30 kHz; 700MHz: 15kHz | 15kHz is not typical for 2.6GHz |
| Number of LP-SS blocks per LP-SS burst set, K | [1] | Typo correction |
| LP-SS/SSB burst periodicity | [320 ms] OFDM based: [320 ms] |  |
|  |  | Included above |
| LP-SS block BW | 132 subcarriers for SCS=30kHz; [TBD subcarriers for SCS=15kHz ] Note: May need be updated based on RAN1 conclusion | Based on RAN1#117 agreements |
| Guard band between LP-SS and NR data | 1RB | Needed to model the interference from other NR transmissions from the serving BS |
| Actual LP-SS block transmissions | always transmitted |  |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 | Remarks |
| --- | --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |  |
| NR-PSS,NR-SSS | - | To be indicated by companies | To be indicated by companies | Suggest to keep LP-SS related settings in one place |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |  |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |  |
| RB Utilization | % | 100 | 100 |  |
| LP-SS |  | To be indicated by companies, including number of OFDM symbols, M value for OOK-4, binary sequence and overlaid sequence | Same as Cell 1 but with different sequences | RAN1 has not made conclusion on LP-SS waveform, sequence, etc., can be updated based on RAN1#118 agreement, otherwise can be left to companies’ report. For Cell 2, suggest to model LP-SS to LP-SS interference, which is more typical |
|  |  |  |  | Included above |
| Data Modulation | - | QPSK | QPSK | We assume this is for NR data. LP-SS parameters included above |
| Slot length | - | 14 symbols | 14 symbols |  |
| CP Length | - | Normal | Normal |  |
| Frequency Offset relative to UE frequency reference | Hz | 5 ppm | 5 ppm | Based on our companion discussion paper |
| Time error |  | To be indicated by companies | N/A | Based on our companion discussion paper For Cell 2, the timing error is determined by the relative delay |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | [CP/2] |  |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | [3 ms] |  |
| SNR | dB | -2.7dB, -2.4dB | -11.7dB, -8.4dB | Target to achieve -3dB SINR for Cell 1, and Cell 2 is 9dB or 6dB lower than Cell 1 |
| Es/IoT (calculated from SNR) | dB | -3dB | N/A | SINR for Cell 1 is -3dB |
| Propagation conditions | - | FR1: AWGN TDL-A 30ns (optional) TDL-B 100ns (optional) TDL-C 300ns |  | Make TDL-A and TDL-B optional to reduce simulation efforts |
| UE speed |  | FR1: 3km/h |  | 3km/h is more typical for LP-WUR use case |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [5th Order Butterworth with 4.32MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4/8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84/7.68MHz] |

Proposal: Take suggested updated in Table 1 and Table 2 into account in the simulation assumption for LP-WUR measurement.

Conclusions

In this paper we provided our views on the link level simulation assumption for LP-WUR measurement.

Proposal: Take suggested updated in Table 1 and Table 2 into account in the simulation assumption for LP-WUR measurement.

Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo