3GPP TSG-RAN WG4 Meeting #112	R4-2412671

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Initial discussion on RRM requirements for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	8.19.3

Document for:	Discussion

Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the latest WID revised in RAN#104 can be found in [1].

RRM related objectives are reproduced below.

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this paper, we will provide our initial views on RRM requirements for SBFD.

Discussion

Based on the WID, RRM impacts are mainly from two aspects, CLI handling and SBFD operation.

CLI handling

CLI handling mechanisms have been discussed in RAN1 for several meetings, and in RAN1#117 there are agreements on the scope of the work, which are reflected by the following objective in the WID.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. …… - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

gNB-to-gNB CLI handling

For gNB-to-gNB CLI handling, several CLI handling mechanisms including spatial domain beam nulling and beam paring, time and frequency domain scheduling coordination, and interference suppression are discussed. According to the WID, the spec impacts of gNB-to-gNB CLI handling are 1) information exchange among gNBs, and 2) UL resource muting for PUSCH. It is noted that the exact CLI handling mechanisms are up to NW implementation but not specified in spec.

For CLI handling gNB may take some measurement, e.g. for beam nulling the aggressor gNB may measure the CSI from the SSB or CSI-RS of the victim gNB, and for interference suppression the victim gNB may measure the covariance matrix of the interference leakage from muted UL resources. However, there is no gNB measurement to be defined in spec or to be exchanged among gNBs (this is quite different from UE CLI measurement). Therefore, we do not see the need to define RRM requirements for gNB measurement.

Proposal 1: RAN4 not to define RRM requirements for gNB-to-gNB CLI handling.

UE-to-UE CLI handling

For UE-to-UE CLI handling, it is agreed to support UE-to-UE CLI measurement and reporting. Different from CLI measurement in R16, it is L1 measurement based on existing CSI framework, so the existing R16 requirements in clause 9.7 cannot be directly re-used. Similar to L1-RSRP or L1-SINR measurements, RAN4 should define core requirements for L1 based UE-to-UE CLI measurement.

Based on the WID, RAN1 has not finalized the measurement quantities, but L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices are mentioned as examples. RAN4 can further wait for the conclusion from RAN1 on the measurement quantities while use L1-SRS-RSRP and L1-CLI-RSSI for discussion.

Proposal 2: RAN4 to define requirements for L1 based UE-to-UE CLI measurement, e.g. L1-SRS-RSRP and/or L1-CLI-RSSI.

To define the requirements, we understand the following aspects need to be discussed by RAN4.

Measurement delay

In R16, the measurement delay in terms of number of samples for CLI measurement is 3 for SRS-RSRP and 1 for CLI-RSSI. For L1 based CLI measurement, we assume one motivation is to enable fast report to reflect the dynamic change in UE-to-UE CLI. In this sense, single sample measurement should be considered for L1-SRS-RSRP. In L1-RSRP measurement, the number of samples (M=3 or 1) depends on configuration of measurement restriction. RAN4 can discuss whether to re-use the same principle for L1-SRS-RSRP. For L1-CLI-RSSI, it is straightforward to assume single sample as in CLI-RSSI.

RAN1 agreed to support at least aperiodic reporting for L1 based CLI measurement, and periodic and semi-persistent reporting can be considered. We would like to note that even for aperiodic reporting, the resources for the measurement can still be periodic, semi-persistent, or aperiodic. As the measurement period is based on the resource type rather than reporting type, RAN4 still needs to define the measurement delay at least for periodic and semi-persistent resources. For aperiodic resource, single sample should be assumed.

For FR2, the typeD QCL assumptions for the L1 based CLI measurement resource may be configured to UE. If agreed, this would be different from R16 where the UE is assumed to use the same Rx beam as PDCCH or PDSCH for CLI measurement. However, in both cases no Rx beam sweeping is needed, so it may not impact measurement delay requirements.

Side conditions and measurement accuracy

In R16, the SRS-RSRP accuracy is based on a side condition of Es/Iot >= 1dB and timing error of NTA_offset + 4.67us (FR1) or NTA_offset + 3.67us (FR2). The Es/Iot condition was selected based on a joint consideration of 1) the fact that the distance between the aggressor UE and victim UE would not be very large for the CLI measurement to meaningful, and 2) the measurement accuracy with selected number of samples and possible timing error. The timing error condition was selected based on an assumed cell size plus cell phase error between cells.

Some of these consideration factors also apply for L1-RSR-RSRP and some may need to be revisited. For example, if number of samples is reduced from 3 to 1, the condition on Es/Iot or the timing error may need to be improved. Side conditions and measurement accuracy should be discussed together, and link level simulations may be needed.

For L1-CLI-RSSI, it is straightforward to re-use the same accuracy as CLI-RSSI (there is no Es/Iot or timing error condition for CLI-RSSI as no particular signal is measured).

Scheduling and measurement restriction

In R16, scheduling restriction are defined for both SRS-RSRP and CLI-RSSI assuming UE may use a different (earlier) timing than DL timing, considering that the transmission from aggressor UE is with TA. RAN4 can discuss whether to follow same assumption for the measurement timing or not. At same time, RAN4 can also wait for RAN1 conclusion on which of the 4 methods as agreed in RAN1#116 will be supported, and whether they have any difference in terms of impacts to other DL reception (UL transmission is anyway not possible when UE is doing measurement) compared to SRS-RSRP or CLI-RSSI. Besides the measurement timing, the Rx beam may also need to be considered in defining scheduling restriction.

Agreement For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured.

On measurement restriction, in R16 SRS-RSRP and CLI-RSSI measurements are assumed to not overlap with other L3 or L1 measurement, otherwise no requirements for SRS-RSRP and CLI-RSSI apply. RAN4 can discuss whether to follow the same principle or not for L1-RSR-RSRP and L1-CLI-RSSI.

#### 9.7.2.5 SRS-RSRP measurement period If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TSRS_RSRP_measurement_period. #### 9.7.3.5 CLI-RSSI measurement period If the CLI-RSSI measurement resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TCLI_RSSI_measurement_period.

Report mapping

Report mapping for SRS-RSRP and CLI-RSSI are defined in clause 10.1.22. Although report mapping is performance part, it will impact the L1 signaling, e.g. number of bits. RAN4 can discuss whether to re-use the mapping tables for L1-RSR-RSRP and L1-CLI-RSSI.

Proposal 3: RAN4 to discuss following aspects for L1 based UE-to-UE CLI measurement.

- Measurement delay

- Side conditions and measurement accuracy

- Scheduling and measurement restriction

- Report mapping

SBFD operations

Background

The basic concept of SBFD operation is shown in Figure 1.

Figure 1: Illustration of basic SBFD operation

In a TDD carrier, some DL and flexible symbols can be configured as SBFD symbols. On each SBFD symbol, there is 1 or 2 DL subbands and 1 UL subband. Some UE may transmit on the UL subband, and some other UE may receive on the DL subband, i.e. full duplex is supported from gNB perspective based on subband, while it is still half duplex from UE perspective.

Based on RAN1 agreement, the time and location of SBFD subband is semi-static, and will be provided to UE via cell specific signaling. In time domain, the periodicity of SBFD subband occurrence is same as TDD-UL-DL pattern periodicity, and SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. In frequency domain, the subband resources are same across different SBFD symbols within a TDD carrier.

Impacts to RRM requirements

In this section, we will go through main categories of RRM requirements and try to identify those impacted due to SBFD operations based on the RAN1 agreements related to SBFD TX/RX/measurement procedures.

SSB measurement

In RAN1#117, following is agreed regarding collision handling between UL and DL on SBFD symbols.

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

It can be seen that SSB reception is prioritized over UL transmission. In our view, this means most of the RAN4 RRM requirements that are based on SSB measurement will not be impacted. Although the RAN1 agreement is mainly for L1 measurement (reception of SSB from serving cell), we believe the same should apply for L3 measurement following existing RAN4 requirements. For measurement outside MG, RAN4 has defined scheduling restriction for TDD where the L3 measurement within SMTC is prioritized over UL; for measurement within MG, UE is not required to transmit UL during MG. It is straightforward to re-use the same prioritization for handling collision between L3 measurement and UL transmission on SBFD symbols.

Although the SSB measurement itself is not impacted by SBFD operation, its scheduling restriction may need update. As discussed above, RAN4 has defined scheduling restriction for TDD for L3 measurement but not for L1 measurement. The reason is that there is always transition time between TX/RX to/from the serving cell at UE side (in particular, there is guard period for the DL-to-UL transition), and UL transmission on UL symbols would never overlap with SSB reception for the serving cell in a reasonable configuration. However, there is no transition time between UL transmission on SBFD symbols and SSB reception, so the scheduling restriction for TDD should be extended to SSB based L1 measurement.

CSI-RS measurement

In RAN1#116-bis, following is agreed regarding collision handling between UL and DL on SBFD symbols.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

In our view, this means CSI-RS measurement, which is a semi-static DL reception except for aperiodic CSI-RS resource, will be impacted, if the CSI-RS resource is on SBFD symbols. When dynamic UL transmission is scheduled to be colliding with the CSI-RS resource, the measurement occasion will be dropped and the measurement period should be extended accordingly. RAN4 can further discuss whether and how to define the extension, and in our view, we can re-use the approach as in NR-U where the measurement occasion may get unavailable due to dynamic LBT outcome.

In RAN1#117, following is agreed regarding CSI-RS resource mapping in SBFD symbols in frequency domain.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation. - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured). - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands.

It can be seen that on SBFD symbols, CSI-RS REs mapped to RBs outside DL usable PRBs are punctured. It will impact the requirements which apply when CSI-RS BW is larger than a threshold. For example, CSI-RS based RLM requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. It is clear that CSI-RS REs mapped outside DL usable PRBs should not be counted in the transmitted BW, and some clarification may be needed on the transmitted BW.

Above discussion is mainly focused on CSI-RS based L1 measurement. RAN4 can discuss the impacts of SBFD operation to L3 CSI-RS measurement which could be more complex since the CSI-RS resources are transmitted from a neighbor cell that may have configured SBFD subbands.

Mobility

We do not see impacts of SBFD operation to the mobility requirements, including HO, re-establishment and re-direction.

- The cell search and fine time tracking during mobility procedures are based on SSB, and are not impacted due to SBFD operation.

- The time to obtain PRACH resource is defined generically in current requirements and should apply with the new PRACH resources on SBFD symbols.

RACH

We do not see impacts of SBFD operation to the RACH requirements. RAN1 has agreed to support new RACH occasions (ROs) on SBFD symbols, and to support two options where the legacy and new ROs are used under a single RACH configuration and under separate RACH configurations. We understand the current RACH requirements in RAN4 are generic, and can apply to both options.

Timing

We do not see impacts of SBFD operation to the timing requirements.

- The timing accuracy should apply to all UL transmissions, no matter on UL symbols or SBFD symbols.

- The timing accuracy is based on SSB measurement, which is not impacted by SBFD operation.

CA/DC

We do not see impacts of SBFD operation to the CA/DC related requirements, including SCell activation, PSCell addition/change, measurement of deactivated SCell/PSCell and MTTD/MRTD.

- The AGC, cell search, fine time tracking and measurement during those procedures are mostly based on SSB, and are not impacted due to SBFD operation.

- CSI-RS (TRS) may be used during SCell activation e.g. for fast SCell activation, but there is no UL transmission before the SCell is activated, so the CSI-RS reception should not be impacted even the CSI-RS resource is on SBFD symbols.

- The time to obtain CSI-RS for CSI measurement, UL resource for CSI reporting and PRACH resource is defined generically in current requirements and should apply with SBFD operation.

BWP/CBW switch

We do not see impacts of SBFD operation to BWP or CBW switch, as those procedures do not rely on measurement. However, some clarification may be needed because the requirements for DL (and UL) BWP switch are currently defined based on DL (and UL) transmission only on DL (and UL) slot. DL (and UL) transmission on SBFD symbols needs to be accounted.

TCI/UL spatial info/PL-RS switch

We do not see impacts of SBFD operation to TCI/UL spatial info/PL-RS switch requirements.

- The fine time tracking is based on SSB measurement, which is not impacted by SBFD operation.

- The requirements for L1 measurement during those procedures are re-used from general measurement requirements, so the impact of SBFD operation is already accounted as discussed above.

CGI reading

We do not see impacts of SBFD operation to CGI reading. The reception of MIB and SIB1 is in autonomous gaps, and the autonomous gaps are for both UL and DL, and are not limited to specific type of symbols.

MG

We do not see impacts of SBFD operation to the MG related requirements, but one clarification may be needed for the UL transmission immediately after MG. Currently it is only defined for UL slot, while with SBFD operation, the UL transmission can be on SBFD symbol.

Summary

Above we analysed the impacts of SBFD operation to RRM requirements, and identified those impacted by SBFD operation. As this is the first meeting for RAN4 RRM and RAN1 discussion is still ongoing, our analysis is preliminary, and we are open to discuss other possible RRM impacts.

Proposal 4: RAN4 to discuss the impacts of SBFD operation for at least following requirements:

- SSB based L1 measurement

- CSI-RS based L1 and L3 measurement

- MG

- BWP switch

Other impacts of SBFD operation are not precluded.

Conclusions

In this paper we provided our initial views on RRM requirements for SBFD.

Proposal 1: RAN4 not to define RRM requirements for gNB-to-gNB CLI handling.

Proposal 2: RAN4 to define requirements for L1 based UE-to-UE CLI measurement, e.g. L1-SRS-RSRP and/or L1-CLI-RSSI.

Proposal 3: RAN4 to discuss following aspects for L1 based UE-to-UE CLI measurement.

- Measurement delay

- Side conditions and measurement accuracy

- Scheduling and measurement restriction

- Report mapping

Proposal 4: RAN4 to discuss the impacts of SBFD operation for at least following requirements:

- SSB based L1 measurement

- CSI-RS based L1 and L3 measurement

- MG

- BWP switch

Other impacts of SBFD operation are not precluded.

Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei