3GPP TSG-RAN WG4 Meeting #112	R4-2412672

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Initial discussion on RRM impacts of R19 NR NTN

Source: 	Huawei, HiSilicon

Agenda item:	8.25.4

Document for:	Discussion

Introduction

In RAN#102, a new R19 WI on Non-Terrestrial Networks (NTN) for NR Phase 3 was approved, and the latest WID revised in RAN#104 can be found in [1].

The high level objectives of the WI are reproduced below.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1]

In our view, objective #1, #3 and #5 may have impacts to RRM requirements, and we will discuss them in more detail in section 2. Objective #2 may impact demod and RF requirements but not RRM requirements. Objective #3 does not have RAN4 involvement.

In this paper, we will provide our initial views on RRM requirements for R19 NR NTN.

Discussion

DL coverage enhancement

Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

This objective has been discussed in RAN1 for several meetings, and 3 sets of additional reference satellite parameters, namely Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1, are agreed for evaluation. The enhancements discussed in RAN1 include link level enhancements and/or system level enhancements.

In our view, the potential link level enhancements mainly relate to demod performance of DL data and control channel, so they do not have impacts to RRM requirements.

The potential system level enhancements include extension of SSB periodicity and DL satellite beam hopping, and maybe cell DTX/DRX. The motivation is to use beam hopping (kind of TMD-ed) transmission of common control signal/channel within increased SSB periodicity to increase the common control channel coverage ratio, thus improving the system level coverage for all three additional sets of satellite parameter.

In our view, the SSB periodicity extension may have some RRM impacts, e.g. if 320ms SSB periodicity is supported, RAN4 may need to discuss if existing timing accuracy requirements should apply. Also, RAN4 may need discuss if the existing measurement requirements apply with beam hopping.

As RAN1 has not agreed on whether and how to support SSB periodicity extension and beam hopping, we suggest RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement.

Proposal 1: RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement.

Support of regenerative payload

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In our understanding, the main characteristic of regenerative payload over air interface is that the common TA and Kmac equals to zero because the full gNB is on board of satellite. Zero common TA and Kmac is already a configuration option since R17, so we do not see particular impacts on RRM requirements from support of regenerative payload, and we assume existing RRM requirements are applicable for regenerative payload.

On the other hand, RAN2 has discussed possible impacts to the procedures of RRC re-establishment, INACTIVE mode and RACH-less HO from regenerative payload.

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

If those procedures are modified for regenerative payload and the modification results in change in RRM requirements, RAN4 can of course revisit those related requirements.

Proposal 2: RAN4 to assume existing RRM requirements are applicable for regenerative payload as baseline, and it can be revisited based on RAN2 agreements.

Support of RedCap UE with NTN

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

RAN4 has not discussed RRM requirements for RedCap + NTN (RedCap UE operating with NTN), and currently in 38.133 the NTN requirements and RedCap requirements are defined in different clauses. Defining requirements for RedCap + NTN requires RAN4 to go through all the applicable requirements for RedCap and NTN, and to discuss whether and how to combine RedCap and NTN features for each of them.

As the number of requirements for RedCap and NTN are quite large, we suggest RAN4 to start with the most fundamental ones for the basic RRM related procedures. The list of such requirements are in Proposal 3.

Proposal 3: For RedCap UE operating in NTN, RAN4 to start with the following requirements.

- Cell reselection

- Connected mode mobility: HO, RACH, Re-establishment and Re-direction

- Timing

- BWP and TCI switching

- RLM/BFD

- L1-RSRP and CBD

- L3 measurement

In Table 1, we summarize the for each of the requirements in Proposal 3. In addition, it is noted that all NTN requirements are based on the availability of valid assistance information of serving or neighbor cell. This applicability condition should also apply to RedCap UE.

Table 1: Summary of RedCap and NTN features in RRM requirements

| Requirements | RedCap features | NTN features |
| --- | --- | --- |
| Cell reselection | - Reduced measurement capability - 1RX related offset - 1RX relaxed accuracy - eDRX support - R17 relaxed measurement | - Time and location triggered measurement - Enhanced measurement delay - Paging interruption for known and unknown - Cell reselection between TN and NTN |
| HO | - NCD-SSB based HO scenarios - 1RX relaxation for Tsearch - HD-FDD conditions | - Time and location based CHO - RACH-less HO - Satellite switch with re-sync |
| RACH |  |  |
| Re-establishment and Re-direction | - 1RX relaxation for Tidentify - HD-FDD conditions - RedCap specific initial BWP impact | - New scaling factor Ksat for Tidentify |
| Timing |  | - Relaxed for GNSS and satellite location error |
| RLM/BFD | - NCD-SSB based measurement - 1RX change in PDCCH parameters for RLM - 1RX relaxation for evaluation period - HD-FDD conditions | - P factor due to Doppler |
| BWP and TCI switching |  |  |
| L1-RSRP and CBD | - NCD-SSB based measurement - 1RX relaxation for accuracy | - P factor due to Doppler |
| L3 measurement | - Reduced measurement capability - NCD-SSB based intra-frequency measurement - CSSF for 1 searcher - 1RX relaxation for delay and accuracy | - Support of multiple SMTC and MGs - New scaling factor Ksat for measurement delay - Scheduling restriction due to Doppler |
| Positioning measurement | - Frequency hopping - Limited BW - 1RX relaxation for accuracy |  |

Among the RedCap features, we believe RAN4 should at least support 1RX and HD-FDD in RedCap + NTN requirements. 1RX is an important means for RedCap UE to reduce the cost and complexity, and HD-FDD is explicitly mentioned in the objective description.

For other RedCap features, namely eDRX, NCD-SSB and R17 relaxed measurement, more discussions are needed. Those features, although originally introduced for RedCap, can also apply to normal UEs. However, RAN4 has not defined requirements for those features in NTN for normal UEs, and it may be a bit strange to apply a requirement for RedCap UE but not for normal UE. FH in PRS measurement aims at improving measurement accuracy where the measurement is not restricted by the UE RF BW (20MHz) but can reach cell BW (100MHz). However, the cell BW in FR1-NTN bands are limited (~30MHz), so the gain from FH may be limited.

Proposal 4a: RAN4 to support the following RedCap features in RedCap + NTN requirements

- 1RX

- HD-FDD

Proposal 4b: RAN4 to discuss whether to support the following in RedCap + NTN requirements.

- eDRX

- NCD-SSB

- R17 relaxed measurement

- FH in PRS measurement

Among the NTN features, we believe they should be all supported in RedCap + NTN requirements. Those features are applicable to normal UEs, and we do not see technical issues to apply them to RedCap UE. Note that some of the NTN features are NTN specific, e.g. time and location based CHO, or satellite switch with re-sync. They do not have counterpart in TN but they are important to NTN. Also, they are anyway optional features, so defining requirements for RedCap UE does not enforce a RedCap UE to implement them.

One exception is support of multiple SMTC. So far, NTN is mandatorily required to support 2 SMTCs per MO, and supporting more is an optional capability. When the two SMTCs do not overlap there is no scaling in measurement period meaning UE has to support parallel measurement for the two SMTCs. This may be a bit challenging for RedCap UE with reduced complexity.

Proposal 5: RAN4 to support all NTN features in RedCap + NTN requirements except the support of multiple SMTCs.

Conclusions

In this paper we provided our initial views on RRM requirements for R19 NR NTN.

Proposal 1: RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement.

Proposal 2: RAN4 to assume existing RRM requirements are applicable for regenerative payload as baseline, and it can be revisited based on RAN2 agreements.

Proposal 3: For RedCap UE operating in NTN, RAN4 to start with the following requirements.

- Cell reselection

- Connected mode mobility: HO, RACH, Re-establishment and Re-direction

- Timing

- BWP and TCI switching

- RLM/BFD

- L1-RSRP and CBD

- L3 measurement

Proposal 4a: RAN4 to support the following RedCap features in RedCap + NTN requirements.

- 1RX

- HD-FDD

Proposal 4b: RAN4 to discuss whether to support the following in RedCap + NTN requirements.

- eDRX

- NCD-SSB

- R17 relaxed measurement

- FH in PRS measurement

Proposal 5: RAN4 to support all NTN features in RedCap + NTN requirements except the support of multiple SMTCs.

Reference

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT