3GPP RAN WG4 Meeting #112 		R4-2412678

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.12.2.2

Source:	CAICT, SAICT

Title:	Discussion on NTN OTA test method

Document for:	Approval

1	Introduction

This document is to provide our further views on OTA Test methodology for FR1 NTN devices.

2	Test methodology for FR1 NTN devices

2.1	Performance metric for handheld NTN devices

During the previous meeting, various companies shared their views on performance metrics for FR1 NTN handheld UEs, which were consolidated into five main areas as outlined in [1]. It was decided that the first direction would be discarded because of the form factor constraints and omnidirectional pattern for NTN UE was not precluded.

Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

When considering NTN handheld devices that utilize directional antennas with a high-gain main lobe for uplink/downlink transmissions to satellites, the primary focus should be on assessing the radiated power within the main lobe of the UE's antenna, as this directly impacts the quality and strength of the signal transmitted to the satellite.

For devices equipped with omnidirectional antennas, testing should be conducted across a spherical range to capture the performance in all directions. However, since NTN communication involves transmitting and receiving signals upward, the partial sphere performance metric is of greater importance.

Proposal 1:	More detailed information of NTN devices’ antenna pattern from UE vendors are needed.

- For handheld NTN devices with directional antenna, OTA testing should be performed in a partial sphere.

- For handheld NTN devices with omni-directional antenna, OTA testing should be in a full spherical range.

2.2	Usage scenarios for NR-NTN and IoT-NTN handheld UE

Following the previous meeting discussions, handheld UE test scenarios for NTN communication include four options.

Issue 3-1-6: Usage scenarios for NR-NTN and IoT-NTN handheld UE Agreements - RAN4 further discuss prioritized usage scenarios. Free space only is not considered. - Option 1: Prioritize the browsing mode (with hand phantom) - Option 2: Prioritize the browsing mode (with hand phantom) and Free Space - Option 3: Prioritize browsing mode (with hand phantom) and talk mode (Head+Hand) - Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space - Option 5: Prioritize Free Space mode only

Typical use cases in TN involve browse mode (with hand phantom) and talk mode (head and hand phantom). However, in satellite communications, the browse mode should be prioritized over the talk mode test scenario due to following reasons:

Satellite communications require stronger antennas to overcome the signal loss associated with long-distance transmission. When a handheld device is positioned near the head, the orientation of the antenna may not be optimal, affecting signal reception. Additionally, the blockage of human head and body will lead to signal attenuation.

For GEO satellites, a handheld device must be oriented towards a specific direction in the sky to maintain a strong signal. For LEO, the rapid movement of satellites may require frequent adjustment of the antenna direction of equipment to maintain connectivity, or require equipment to have excellent radiation and reception performance over a large spherical range.

Given these considerations, implementing voice calls with handheld NTN devices presents significant challenges. Users are likely to rely more on earphones or other external devices to mitigate the impact of head and hand placement on the connection. As a result, browsing mode becomes the most common use case for handheld NTN UEs. Based on the technical limitations and practical considerations in satellite communications, the hand-only test scenario should be prioritized for handheld NTN UEs.

Proposal 2:	Support Option 1 (Prioritize the browsing mode) and Option 2 (Prioritize the browsing mode and FS) as the Usage scenario for NTN handheld UE.

3	Conclusions

Proposal 1:	More detailed information of NTN devices’ antenna pattern from UE vendors are needed.

- For handheld NTN devices with directional antenna, OTA testing should be performed in a partial sphere.

- For handheld NTN devices with omni-directional antenna, OTA testing should be in a full spherical range.

Proposal 2:	Support Option 1 (Prioritize the browsing mode) and Option 2 (Prioritize the browsing mode and FS) as the Usage scenario for NTN handheld UE.

4	References

- R4-2409931, Way Forward for [110bis][338] TRP_TRS_MIMO_OTA, vivo, CAICT