3GPP TSG-WG4 Meeting # 112	R4-2412679

# Maastricht, NL,  19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR 38.133 Core requirements for CPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To align the core requirement for CPP measurements with RAN2 signaling for CPP measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Core requirement for CPP measurements are aligned with RAN2 signaling for CPP measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirement for CPP measurement is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.5.3, 4.5.5.5, 5.6.7.3, 5.6.7.5, 5.6.8.3, 5.6.8.5, 9.9.7.3, 9.9.7.5, 9.9.8.3, 9.9.8.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE #1

4.5.5.3	Measurement Capability

The UE PRS RSTD and DL-RSCPD measurement capabilities in RRC_IDLE state are as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355 [34].

END OF CHANGE #1

START OF CHANGE #2

#### 4.5.5.5	Measurements Period Requirements

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34] with a request to perform rscpd measurement or rscpd with rstd measurement in nr-DL-PRS-MeasurementTimeWindowsConfig, the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.5.5.3) DL RSCPD or DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4] within the measurement period  TRSCPD witℎ or witℎout RSTD,Total.

When UE supports DL PRS-RSCPD measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and

LMF configures measurement time window(s) for rscpd measurement:

-	The requirements in the Clause 4.5.2.5 apply to DL RSTD measurement.

-	The requirements in Clause 4.5.5.5 apply to DL RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

When UE supports DL PRS-RSCPD and DL RSTD measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for rscpd and rstd measurements:

-	The requirements in Clause 4.5.5.5 apply to DL RSTD measurement and DL RSCPD measurement performed on the PRS resources in indicated PFL.

- 	The requirements in the Clause 4.5.2.5 apply to DL RSTD measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

Measurement period TRSCPD witℎ or witℎout RSTD,Total is defined as:

TRSCPD with or without RSTD,Total=i=1LTRSCPD with or without RSTD,i+ L−1∗maxTeffect,i

Where:

-	i is the index of positioning frequency layer indicated in nr-DL-PRS-MeasurementTimeWindowsConfig,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the measurement in positioning frequency layer i.

Measurement period TRSCPD with or without RSTD,i in positioning frequency layer i is as specified below:

TRSCPD with or without RSTD,i= Kcarrier_PRS∗ NRx,TEG,i∗NRxBeam,i∗NPRS,islotN'∗Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast,i ,

Where:

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation.

-	NRxBeam,i equals to 8, otherwise.

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,i is the Rx TEG specific scaling factor:

-	NRx,TEG,i =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,i is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

-	Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the unmuted PRS resources in the indicated resources sets that are not fully overlapped with other higher-priority DL signals/channels and overlapped with the configured PRS measurement time window(s) are considered.

-	Nsample is the number of PRS RSTD samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i∗Tavailable_PRS,i

Where:

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	when the configured PRS measurement time window is periodic, Tavailable_PRS,i=LCMTPRS,i,TDRX, Twindow, the least common multiple between the time window periodicity Twindow, TPRS,i, and the DRX cycle length TDRX, defined in TS 38.304 [1], clause 7.1, if PFL i is associated with the time window,. Twindow is the periodicity of the PRS measurement time window. Otherwise Tavailable_PRS,i=LCMTPRS,i,TDRX.

TPRS,i is TPRS,ias calculated in Clause 4.5.2.5.

Twindow

TperPRS witℎ mutingTPRS,i

TperPRS witℎ muting=Nmuting∗TperPRS

TperPRS

NmutingNmuting=TmutingPRS∗Lmuting

TmutingPRSLmutingb1

{N,T}

N’

The time TRSCPD with or without RSTD starts from the first time window configured by LMF within DRX cycle containing the DL PRS resource(s) in the assistance data after both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34]. TRSTD,Total

TRSTD,Total

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 4.5.1.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 4.5.5 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD and DL RSCPD measurements are being performed, then the UE shall continue and complete the on-going RSTD and DL RSCPD measurements after the cell selection is completed. The RSTD and DL RSCPD measurement period can be longer.

If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the measurement period then the UE shall continue the RSTD and DL RSCPD measurement in the RRC_CONNECTED state. The RSTD and DL RSCPD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2.

The UE shall meet the DL-RSCPD measurement accuracy requirements in clause 10.x.x.x.

END OF CHANGE #2

START OF CHANGE #3

5.6.7.3	Measurement Capability

The UE PRS RSTD and DL-RSCPD measurement measurement capabilities in RRC_INACTIVE state are as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355 [34].

END OF CHANGE #3

START OF CHANGE #4

#### 5.6.7.5	Measurements Period Requirements

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message with a request to perform rscpd measurement or rscpd with rstd measurement in nr-DL-PRS-MeasurementTimeWindowsConfig, the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.7.3) DL RSCPD or DL RSTD and RSCPD measurements, defined in TS 38.215 [4], within the measurement period  TRSCPD witℎ or witℎout RSTD,Total.

When UE supports DL PRS-RSCPD measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for rscpd measurement:

-	The requirements in the Clause 5.6.2.5 apply to DL RSTD measurement.

-	The requirements in Clause 5.6.7.5 apply to DL RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

When UE supports DL PRS-RSCPD and DL RSTD measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow and LMF configures measurement time window(s) for rscpd and rstd measurements:

-	The requirements in Clause 5.6.7.5 apply to DL RSTD measurement and DL RSCPD measurement performed on the PRS resources in indicated PFL.

- 	The requirements in the Clause 5.6.2.5 apply to DL RSTD measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

Measurement period TRSCPD witℎ or witℎout RSTD,Total is defined as:

TRSCPD with or without RSTD,Total=i=1LTRSCPD with RSTD,i+ L−1∗maxTeffect,i

Where:

-	i is the index of positioning frequency layer indicated in nr-DL-PRS-MeasurementTimeWindowsConfig,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

Measurement period TRSCPD with or without RSTD,i in positioning frequency layer i as specified below:

TRSCPD with or without RSTD=Kcarrier_PRS∗ NRx,TEG∗NRxBeam∗NPRSslotN'∗Lavailable_PRSN∗Nsample−1∗Teffect+Tlast ,

where:

-	Kcarrier_PRS, NRx,TEG, NRxBeam, NPRSslot, N, N’, Nsample, Teffect and Tlast are defined in clause 5.6.2.5

-	Lavailable_PRS is the time duration of available PRS in the positioning frequency layer to be measured during TPRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels and overlapped with the configured PRS measurement time window(s) are considered.

-	When periodic time window(s) are configured by the LMF, Tavailable_PRS=LCMTPRS,TDRX, Twindow, the least common multiple between TPRS , the DRX cycle length TDRX and Twindow being the periodicity of the indicated time window(s). TPRS is defined in clause 5.6.2.5.

-	When periodic time window(s) are not configured by the LMF, Tavailable_PRS=LCMTPRS,TDRX, the least common multiple between TPRS and the DRX cycle length TDRX .  TPRS is defined in clause 5.6.2.5.

-	Lavailable_PRS and TPRS are calculated by only considering the PRS resources in the indicated resources sets overlapping with the indicated time window(s).

The time TRSCPD with or without RSTD starts from the first time window configured by LMF within DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

If the DRX cycle is reconfigured during the measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for the PFL during the measurement period, the measurement period can be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.6.7 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSCPD together with RSTD measurements are being performed, then the UE shall continue and complete the on-going RSCPD and RSTD measurements after the cell re-selection is completed. The measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the measurement period then the UE shall continue the RSCPD and RSTD measurements in the RRC_CONNECTED state. The measurement period can be longer.

END OF CHANGE #4

START OF CHANGE #5

5.6.8.3	Measurement Capability

UE Rx-Tx time difference and DL-RSCP measurement capabilities are as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

END OF CHANGE #5

START OF CHANGE #6

#### 5.6.8.5	Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with a request to perform rscp or rscp with ueRxTx measurement in nr-DL-PRS-MeasurementTimeWindowsConfig, UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.8.3) UE Rx-Tx and RSCP or RSCP measurements, defined in TS 38.215 [4],

within the measurement period TRSCP witℎ or witℎout UERxTx,Total.

When UE supports DL PRS-RSCP measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for rscp measurement:

-	The requirements in the Clause 5.6.4.5 apply to UE Rx-Tx measurement.

-	The requirements in Clause 5.6.8.5 apply to DL RSCP measurement for the PRS resource(s) that have occasions only within the measurement time window.

When UE supports DL PRS-RSCP and UE Rx-Tx measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow and LMF configures measurement time window(s) for rscp and ueRxTx measurements:

-	The requirements in Clause 5.6.8.5 apply to UE Rx-Tx measurement and DL RSCP measurement performed on the PRS resources in indicated PFL.

- 	The requirements in the Clause 5.6.4.5 apply to UE Rx-Tx measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

Measurement period TRSCP witℎ or witℎout UERxTx,Total is defined as:

TRSCP with or without UERxTx,Total=i=1LTDL RSCP with or without UERxTx,i+ L−1∗maxTeffect,i

Where:

-	i is the index of positioning frequency layer indicated in nr-DL-PRS-MeasurementTimeWindowsConfig,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i.

TDL RSCP with UERxTx,iMeasurement period for RSCP with or without UE Rx-Tx measurement in positioning frequency layer i is as specified below:

TDL RSCP with or without UERxTx=Kcarrier_PRS∗NRxTx,TEG,i∗NRxBeam∗NPRSslotN'Lavailable_PRSN∗Nsample−1∗Teffect+Tlast ,

where:

-	Kcarrier_PRS, NRxTx,TEG, NRxBeam, NPRS,islot, N, N’, Nsample, Teffect and Tlast are defined in clause 5.6.4.5.

-	DL-RSCP performed during TDL RSCP with or without UERxTx is a single sample measurement where DL-RSCP and UE Rx-Tx measurements are performed on the same PFL.

-	Lavailable_PRS is the time duration of available PRS resources in the positioning frequency layer, to be measured during TPRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels and overlapped with the configured PRS measurement time window(s) are considered.

-	When periodic time window(s) are configured by the LMF,	Tavailable_PRS= LCMTPRS,TDRX, Twindow, the least common multiple between TPRS , TDRX and Twindow being the periodicity of the indicated time window(s).

-	When periodic time window(s) are not configured by the LMF, Tavailable_PRS=LCMTPRS,TDRX, the least common multiple between TPRS and the DRX cycle length TDRX .

-	Lavailable_PRS and TPRS is as calculated in Clause 5.6.4.5.

The time TDL RSCP with or without UERxTx starts from the first time window configured by LMF within DRX cycle containing the DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the measurement period then the UE shall continue the DL RSCP measurement and shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the measurement period then the UE shall restart the DL RSCP and UE Rx-Tx time difference measurements after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the measurement period then the measurement period can be longer.

If during the measurement period, PRS resources overlap with other DL signals/channels then the measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 5.6.8 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the network-configured Timing Advance command or due to the change in the NTA_offset defined in Table 7.1.2-2 during the measurement period, the UE may continue and complete the DL RSCP measurement.

END OF CHANGE #6

START OF CHANGE #7

#### 9.9.7.3	Measurement Capability

UE PRS RSTD and RSCPD measurement capabilities are as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355[34].

END OF CHANGE #7

START OF CHANGE #8

#### 9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message from the LMF via LPP [34], with a request to perform rscpd measurement or rscpd with rstd measurement in nr-DL-PRS-MeasurementTimeWindowsConfig the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.7.3) DL RSCPD or DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4] within the measurement period  TRSCPD witℎ or witℎout RSTD,Total.

When UE supports DL PRS-RSCPD and DL RSTD measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow and LMF configures measurement time window(s) for rscpd measurement:

-	The requirements in the Clause 9.9.2.5 apply to RSTD measurements.

-	The requirements in Clause 9.9.7.5 apply to RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

When UE supports DL PRS-RSCPD and DL RSTD measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow and LMF configures measurement time window(s) for rscpd and rstd measurements:

-	The requirements in Clause 9.9.7.5 apply to DL RSTD measurement and DL RSCPD measurement performed on the PRS resources in indicated PFL.

- 	The requirements in the Clause 9.9.2.5 apply to DL RSTD measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

Measurement period TRSCPD witℎ or witℎout RSTD,Total defined as:

TRSCPD with RSTD,Total=i=1LTRSCPD with RSTD,i+ L−1∗maxTeffect,i

Where:

-	i is the index of positioning frequency layer indicated in nr-DL-PRS-MeasurementTimeWindowsConfig,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i.

Measurement period TRSCPD with or without RSTD,i in positioning frequency layer i is as specified below:

TRSCPD with or without RSTD=kmultiTEG×CSSFPRS×ceil( Kp,PRS)×NRxBeam×NPRS,islotN'Lavailable_PRS,iN×Nsample−1∗Teffect+Tlast,

where:

DL-RSCPD measurement performed during TRSCPD with or without RSTD is a single sample measurement.

NRxBeam, CSSFPRS, kmultiTEG,  NPRS,islot, N, N’, Nsample, Teffect and Tlast are defined in clause 9.9.2.5.

Nsample

Lavailable_PRS is the time duration of available PRS in the positioning frequency layer to be measured during Tavailable_PRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].

When periodic time window(s) are configured by the LMF, Tavailable_PRS=LCM(Tprs, MGRP, Twindow), otherwise  Tavailable_PRS=LCM(Tprs, MGRP). Twindow is the periodicity of the configured time window(s). Tprs and MGRP are defined in clause 9.9.2.5.

Lavailable_PRS Tprsis calculated by only considering the PRS resources in the indicated resources sets overlapping with both the MG and the indicated time window(s).

Kp,PRS is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS = 1 for UE not configured with concurrent measurement gap.

When periodic time window(s) are configured by the LMF, for a window W of duration max(TPRS,  MGRP_max, Twindow), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion and the periodic time window:

Ntotal is the total number of associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

Otherwise, for a window W of duration max(TPRS,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TRSCPD with or without RSTD starts from the first time window configured by LMF within MG instance aligned with a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TRSCPD with or without RSTD starts from the first time window configured by LMF within MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the RSCPD or RSCPD with RSTD measurement in each reporting period within the time window configured by LMF within MG instance and activate the location report at the time when the periodic timer expires.

If during the measurement period, the MG pattern is reconfigured or time window for RSCPD or RSCPD with RSTD measurement is reconfigured, the measurement period can be longer. When PRS-RSRP is also configured to UE, RSCPD with RSTD and RSRP are performed over the same measurement period.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSCPD with or without RSTD within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.7 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD and RSCPD measurements are being performed together by UE, then the UE shall continue and complete the on-going RSTD and RSCPD measurements. The measurement period for RSCPD with RSTD measurement can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23. The UE shall also meet the RSCPD measurement accuracy requirements in clause 10.1.x.

END OF CHANGE #8

START OF CHANGE #9

9.9.8.3	Measurement Capability

UE Rx-Tx time difference and DL-RSCP measurement capabilities are as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

END OF CHANGE #9

START OF CHANGE #10

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference

When the physical layer receives the last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with a request to perform rscp or rscp with ueRxTx measurement in nr-DL-PRS-MeasurementTimeWindowsConfig, the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.8.3) UE Rx-Tx and DL RSCP or RSCP measurements, defined in TS 38.215 [4], within the measurement period TRSCP witℎ or witℎout UERxTx,Total.

When UE supports DL PRS-RSCP measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for rscp measurement:

- The requirements in the Clause 9.9.4.5 apply to UE Rx-Tx measurement.

- The requirements in Clause 9.9.8.5 apply to RSCP measurement for the PRS resource(s) that have occasions only within the measurement time window.

When UE supports DL PRS-RSCP and UE Rx-Tx measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow and LMF configures measurement time window(s) for rscp and ueRxTx measurements:

- The requirements in Clause 9.9.8.5 apply to UE Rx-Tx measurement and DL RSCP measurement performed on the PRS resources in indicated PFL.

- The requirements in the Clause 9.9.4.5 apply to UE Rx-Tx measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

Measurement period TRSCP witℎ or witℎout UERxTx,Total is defined as:

TDL RSCP with or without UERxTx,Total=i=1LTDL RSCP with or without UERxTx,i+ L−1∗maxTeffect,i

Where:

-	i is the index of positioning frequency layer indicated in nr-DL-PRS-MeasurementTimeWindowsConfig,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i

TDL RSCP with UERxTx,iMeasurement period for DL RSCP with UE Rx-Tx measurement in positioning frequency layer i is as specified below:

TDL RSCP with or without UERxTx=kmultiTEG∗CSSFPRS∗ceil( Kp,PRS)∗NRxBeam∗NPRSslotN'Lavailable_PRSN∗Nsample−1∗Teffect+Tlast ,

where:

DL RSCP performed during TDL RSCP with or without UERxTx is a single sample measurement where DL RSCP and UE Rx-Tx measurements are performed on the same PFL.

NRxBeam, CSSFPRS, kmultiTEG, Nsample, NPRS,islot, N, N’, Teffect and Tlast are defined in clause 9.9.4.5.

Nsample

-	Kp,PRS is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS = 1 for UE not configured with concurrent measurement gap.

When periodic time window(s) are configured by the LMF, for a window W of duration max(TPRS,  MGRP_max, Twindow), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion and the periodic time window:

Ntotal is the total number of associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

Otherwise, for a window W of duration max(TPRS,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

-	Lavailable_PRS is the time duration of available PRS resources in the positioning frequency layer, to be measured during Tavailable_PRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].

-	When periodic time window(s) are configured by the LMF, Tavailable_PRS=LCM(Tprs, MGRP, Twindow), otherwise  Tavailable_PRS=LCM(Tprs, MGRP). Twindow is the maximum periodicity of the configured time window(s). Tprs and MGRP are defined in clause 9.9.8.5.

−Lavailable_PRS and Tprs calculation is as in Clause 9.9.4.5.

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TDL RSCP with or without UERxTx starts from the first time window configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TDL RSCP with or without UERxTx starts from the first time window configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the DL RSCP with UE Rx-Tx time difference measurement in each reporting period and activate the location report at the time when the periodic timer expires.

The DL RSCP with UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of the positioning frequency layer, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TDL RSCP with UERxTx within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.8 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE shall continue and complete the DL RSCP and UE Rx-Tx time difference measurements, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration is complete.

When a serving cell change occurs during the measurement period, the UE shall restart DL RSCP and UE Rx-Tx time difference measurements.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the measurement period, then:

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The DL RSCP with UE Rx-Tx time difference measurement period may be restarted in such case.

END OF CHANGE #10