3GPP TSG-RAN WG4 Meeting # 112 	    R4-2412690

Maastricht, NL, 19 – 23 August 2024

Agenda Item:	6.1.2.5

Source: 	Ericsson

Title:	On performance requirement for PRS/SRS aggregation

Document for:	Approval

Introduction

RAN4#111 discussed performance requirement for bandwidth aggregation-based positioning measurements. The agreements reached are captured in the WF document [1]. In this contribution, group delay calibration margin and frequency drift margin values applicable to RSTD and UE Rx-Tx measurements performed by aggregating resources from multiple PFLs are discussed.

Discussion for RAN4#112

Group delay margin values

In the existing specification, group delay margin values are defined for the RSTD measurements when the PRS resources from reference and target TRPs are in the same PFL or different PFLs. Group delay margin values applicable to RSTD measurement on PRS resources in the same PFL are defined in Table 10.1.23.2-5 for FR1 and Table 10.1.23.2-6 for FR2. And the group delay margin values applicable to RSTD measurement on PRS resources in different PFLs are defined in Table 10.1.23.2-5a for FR1 and Table 10.1.23.2-6a for FR2.

The group delay margin values for single PFL are also defined for the UE Rx-Tx measurements in the existing specification. Group delay margin values applicable to measurements in FR1 and FR2 are defined in Table 10.1.25.2-5 and Table 10.1.25.2-6 respectively.

The group delay margin values for RSTD measurements depend on the bandwidth of the PRS resources and the group delay margin values for UE Rx-Tx measurement depends on the bandwidth of the PRS and SRS resources. Since the existing values depend only on the bandwidth of the resources for measurement, these values can be reused to define the group delay margin for corresponding measurements performed by aggregating resources in multiple PFLs.

Group delay margin values defined in Tables 10.1.23.2-5/5a and 10.1.23.2-6/6a are reused to define group delay margin values for RSTD measurements performed by aggregating PRS resources on multiple PFLs.

Group delay margin values defined in Tables 10.1.25.2-5 and 10.1.25.2-6 are reused to define group delay margin values for UE Rx-Tx measurements performed by aggregating resources on multiple PFLs.

Frequency drift margin values

The performance requirements for PRS aggregation is defined for the scenario where the PFLs with PRS resources for aggregation are intra-band and contiguous to each other and the PRS resources are transmitted/received by using single Tx/Rx branch. The frequency drift at UE while receiving the PRS resources on different PFLs from the same TRP becomes negligible. The frequency drift error for the case where the UE is performing RSTD measurement on the PRS resources from the reference and target TRPs belonging to the same PFL group or different PFL groups shall however be defined. For the latter case, the values of Y from the existing specification can be reused.

RAN4 to define frequency drift margin for RSTD measurement on the PRS resources from the reference and target TRPs belonging to the same PFL group or different PFL groups by reusing the values of Y from the existing specification.

Summary

In this contribution remaining issues related to applicable group delay margin values and frequency drift margin for positioning measurements performed by aggregation resources in multiple PFLs are discussed. Proposals below summarize the discussion presented in this paper.

- Group delay margin values defined in Tables 10.1.23.2-5/5a and 10.1.23.2-6/6a are reused to define group delay margin values for RSTD measurements performed by aggregating PRS resources on multiple PFLs.

- Group delay margin values defined in Tables 10.1.25.2-5 and 10.1.25.2-6 are reused to define group delay margin values for UE Rx-Tx measurements performed by aggregating resources on multiple PFLs.

- RAN4 to define frequency drift margin for RSTD measurement on the PRS resources from the reference and target TRPs belonging to the same PFL group or different PFL groups by reusing the values of Y from the existing specification.

References

- R4-2410192, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson.

Annex

Table 10.1.23.2-5: Margin for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 120 |
| ≥ 52 | ≥ 24 | N/A | 72 |
| ≥ 104 | ≥ 48 | ≥ 24 | 36 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 12 |

Table 10.1.23.2-5a: Margin Δ for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 128 |
| ≥ 52 | ≥ 24 | N/A | 64 |
| ≥ 104 | ≥ 48 | ≥ 24 | 32 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 8 |

Table 10.1.23.2-6: Margin for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 72 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 16 |
| N/A | ≥ 128 | 12 |

Table 10.1.23.2-6a: Margin Δ for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 32 |
| ≥ 64 | ≥ 32 | 16 |
| ≥ 132 | ≥ 64 | 8 |
| N/A | ≥ 128 | 4 |

Table 10.1.25.2-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

Table 10.1.25.2-6: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |