3GPP TSG-RAN WG4 Meeting # 112 	    R4-2412695

Maastricht, NL, 19 – 23 August 2024

Agenda Item:	8.17.3

Source: 	Ericsson

Title:	On AI/ML for positioning

Document for: 	Approval

Introduction

In this contribution RAN4 aspects related to AI/ML for positioning are highlighted. Based on the agreements reached in the earlier meetings, and the progress made in RAN1, issues related to use case 2a and use case 2b are discussed in this paper.

Discussion

Relating to use case 2a and use case 2b, RAN4#110 agreed to come back to issues related to these use cases based on the progress in the other working groups [1].

Agreement:

- RAN4 to come back to case 2a/2b based on progress in the other working groups.

Further discussion on use case 2a/2b in RAN4#111 concluded by reaching to the agreements below [2].

Requirements for case 2a

Agreement:

- No requirements on LMF for positioning accuracy.

- FFS on RAN4 requirements for any UE reported measurements defined by other groups.

Requirements for case 2b

Agreement:

- No requirements on LMF for positioning accuracy.

- FFS on RAN4 requirements for any UE reported measurements defined by other groups.

Requirements for use case 2a

Use case 2a is one of the UE assisted positioning use cases within the scope of Rel. 19 WI on AI/ML based positioning. In this use case, the AI/ML model is deployed at the UE as shown in Figure 1. The UE, when configured to report positioning measurement by LMF, reports the positioning measurement that can be an output of the deployed AI/ML model. TR 38.843 defines this procedure as AI/ML model inference [3]. Based on RAN1 agreement, for use case 2a, AI/ML model inference for positioning can be at least DL-RSTD or UE Rx-Tx time difference measurement defined in TS38.215. Like the core requirements for DL-RSTD and UE Rx-Tx time difference measurement defined by RAN4 for UE assisted non-AI/ML based positioning, RAN4 should define the core requirement applicable for UE assisted AI/ML model inferencing for positioning in use case 2a.

RAN1 agreement from RAN1#116

For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215.

Note: details of the report are pending further discussion.

RAN4 to define core requirement for use case 2a where RSTD measurement is the AI/ML model output.

RAN4 to define core requirement for use case 2a where UE Rx-Tx time difference measurement is the AI/ML model output.

Figure 1. Use case 2a AI/ML based positioning.

AI/ML model needs to be trained before it can be used for inferencing. Unlike non-AI/ML based method, AI/ML based method, in this regard, may induce delay also due to model training, specifically in situations when dedicated resources (in terms of time or transmission and reception of signal to perform measurements or collection of data for AI/ML model training) need to be allocated to UE for model training purposes. Since model training is critical for AI/ML model inference and retraining is prominent aspect relating to LCM of an AI/ML model, it is important that RAN4 takes this into account and discuss delay requirement for AI/ML model training for use case 2a.

- Observation 1:  Training is critical for AI/ML model inference and retraining is prominent aspect relating to LCM of an AI/ML model, it is important that RAN4 takes these aspects into account and discuss delay requirement for AI/ML model training for use case 2a based on the agreement in other WG.

During Rel. 18 SI, when the potential of AI/ML based methods were being evaluated for high accuracy positioning it was observed that the AI/ML based positioning can provide a significant gain in terms of positioning accuracy in comparison to non-AI/ML based positioning [3]. The existing requirements for the RSTD and UE Rx-Tx measurements may therefore be not relevant for the RSTD and UE Rx-Tx measurements reported by UE as AI/ML model inference. To make efficient progress during the performance part of the WI, it is critical that RAN4 starts evaluating achievable accuracies of RSTD and UE Rx-Tx measurements performed by UE by employing AI/ML model for positioning measurements. If it is perceived too early to start such an evaluation, RAN4 can at least start discussion on simulation assumptions that are relevant for AI/ML model inference. In our view, the outcome of the evaluation in terms of achievable accuracy of RSTD and UE Rx-Tx time difference measurement as AI/ML model inference will not only pave a way forward for the work to be done during the performance part of the WI but will also help us understand the relevance of the existing report mapping tables for AI/ML based RSTD and UE Rx-Tx measurement reporting.

Observation 2: The existing requirements for the RSTD and UE Rx-Tx measurements may not be relevant for the RSTD and UE Rx-Tx measurements reported by UE as AI/ML model inference.

RAN4 to at least start discussion on simulation assumptions to evaluate achievable accuracy of AI/ML based RSTD and UE Rx-Tx measurements.

Requirements for use case 2b

Use case 2b is another UE assisted AI/ML based positioning that is within the scope of Rel. 19 WI. In this use case, the AI/ML model is deployed at the LMF as shown in Figure 2. The UE in this use case is configured by the LMF to report the measurements that are used by the LMF as an input to its AI/ML model to inference UE location. As per RAN1 agreement, the measurement that can be reported by the UE for use case 2b can be sample-based and path-based. From the requirements point of view, new core and performance requirements need to be defined for this use case.

Agreement from RAN1# 116bis [4]

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

      - Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

      - Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

The issues to be studied include, but not limited to, the following:

Trade-off of positioning accuracy and signalling overhead

      - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values.

      - Whether the same Alternative(s) applies to all cases or not

      - Applicability and necessity of specifying the Alternative(s) to different cases

      - Note: different sub-cases may have different issues.

Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

Agreement from RAN1# 117 [5]

Sample-based measurement is defined as:

- The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples.

- Nt' and k can be signalled

  - FFS: the value range of Nt'; the value range of integer k for the timing granularity T.

- The timing information is defined relative to a reference time

Further discussion is expected on the determination of Nt' and k (including signaling), and a rule to be introduced for selecting Nt' samples.

Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec.

Agreement from RAN1# 117 [5]

Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

Figure 2. Use case 2b, AI/ML based positioning.

RAN4 to define measurement requirements for path based and sample-based reporting for use case 2b.

Summary

In this contribution, RAN4 specific issues related to AI/ML based positioning use cases 2a and 2b are discussed. The proposals below summarize the discussions presented in this paper.

- RAN4 to define core requirement for use case 2a where RSTD measurement is the AI/ML model output.

RAN4 to define core requirement for use case 2a where UE Rx-Tx time difference measurement is the AI/ML model output.

- Observation 1:  Training is critical for AI/ML model inference and retraining is prominent aspect relating to LCM of an AI/ML model, it is important that RAN4 takes these aspects into account and discuss delay requirement for AI/ML model training for use case 2a based on the agreement in other WG.

- Observation 2: The existing requirements for the RSTD and UE Rx-Tx measurements may not be relevant for the RSTD and UE Rx-Tx measurements reported by UE as AI/ML model inference.

RAN4 to at least start discussion on simulation assumptions to evaluate achievable accuracy of AI/ML based RSTD and UE Rx-Tx measurements.

RAN4 to define measurement requirements for path based and sample-based reporting for use case 2b.

References

- R4-2403712, WF on AI/ML, Qualcomm Incorporated.

- R4-2401570, WF on AI/ML, Qualcomm Incorporated.

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML), 3GPP.

- R1-2403741, Final summary of specification support for positioning accuracy enhancement, Moderator (Ericsson).

- R1-2405389, Summary #5 of specification support for positioning accuracy enhancement, Moderator (Ericsson).