3GPP TSG-RAN WG4 Meeting#112                              R4-2412696

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.20.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on deployment scenarios and spectrum usage

Document for:	Approval

Introduction

In the RAN#102 meeting, Study on solutions for Ambient IoT (Internet of Things) in NR has been approved and it’s expected to start the normative work from this April meeting. In the last RAN4#110bis meeting, we have reached some initial consensus on the deployment scenarios and spectrum usage, however there are still some remaining issues. In this contribution, we would like to share some further views on these remaining issues.

RAN4#110bis meeting: Way forward for co-existence evaluation - Topic 2-1: Deployment scenario Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting) Agreement: - RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2. Issue 2-1-2: deployment scenarios for D2T2 Option 1-1: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is only allowed outdoor. Option 1-2: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is indoor. Agreement: - For D2T2 co-existence evaluation, Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. - Consider option 1-1 and option 1-2 as the starting point - Topic 2-2: Spectrum usage Issue 2-2-1: Spectrum usage for R2D in D1T1 Option 1: FDD DL spectrum for R2D Option 2: FDD UL spectrum for R2D Agreement: - FFS on whether to prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation. Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, consider the following for co-existence evaluation - CW transmits in either UL or DL spectrum - FFS on inside topology and outside topology. Issue 2-2-4: Spectrum usage for R2D in D2T2 Agreement: - Use FDD UL spectrum for R2D in D2T2. Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. · Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum · Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum · Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering - Use UL spectrum as the starting point for co-existence evaluation. - It won’t preclude the use of DL for backscattering transmission. - FFS on the minimum distance between the intermediate UE and A-IoT device - Topic 2-3: Spectrum deployment mode Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation Agreement: - Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation - A-IoT is located within a NR transmission bandwidth configuration - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS - Topic 2-1: Deployment scenario Issue 2-1-1: deployment scenarios for D1T1 Agreement in RAN4#110bis: Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting) Agreement: - RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2. Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation Agreement: - Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation - A-IoT is located within a NR transmission bandwidth configuration - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS. Agreement in RAN4#111: - Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT - Encourage companies to provide the simulation results for option 1-1 and 1-2 - FFS on co-site scenario (option 2-1 and 2-2) Issue 2-1-2: deployment scenarios for D2T2 Agreement in RAN4#110bis: Option 1-1: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is only allowed outdoor. Option 1-2: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is indoor. Agreement: - For D2T2 co-existence evaluation, Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. - Consider option 1-1 and option 1-2 as the starting point. - Topic 2-2: Spectrum usage Issue 2-2-1: Spectrum usage for R2D in D1T1 Agreement in RAN4#110bis: - FFS on whether to prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation.. Agreement in RAN4#111: - Use FDD DL as starting point for co-existence evaluation for R2D in D1T1 - FFS on FDD UL spectrum. Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering Agreement in RAN4#110bis: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, consider the following for co-existence evaluation - CW transmits in either UL or DL spectrum - FFS on inside topology and outside topology.. Agreement in RAN4#111: - Use inside topology as starting point for co-existence evaluation (case 1-1, case 1-2) for calibration. - Further discuss the difference of outside topology (case 1-4) from co-existence study perspective. Issue 2-2-4: Spectrum usage for R2D in D2T2 Agreement in RAN4#110bis: - Use FDD UL spectrum for R2D in D2T2.. Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering Agreement in RAN4#110bis: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. · Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum · Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum · Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering - Use UL spectrum as the starting point for co-existence evaluation. - It won’t preclude the use of DL for backscattering transmission. - FFS on the minimum distance between the intermediate UE and A-IoT device. Agreement: - Use case 2-2 as starting point for co-existence evaluation for calibration. - FFS on case 2-3 - Further discuss the difference of outside topology (case2-4) from co-existence study perspective.

Discussion

- 2.1. Spectrum usage

Issue 2-2-1: Spectrum usage for R2D in D1T1

Agreement in RAN4#110bis: - FFS on whether to prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation.

Agreement in RAN4#111:

- Use FDD DL as starting point for co-existence evaluation for R2D in D1T1

  - FFS on FDD UL spectrum.

ZTE: during the last RAN4 meeting, there were some discussions on DL/UL spectrum for R2D transmission in D1T1. If R2D signal in D1T1 could be transmitted by the uplink spectrum, then the interference towards the device from NR would be originating from UE transmissions which will make the interference limited or lower compared with NR BS transmission interference. However considering the coexistence assumptions for D1T1 with NR Macro BS deployed outside the factory and A-IOT BS/device are isolated within the indoor factory, then isolation distance and also penetration loss across the factory wall will help mitigate the NR BS interference a lot. In addition, considering the ratio of indoor UE which might transmit with highest power to compensate the penetration loss, then the interference in the uplink spectrum from indoor UE transmission might be also comparable with the interference in the downlink spectrum from outdoor BS transmission. In short, either DL or UL spectrum for R2D transmission are fine for us if the NR BS/UE interference are acceptable or manageable for A-IoT device reception.

Proposal 1: both FDD DL or UL spectrum are fine for R2D transmission if there are no regulatory power limitation/restriction in the uplink spectrum.

Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Agreement in RAN4#110bis: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, consider the following for co-existence evaluation - CW transmits in either UL or DL spectrum - FFS on inside topology and outside topology.

Agreement in RAN4#111:

- Use inside topology as starting point for co-existence evaluation (case 1-1, case 1-2) for calibration.

- Further discuss the difference of outside topology (case 1-4) from co-existence study perspective.

ZTE:

Regardless from CW signal transmission from DL or UL or inside topology or outside topology, it should be clarified that R2D signal transmission and CW signal transmission should be in TDM manner from the procedure wise design as shown in the following figure, however whether different A-IoT BS will have the same R2D/CW transmission timeline, this should be further clarified. The simplest assumptions should be that transmission timeline for R2D and CW signal among different A-IoT BS/CW node transmission are aligned, then we only need to consider the co-channel interference from R2D transmission or D2R transmission.

Proposal 2: it’s necessary to clarify that R2D transmission and CW transmission timeline among different A-IoT BSs or CW nodes within single indoor factor are aligned or not.

In addition, during the last RAN4 meeting, there were some discussions on difference of outside of topology and inside topology from coexistence study perspective. As shown in the following figure, for inside topology, A-IoT BS transmit the R2D signal and CW signal in TDM manner, once the device is far away from the serving BS, then its backscattering signal towards the serving BS will be also very weak e.g. Pw-PL-reflection loss-PL, however for outside of topology as shown in the following figure, if CW node is deployed at the cell edge, then its transmission power of cell edge device will be higher due to the small coupling loss from CW node which is quite similar as the legacy power control in the NR network.

Observation 1: the backscattering transmission power from the device are different between inside topology and outside of topology which may cause some different evaluation results for coexistence sharing study. .

However it’s not clear how to assume the outside the topology in the network which is up to the operator’s decision, therefore before we reached the consensus or common understanding on the assumptions for outside of topology, we prefer to use the inside topology as baseline for the coexistence evaluation.

Proposal 3:before reaching the consensus on assumption for outside of topology, propose to use the inside topology as baseline for the coexistence evaluation.

Conclusions

In this contribution, we shared some further views on the deployment scenarios and spectrum usage and proposals are made as following:

Proposal 1: both FDD DL or UL spectrum are fine for R2D transmission if there are no regulatory power limitation/restriction in the uplink spectrum.

Proposal 2: it’s necessary to clarify that R2D transmission and CW transmission timeline among different A-IoT BSs or CW nodes within single indoor factor are aligned or not.

Observation 1: the backscattering transmission power from the device are different between inside topology and outside of topology which may cause some different evaluation results for coexistence sharing study.

Proposal 3:before reaching the consensus on assumption for outside of topology, propose to use the inside topology as baseline for the coexistence evaluation.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2406714,WF on Ambient IoT in NR,CMCC, Approved.

- R4-2410567,WF on co-existence study for ambient IoT and NR/LTE, Approved.