3GPP TSG-RAN WG4 Meeting#112                              R4-2412701

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.10.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	RF requirement for Intra-band contiguous CA of ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions for ATG UE supporting intra-band CA. In this contribution, we would like to share some further views on these remaining issues.

Is Issue 2-1-1: BCS for DL CA_n79C Agreement: - Reuse the existing BCS table for ATG - Clarify that uplink CA is not supported for ATG UE elsewhere for Rel-19 - FFS on whether BCS#4 and #5 are not supported for ATG UE for Rel-19 Issue 2-1-2: whether/how to reflect operation band information in spec Agreement: - The information that only DL CA is supported in Rel-19 for ATG can be reflected in the clause of operating band for ATG Issue 2-2: ATG Rx requirements applicability Agreement: - Following requirements needs to be updated for ATG CA compared with TN CA. - Max input level - ACS testing case 2 - Following requirements can reuse legacy TN CA requirements - Blocking including in-band and out of band blocking, narrow-band blocking - REFSENSE - Spurious response - Intermodulation - Following requirement is not applicable for ATG CA - No CA Rx Spurious emission - No CA narrow-band blocking - Following single carrier requirement of ATG is enough and no need to define ATG CA specific requirements - diversity characteristics requirement Issue 2-3: max input level Agreement: Table 3: Maximum input level for ATG with CA_n79C Rx Parameter; Units; ATG UE Types. Omni-directional antenna; Antenna array. Power in largest transmission bandwidth configuration CC, Plargest BW; dBm; -422; -302. -443; -323. Power in each other CC; dBm; Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}. The applicable NR CA Bandwidth Class; MHz; C. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM.. Issue 2-4: ACS testing parameters for case 2 Agreement: - the principle of TN contiguous CA could be reused

Discussion

- 2.1. BCS for CA_n79C

Usually for BCS configuration of certain band combination, it’s up to operator’s decision or request. From the practical market perspective, it seems that minimum channel bandwidth to be supported is 40MHz and 10/20/30MHz might be not necessary. In addition, as we know that 70MHz channel bandwidth is somehow optional. if there are no practical market request for 10/20/30MHz and 70MHz channel bandwidth, BCS0 could be used for band combination n79C.

Proposal 1: use BCS0 for band combination n79C if there are no practical market demand for 10/20/30/70MHz channel bandwidth.

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA config | Uplink CA config | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | BCS |
| CA_n79C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |

- 2.2. RF requirements for Intra-band CA

In this section, we provide some initial analysis on the overall requirement for ATG UE supporting intra-band DL CA and also provided the summary of proposals for it.

Table 2. Analysis and summary of proposals for ATG UE supporting intra-band DL CA

| The existing RF requirement for intra-band contiguous CA | ATG UE supporting intra-band contiguous CA |
| --- | --- |
| 7.3A.2.1 Reference sensitivity power level for Intra-band contiguous CA | Reference sensitivity power level for Intra-band contiguous CA For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3.2-3. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. |
| 7.4A.1 Maximum input level for Intra-band contiguous CA | Maximum input level for Intra-band contiguous CA For intra-band contiguous carrier aggregation maximum input level is defined as the maximum mean power received at the UE antenna port, over the Transmission bandwidth configuration of each CC. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4A.1-1 for each component carrier. Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA Rx Parameter; Units; NR CA Bandwidth Class. B; C; D. Power in largest transmission bandwidth configuration CC, Plargest BW; dBm; [-42]2 for omini antenna [-30] for array antenna; [-42]2 for omini antenna [-30] for array antenna; N/A. [-44]3 for omini antenna [-32] for array antenna; [-44]3 for omini antenna [-32] for array antenna; N/A. Power in each other CC; dBm; Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM.. |
| 7.5A.1 Adjacent channel selectivity for Intra-band contiguous CA | To follow the existing requirement defined in clause 7.5A.1 with the following power to be updated for ATG UE with agreed maximum input power for Case 2 and also scale with wanted signal power level. - Pw in Transmission Bandwidth Configuration, per CC - PInterferer |
| 7.6A.2.1 In-band blocking for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.6A.2.1. |
| 7.6A.3.1 Out-of-band blocking for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.6A.3.1. |
| 7.6A.4.1 Narrow band blocking for Intra-band contiguous CA | N/A |
| 7.7A.1 Spurious response for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.7A.1. |
| 7.8A.2.1 Wide band intermodulation for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.8A.2.1. |

Conclusions

In this contribution, we shared some further views on the RF requirements for ATG UE supporting intra-band DL CA and proposals are made as following:

Proposal 1: use BCS0 for band combination n79C if there are no practical market demand for 10/20/30/70MHz channel bandwidth.

Proposal 2: for RF requirements for inter-band DL CA, refer to the proposals in table 2.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.