3GPP TSG-RAN WG4 Meeting#112                              R4-2412707

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.11.2

Source: 	ZTE Corporation,Sanechips

Title: 	TP to TR 38.908 Background of U6GHz EEIRP mask requirement

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In this TP, the background for U6GHz EEIRP mask requirements are provided.

<Start of the change>

- Background

The frequency band of 6 425-7 125 MHz was identified in every ITU region as in the RR Footnotes 5. 6A12, 5. 6B12 and 5.6C12 with associated technical condition of limits on the expected equivalent isotropically radiated power (e.i.r.p.) spectral density of IMT base-stations for protecting Earth-to-space fixed satellite services (FSS) as in Resolution COM4/7 (WRC-23). The expected e.i.r.p. is a new regulatory requirement and the Annex to the Resolution COM4/7 (WRC-23) outlines the theoretical calculation of the expected equivalent isotropically radiated power (e.i.r.p.) of an International Mobile Telecommunications (IMT) base station for assessing the compliance of IMT base station equipment with the limit on expected e.i.r.p. When developing the expected e.i.r.p, it is expected that 3GPP to develop the harmonized specification for compliance testing.

The aim of this task is as following:

- to specify the core requirement in BS (and other nodes like IAB, Repeater and NCR) RF core specification (TS 38.104)

- to specify the test procedures in the conformance test specification (TS 38.141-2) to measure the expected EIRP mask performance.

- Having the requirement included in the 3GPP standard would guarantee a harmonized terminology and conformance test method. Eventually, the concept for conformance testing can be adopted to similar situations with co-channel spectrum sharing between IMT and FSS UL.

<End of the change>