3GPP TSG-RAN WG4 Meeting#112                              R4-2412710

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.2.3

Source: 	ZTE Corporation

Title: 	Discussion on IMT parameters for 7125-8400MHz

Document for:	Approval

Introduction

In the latest ITU-R meeting, WP5D has sent one LS to RAN/RAN4 to request the Parameters of terrestrial component of IMT in the following frequency ranges for sharing and compatibility studies. During the RAN#103 meeting, the corresponding SID for WP5D reply LS [3] is approved to facilitate the technical discussions in RAN4. In last RAN4 meeting, we reached some progress for 7125-8400MHz, however there are still lots of open issues. Therefore in this contribution, we would like to share some further views on these remaining issues.

Way forward at RAN4#110bis meeting: - n104 as starting point o Identify which parameters need further discussion and could differ from n104 Issue 2-1 Duplex Method - Option 1: Align with 4GHz outcome, i.e., TDD as baseline, check SBFD - Option 2: Follow n104 Issue 2-2 Typical Channel Bandwidth (100MHz is maximum bandwidth) - n104 (100MHz) baseline - FFS higher bandwidths (e.g. 200MHz) Issue 2-3 Transmit bandwidth configuration (Signal bandwidth) - n104 spectrum utilization assumed - FFS check for higher bandwidth Issue 2-4 Power dynamic range (UE) - Use n104, as appropriate for maximum output power Issue 2-5 Spectrum mask - BS: - n104 as basis for emissions levels - ΔfOBUE and ΔfOOB for the BS side can be considered further. - UE: - n104 as basis for emissions levels Issue 2-6 ACLR - BS - Use n104 - UE - Option 1: 26dB, 27dB (study) for PC3 - Option 2: 30dB (n104) for PC3, 31dB (n104) for PC2 Issue 2-7 Spurious Emissions - BS: - Use n104 - UE: - Use n104 Issue 2-8 Maximum output power (UE) - Option 1: 23dBm only. - Option 2: 20dBm - Option 3: Use n104 (23 and 26dBm) - Option 4: 29dBm Issue 2-9 Noise Figure - UE - Option 1: Follow n104 noise figure (12dB) - Option 2: Be consistent with information sent previously IMT-2020 28GHz, e.g. 10dB - Option 3: Be consistent with Previous LS to ITU-R on 6, 10GHz, NF was 9-13dB - BS - Option 1: Follow 38.820, i.e., 6dB WA, 11dB MR, 14dB LA - Option 2: Follow n104 if different Issue 2-10 Sensitivity - To be discussed further Issue 2-11 Blocking response - BS: - Follow n104 - Discuss ΔfOOB - UE: - FFS Issue 2-12 ACS - UE: - Follow n104 or follow previous studies - BS: - Follow n104 (42dB) Issue 2-13 SINR operating range - To be discussed further Issue 2-14: Deployment scenarios to consider - Include urban and sub-urban macro, but double check coverage Way forward at RAN4#111 meeting: Sub-topic 2-1 General / system issues Issue 2-1: Duplex mode Agreement: - Do not include SBFD in the LS, but add information in the TR. Add a reference to the TR in the LS Issue 2-3: Typical channel bandwidth Agreement: - Mention 100MHz as typical bandwidth in the LS - FFS whether to mention wider bandwidths may be possible in future in the LS - Describe other possibilities for bandwidth in the TR, mentioning that additional specification effort needed for >100MHz Issue 2-4: Typical signal bandwidth Outcome of discussion (Not agreed): Option 1: 273 RB, 30k SCS as 38.104 Option 2: Quote formula of RBs * SCS without number of RBs Option 3: Directly quote the signal bandwidth in MHz Issue 2-6: Deployment scenarios Agreement: - Confirm Urban macro and urban micro should be considered - This is based on assumed antenna size that is sufficient for 450m or lower ISD for urban macro Sub-topic 2-2 BS parameters Issue 2-7: Emissions mask Agreement: - AAS BS: - Use n104 unwanted emissions mask - [Aim to use 100MHz for delta_obue, conclude feasibility until RAN4#112] - Non-AAS BS: - [n104 unwanted emissions mask for IMT response] - Double check on whether there are any concerns towards sharing by RAN4#112 and in case concerns are identified, consider n96 if needed to resolve concerns Issue 2-10: Noise figure Agreement: - Follow 38.820, i.e., 6dB WA, 11dB MR, 14dB LA for noise figure of BS Issue 2-11: Sensitivity - Proposals - Option 1: As for n104 in 38.104 (CMCC, Samsung, Nokia) Agreement: - Agree on option 1. - Capture it in the TR only. Issue 2-14: BS antenna array parameters Agreement: Rural macro (If it’s available) Parameters proposed by ZTE; Suburban macro; Urban macro; Urban small cell (outdoor)/Micro cell; Indoor (small cell). 1; Base station antenna characteristics. 1.6; Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements; Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Spark) Option 2: 8 x 8 (ZTE, Samsung); Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Cable Labs) Option 2: 8 x 8 (ZTE, Samsung); 8 × 8; 4 × 4. 1.7a; Number of element rows in sub-array, Msub; Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung); Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung); N/A; N/A. Agreement: - Remove the rural macro scenario from the table. Agreement: - non-AAS can be used for Urban small cell/Micro cell scenario - non-AAS can be used for indoor scenario - FFS on where to capture the above two bullets Sub-topic 2-3 UE parameters Issue 2-15: Maximum output power Tentative agreement: - Capture 23dBm as typical maximum output power in the LS to ITU. - Other output powers are not precluded and refer to TR - Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

Discussion

## 2.1. System parameters

Regarding the channel bandwidth and transmission bandwidth configuration, even though this frequency range might be targeted for the usage of both IMT-2020 and IMT-2030, this is still discussed under the 5G-A phase and it’s not clear on 6G system parameter assumption, therefore we propose to follow NR baseline assumptions for 7125-8400MHz with maximum channel bandwidth as 100MHz for 30kHz SCS and 50MHz for 15kHz.

Regarding the higher channel bandwidth e.g. 200MHz per carrier, from our understanding, it might be feasible to have 200MHz carrier bandwidth with FFT size as 8192 and SCS as 30kHz for 6G technology, however for 5G-A, we cannot change the basic assumption for maximum FFT size as 4096. In other words, we could indicate the potential lager channel bandwidth for IMT-2030, however we cannot agree the specific value and spectral utilization for it according to the current situation.

In short, we can follow the assumption as agreed for 6425-7125MH and 10-10.5GHz as following.

TABLE 3A

IMT-2020 technology related parameters in 6425 – 7125 MHz and 10 – 10.5 GHz

|  |  | IMT |  |
| --- | --- | --- | --- |
| No. | Parameter | Base station (AAS) | Mobile station |
| 1 | Duplex Method | TDD | TDD |
| 2 | Channel bandwidth (MHz) | 100 MHz (typical) | 100 MHz (typical) |
| 3 | Signal bandwidth (MHz) | To be specified. Will be derived from Channel Bandwidth, see [1], § 5.3.2. | To be specified. Will be derived from Channel Bandwidth, see [2], § 5.3.2. |

Proposal 1: for carrier bandwidth and transmission bandwidth configuration, reuse NR channel bandwidth and transmission bandwidth configuration as baseline (e.g. 100MHz for 30kHz SCS as typical value) for IMT-2020, and indicate the potential lager channel bandwidth for IMT-2030 in TR.

## 2.2. BS RF requirements

Regarding the BS RF requirements for 7125-8400MHz, we reached the consensus on most of RF requirements, however for the following two requirements, the feasibility should be confirmed until RAN4#112 meeting.

- f_OBUE requirement

- f_OOBB requirement

In the following, we provide some front-end filter response evaluation result according to the f_OBUE and f_OOBB requirement of band n104 in Figure 2.2-1 and the corresponding filtering size evaluation in Figure 2.2-2. The expected ceramic filter size to follow the legacy requirement of band n104 is 8 orders with 4 poles and its filter size would be 22.5mm (length) x9.33mm (width) x5.3mm(height). The insertion loss within any 100MHz of 7125-8400MHz is 0.8dB in normal temperature and 1dB in the extreme temperature. The length of the ceramic size of 7125MHz to 8400MHz by following the legacy band n104 requirement is much longer compared that of band n104 according to our evaluation results. The main reason is that the relative channel bandwidth for band n104 is around 10.33%, however the relative channel bandwidth of 7125-8400MHz increased up to 16.43% which result in the relative bigger filter size.

Figure 2.2-1. analog filter response with f_OBUE and f_OOBB as 100MHz

Figure 2.2-2. analog ceramic filter internal structure and its size with f_OBUE and f_OOBB as 100MHz

According to the above concerns and also try to align the RF layout architecture between band n104 and 7125-8400MHz, we propose to relax the f_OOBB requirement to 200MHz and f_obue requirement as 100MHz considering the higher relative channel bandwidth of 7125-8400MHz. We provide the updated front-end ceramic filter response evaluation result in Figure 2.2-3 and the corresponding filtering size evaluation in Figure 2.2-4. The expected ceramic filter size is 6 orders with 4 poles and its filter size would be 15mm (length) x10mm (width) x5.3mm(height).

Figure 2.2-3. analog filter response with f_OBUE 100MHz and f_OOBB as 200MHz

Figure 2.2-4. analog ceramic filter internal structure and its size with f_OBUE 100MHz and f_OOBB as 200MHz

Proposal 2: to define f_OBUE as 100MHz and f_OOBB requirement as 200MHz of BS type 1-H for 7125-8400MHz;

In the last RAN4 meeting, there were some discussions on the BS antenna array parameters for 7125-8400MHz, there were no consensus reached yet between option 1 and option 2. According to the initial SLS evaluation results shown as following without considering the ACI. The DL coverage is still acceptable even with antenna array size as 8x8 when cell ISD is limited as 450m as shown in Figure 2.2-5, however for uplink coverage as shown in Figure 2.2-6, there will be roughly around 5% UE’s SINR below -10dB if antenna array size assumed as 8x8. In other words, if we consider the additional ACI from other system, then its cell edge throughput or 5% throughput will be further degraded.

Figure 2.2.5. DL SINR for the comparison between antenna array size 8x8 and 8x16 with ISD as 450m

Figure 2.2.6. UL SINR for the comparison between antenna array size 8x8 and 8x16 with ISD as 450m

Figure 2.2.72. UL transmission power for the comparison between antenna array size 8x8 and 8x16 with ISD as 450m

According to the above evaluation results, we think that beamforming antenna characteristic for 7125-8400MHz could follow the reply LS [8] to WP5D on 6425-10500MHz to ensure its coverage especially for uplink coverage perspective.

Proposal 3: regarding BS antenna array for 7125-8400MHz, propose to follow the previous reply LS R4-2103104 to WP5D on 6425-10500MHz.

TABLE 4

Beamforming antenna characteristics for IMT in 6425 – 10500 MHz

|  |  |  | Rural |  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |  |  |
| 1.1 | Antenna pattern |  | Refer to Recommendation ITU-R M.2101 |  |  |  |  |
| 1.2 |  | Element gain (dBi) (Note 1) | N/A |  | 6.4 | 5.5 | 5.5 | 5.5 |
| 1.3 |  | Horizontal/vertical 3 dB beamwidth of single element (degree) | N/A |  | 90º for H 65º for V | 90º for H 90º for V | 90º for H 90º for V | 90º for H 90º for V |
| 1.4 |  | Horizontal/vertical front-to-back ratio (dB) | N/A |  | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 |  | Antenna polarization | N/A |  | Linear ±45º | Linear ±45º | Linear ±45º | Linear ±45º |
| 1.6 |  | Antenna array configuration (Row × Column) | N/A |  | 16 × 8 elements | 16 × 8 elements | 8 × 8 elements | 4 × 4 elements |
| 1.7 |  | Horizontal/Vertical radiating element spacing | N/A |  | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.8 |  | Array Ohmic loss (dB) (Note 1) | N/A |  | 2 | 2 | ## 2 | 2 |
| 1.9 |  | Conducted power (before Ohmic loss) per antenna element (dBm) | N/A |  | 22 (Note 5) | 22 (Note 5) | 16 (Note 6) | 9 (Note 7) |
| 1.10 |  | Base station maximum coverage angle in the horizontal plane (degrees) | N/A |  | 120 | 120 | 120 | N/A (Note 8) |
| 1.11 |  | Base station vertical coverage range (degrees) (Note 3, 4) | N/A |  | 90-100 | 90-120 | 90-120 | N/A (Note 8) |
| 1.12 |  | Mechanical downtilt (degrees) (Note 4) | N/A |  | 6 | 10 | 10 | N/A (Note 8) |

## 2.3. UE RF requirements

Regarding the UE RF requirement for 7125-8400MHz, the following requirement should be discussed with high priorities:

- Maximum output power

- Power dynamic range

- Spectral mask

- ACLR

- Noise figure

- Sensitivity

- ACS

- Blocking

Regarding the maximum output power or UE power class for 7125-8400MHz, we have some initial discussions in last RAN4 meeting and reached the following tentative agreements. From the ITU-R WP5D compatibility sharing study, the interference caused from the active UEs are limited compared with the BS transmission power. In other words, to capture 23dBm as typical maximum output power in LS to ITU looks fine for us, however considering this band is high band next to FR1 frequency range. To make sure its uplink coverage and also its PA performance similarities with band n104, then both PC3 and PC2 should be supported at least. PC1.5 is also preferred to ensure the comparable coverage as other lower FR1 bands.

Issue 2-15: Maximum output power Tentative agreement: - Capture 23dBm as typical maximum output power in the LS to ITU. - Other output powers are not precluded and refer to TR - Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

Proposal 4: at least PC3 and PC2 should be supported for 7125-8400MHz; PC1.5 is also preferred.

Regarding the power dynamic range requirement, assuming the same minimum output power would be applicable for 7125-8400MHz, then 56dB/59dB power dynamic range should be applied according to the -33dBm/100MHz minimum transmission power.

Proposal 5: propose 56dB/59dB/62dB power dynamic range for PC3/PC2/PC1.5 respectively according to the -33dBm/100MHz minimum transmission power.

Regarding the ACLR, SEM requirement for 7125-8400MHz, similar reasons as previous requirements, the same ACLR and SEM requirement for band n104 could be reused for 7125-8400MHz. It should noted that the ACLR requirement and SEM requirement captured in TR 38.921 is somehow different from the final RF requirements for band n104 [6].

According to the simulation results in clause 4.3, it is agreed to specify 26 dB ACLR for 6.425 - 7.125 GHz and 24 dB ACLR for 10.0 - 10.5 GHz.

Proposal 6: reuse the same ACLR and SEM requirement for band n104 for 7125-8400MHz instead of referring to TR 38.921.

Regarding the NF and sensitivity requirements for 7125-8400MHz, this need more discussions since this insertion loss and the impacts of PCB material for this frequency range might be new to the market.

Proposals 7: more discussions are needed for NF and sensitivity.

Regarding the ACS requirements for 7125-8400MHz, similar reasons as previous requirements, the same ACS requirement for band n104 could be reused for 7125-8400MHz. It should noted that the ACS requirement captured in TR 38.921 is somehow different from the final RF requirements for band n104 [6].

According to the simulation results in clause 4.3, adjacent channel selectivity (ACS) is agreed as 32 dBc for 6425 – 7125 MHz and 31 dBc for 10 - 10.5 GHz.

Proposal 8: reuse the same ACS requirements for band n104 for 7125-8400MHz instead of referring to TR 38.921.

For IBB and OOBB requirement for 7125-8400MHz, this is different from 6425-7125MHz since this band is new band and not overlapping with any exiting unlicensed band, therefore we propose not to follow the existing requirement for band n104 and further discuss the exact requirement for it.

Proposal 9: for IBB and OOBB requirements, not reuse the existing band requirement for band n104 and further discuss the exact requirement for it.

Conclusions

In this contribution, we want to share some further views on IMT parameters for 7125-8400MHz and proposals/observations are made as following:

Proposal 1: for carrier bandwidth and transmission bandwidth configuration, reuse NR channel bandwidth and transmission bandwidth configuration as baseline (e.g. 100MHz for 30kHz SCS as typical value) for IMT-2020, and indicate the potential lager channel bandwidth for IMT-2030 in TR.

Proposal 2: to define f_OBUE as 100MHz and f_OOBB requirement as 200MHz of BS type 1-H for 7125-8400MHz;

Proposal 3: regarding BS antenna array for 7125-8400MHz, propose to follow the previous reply LS R4-2103104 to WP5D on 6425-10500MHz.

Proposal 4: at least PC3 and PC2 should be supported for 7125-8400MHz; PC1.5 is also preferred.

Proposal 5: propose 56dB/59dB/62dB power dynamic range for PC3/PC2/PC1.5 respectively according to the -33dBm/100MHz minimum transmission power.

Proposal 6: reuse the same ACLR and SEM requirement for band n104 for 7125-8400MHz instead of referring to TR 38.921.

Proposal 8: reuse the same ACS requirements for band n104 for 7125-8400MHz instead of referring to TR 38.921.

Proposal 9: for IBB and OOBB requirements, not reuse the existing band requirement for band n104 and further discuss the exact requirement for it.

References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz.

- RP-210037, Reply LS to WP5D, Approved.

- RP-240787,New SI proposal: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz, Approved.

- TR 38.921

- https://www.itu.int/en/ITU-D/Regional-Presence/AsiaPacific/Documents/Events/2017/Aug-ISS2017/PAPER_Workshop_S3_Timur.pdf

- R4-2211224, Introduction of NR licensed band 6425 -7125 MHz, Approved.

- R4-2406615, WF on IMT parameters for other frequency ranges, CATT, Approved.

- R4-2103104/RP-210037 Reply LS to ITU-R WP5D