3GPP TSG-RAN WG4 Meeting#112                              R4-2412711

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.2.4.1

Source: 	ZTE Corporation

Title: 	Discussion on co-existence evaluation for 14800 to 15350 MHz

Document for:	Approval

Introduction

In the latest ITU-R meeting, WP5D has sent one LS to RAN/RAN4 to request the Parameters of terrestrial component of IMT in the following frequency ranges for sharing and compatibility studies. During the RAN#103 meeting, the corresponding SID for WP5D reply LS [3] is approved to facilitate the technical discussions in RAN4. In the previous RAN4 meeting, there were no much discussions for coexistence evaluations for 14800MHz to 15350MHz due to the TU limitation. In this contribution, we would like to share some views on coexistence evaluations for 14800-15350MHz.

Way forward - Further discussing the following issues Issue 3-1 Common understanding on UE antenna array options - Option 1: “UE beamforming” (FR2 like) - Assumed to be FR2 like UE, with two panels similar to Rel-14 NR study 38.803 - Analogue like beamforming - Option 1a: 2x2 antennas per panel - Option 2: FR1 like - Assumed to be FR1 like UE with more than one TX/RX antenna - Isotropic antennas - Option 2a: 2 TX - Option 2b: 4 TX - Option 2c: 4 RX - Option 2d: 6 RX - Option 2e: 8 RX Issue 3-2 Initial deployment scenarios - Option 1: Follow 38.921 - Indoor 20m ISD - [0.2 - 0.45]km Urban - [<=0.9]km sub-urban - Option 2: 38.803 FR2 assumptions - Indoor - Dense Urban - Urban macro - Other options such as smaller ISD not precluded depending on antenna array assumptions Issue 3-3 Initial coordinated operation - Option 1: - Co-ordinated for indoor - Both co-ordinated and un-co-ordinated for outdoor Issue 3-4 Initial views on BS array size - Option 1: 1024 elements - Option 2: 2048 elements - Option 3: Other sizes Issue 3-5 Initial views on BS sub-array architecture - Option 1: No sub-arrays - Option 2: Sub array size 2 - Option 3: Sub array size 3 - Option 4: Sub-array size 4 - Option 5: Sub-array size 6 - Option 6: Sub-array size 8 - Option 7: Sub-array size 12 - Agreement: Issue 3-1: Simulation scenarios Agreement: - 1st priority urban macro - 2nd priority indoor - 3rd priority dense urban Issue 3-3: ISD Agreement: - For indoor, agree 20m - For outdoor, consider both 450m (1st priority) and 350 (2nd priority) until August. - Also other ISD not precluded as 3rd priority than 450 and 350. Issue 3-4: Percentage indoor users for urban macro Agreement: - Keep both options Issue 3-5: Co-ordinated and un-coordinated for outdoor Agreement: - Both co-ordinated and un-coordinated Issue 3-6: Co-ordinated and un-coordinated for indoor Agreement: - Only co-ordinated Issue 3-7: Pathloss model Agreement: - Follow 38.901 Issue 3-6: BS antenna array size Agreement: Per polarization, for simulation only: - 2048 (Sub Array size 8) - 1536 (Sub Array size 4) - Array size and Sub Array size for the response will be decided later, taking into account feasibility Issue 3-7: BS antenna array other parameters Agreement: - 0.7 spacing in vertical domain is for simulation assumptions, double check for reply Issue 3-8: UE type Agreement: For simulations, consider both options. - FR1 like - RX diversity gain: [5] dB assuming 4RX - TX: 0dBi omnidirectional - Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm TRP Issue 3-9: BS output power Agreement: - Assume 43dBm BS power for simulations Issue 3-10: Bandwidth Agreement: - For number of UEs per slot: - 1 UE per slot as 1st priority - 3 UE per slot as 2nd priority Issue 3-11: BS noise factor Agreement: - For simulations, assume 8dB Issue 3-11: UE noise factor Agreement: - 11dB for simulations. Actual noise factor for reply will be decided based on feasibility.

Discussion

## 2.1. Simulation assumptions

In last RAN4 meeting, we have reached the consensus on some simulation assumption in high level, however there are still lots of clarifications or further discussions needed in order to conduct the coexistence evaluations.

Regarding the BS antenna geometry assumptions are agreed as following, however there are no clear description of rows and columns e.g. M and N;

- 2048 (Sub Array size 8)

- 1536 (Sub Array size 4)

Regarding the BS antenna geometry assumptions are agreed as following, however there are no clear description of rows and columns e.g. M and N. In this contribution, we would like to further clarify its detailed antenna configurations in the following table given the assumption of maintaining the similar antenna array size as low frequency ranges. e.g. 4.9GHz or U6GHz.

Table 2.1-1: Antenna array parameters for Macro urban

| Parameter | Macro urban |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | Option 1: 8 × 32 elements Option 2: 12 × 32 elements |
| Horizontal/Vertical radiating sub-array spacing | Option 1: 0.5 of wavelength for H, 5.6 of wavelength for V Option 1: 0.5 of wavelength for H, 2.8 of wavelength for V |
| Number of element rows in sub-array | Option 1: 8 elements Option 2: 4 elements |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 or TBD |
| Array Ohmic loss (dB) (Note 2) | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 |
| Base station horizontal coverage range (degrees) | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 |
| Mechanical down-tilt (degrees) | 6 |

Table 2.1-2: Antenna array parameters for small cell indoor

| Parameter | Small cell indoor |
| --- | --- |
| Am (dB) | 30 |
| SLAv (dB) | 30 |
| 3dB (deg.) | 90 |
| 3dB (deg.) | 90 |
| GE,max (dBi) | 5.5 |
| LE (dB) | 2.0 |
| (M, N) | (8, 8) |
| Number of supported polarizations, P | 2 |
| dh (m) | 0.5 |
| dv (m) | 0.5 |
| Horizontal coverage range (deg.) | N/A |
| Vertical coverage range (deg.) | N/A |
| Conducted power (before ohmic loss) per antenna element, Ptx (dBm) | 9 (Note 6) |
| Mechanical downtilt (deg.) | N/A (Note 7) |

In addition, the conducted transmission power per sub-array is assumed to be same as that of low frequency range in the above table, however indeed the actual transmission power per front-end PA will decrease with the increasing operating frequency due to the breaking voltage trends.

Proposal 1: use the antenna array assumption in Table 2.1.1 and Table 2.1.2 with the transmission power per sub-array to be further discussed.

In last RAN4 meeting, there were some discussions on Rx number of FR1 like UE operating at 15GHz, e.g. 4Rx, or 6Rx or 8Rx, at least from our understanding, we didn’t consider the Rx number of FR1 UE in past in the coexistence study and we don’t see the reasons or motivations to have such kind of assumptions to be implemented in the platform. 4Rx or 8Rx number per UE should be useful information from RAN1 dynamic SLS evaluation according to the MIMO metrics, however in RAN4 coexistence study, we don’t need to consider the MIMO metrics in RAN4 static SLS evaluation. In short, we propose to follow the legacy approach and do not consider the Rx number from UE perspective in RAN4 SLS coexistence evaluation.

Proposal 2: follow the legacy approach and do not consider the Rx number from UE perspective in RAN4 SLS coexistence evaluation.

## 2.2.Evaluation results

### 2.2.1. Urban macro

In the following sections, we provide some initial coexistence simulation results based on FR1 like UE type assumption and BS with fully digital beamforming and sub-array based beamforming assumption.

Full digital beamforming:

Figure 2.2.1-1. DL SINR with different antenna parameters without considering ACI

Table 2.2.1-1.DL throughput loss of victim BS for 14800 to 15350 MHz [Relative ACIR value] [co-located]

| Antenna configuration | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -2 | -4 | -6 | -8 | -10 |
| 1536 | AAS based BS | Average throughput loss in % (15 GHz) | 0.25 | 0.32 | 0.41 | 0.53 | 0.69 | 0.90 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 1.41 | 1.73 | 2.07 | 2.60 | 3.25 | 4.22 |
| 2048 | AAS based BS | Average throughput loss in % (15 GHz) | 0.19 | 0.25 | 0.32 | 0.42 | 0.55 | 0.72 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 1.12 | 1.34 | 1.73 | 1.91 | 2.45 | 3.43 |

Table 2.2.1-2.DL throughput loss of victim BS for 14800 to 15350 MHz [Relative ACIR value] [non-colocated]

| Antenna configuration | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -2 | -4 | -6 | -8 | -10 |
| 1536 | AAS based BS | Average throughput loss in % (15 GHz) | 0.33 | 0.40 | 0.50 | 0.62 | 0.78 | 0.99 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 3.14 | 3.31 | 3.74 | 4.37 | 5.20 | 6.10 |
| 2048 | AAS based BS | Average throughput loss in % (15 GHz) | 0.29 | 0.35 | 0.43 | 0.53 | 0.67 | 0.84 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 2.59 | 2.82 | 3.41 | 3.97 | 4.75 | 5.72 |

Observation 1: based on the FR1 like UE assumption, then DL ACIR requirement could be relaxed around 8dB in Urban macro scenario.

Figure 2.2.1-2. UL SINR with different antenna parameters without considering ACI

Figure 2.2.1-3. UL transmission power with different antenna parameters without considering ACI

Table 2.2.1-3.UL throughput loss of victim BS for 14800 to 15350 MHz [Relative ACIR value] [co-located]

| Antenna configuration | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -2 | -4 | -6 | -8 | -10 |
| 1536 | AAS based BS | Average throughput loss in % (15 GHz) | 0.20 | 0.24 | 0.28 | 0.35 | 0.44 | 0.55 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 3.97 | 4.39 | 4.57 | 4.97 | 5.36 | 5.96 |
| 2048 | AAS based BS | Average throughput loss in % (15 GHz) | 0.17 | 0.20 | 0.24 | 0.30 | 0.37 | 0.46 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 4.33 | 4.47 | 4.84 | 5.39 | 5.77 | 6.62 |

Table 2.2.1-3.UL throughput loss of victim BS for 14800 to 15350 MHz [Relative ACIR value] [non-co-located]

| Antenna configuration | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -2 | -4 | -6 | -8 | -10 |
| 1536 | AAS based BS | Average throughput loss in % (15 GHz) | 0.34 | 0.39 | 0.45 | 0.53 | 0.63 | 0.76 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 8.68 | 9.33 | 9.96 | 10.66 | 11.87 | 13.18 |
| 2048 | AAS based BS | Average throughput loss in % (15 GHz) | 0.30 | 0.35 | 0.40 | 0.47 | 0.55 | 0.66 |
|  |  | 5%-tile throughput loss in % (15 GHz) | 7.64 | 8.08 | 8.95 | 9.80 | 10.71 | 12.02 |

Observation 2: based on the FR1 like UE assumption, then UL ACIR requirement for co-located deployment could be relaxed around 6dB, however for non-co-located deployment, even with the existing FR1 ACIR requirements, it is still challenging to meet the 5% throughput loss criteria in Urban macro scenario.

### 2.2.2. Indoor scenario

This is depends on the simulation assumptions for indoor hotpot scenarios, more simulation results would be provided once the simulation assumptions are stable.

Conclusions

In this contribution, we want to share some further views on coexistence evaluations for 14800-15350MHz and proposals/observations are made as following:

Proposal 1: for coexistence study for 14.8-15.35GHz, consider the coexistence cases as shown in Table 2.1-1 for further evaluation.

Proposal 2: follow the legacy approach and do not consider the Rx number from UE perspective in RAN4 SLS coexistence evaluation.

Observation 1: based on the FR1 like UE assumption, then DL ACIR requirement could be relaxed around 8dB in Urban macro scenario.

Observation 2: based on the FR1 like UE assumption, then UL ACIR requirement for co-located deployment could be relaxed around 6dB, however for non-co-located deployment, even with the existing FR1 ACIR requirements, it is still challenging to meet the 5% throughput loss criteria in Urban macro scenario.

References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz.

- RP-210037, Reply LS to WP5D, Approved.

- RP-240787,New SI proposal: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz, Approved.

- TR 38.921

- https://www.itu.int/en/ITU-D/Regional-Presence/AsiaPacific/Documents/Events/2017/Aug-ISS2017/PAPER_Workshop_S3_Timur.pdf

- R4-2211224, Introduction of NR licensed band 6425 -7125 MHz, Approved.