3GPP TSG-RAN WG4 Meeting#112                              R4-2412717

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.25.3

Source: 	ZTE Corporation

Title: 	Discussion on RF requirements for NTN SAN in Rel-19

Document for:	Approval

Introduction

In the RAN#102 meeting, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3 has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would like to share some initial views on RF requirements for regenerative NTN SAN.

Issue 1-3-1: Cell DTX Issue 1-3-2: RF requirements enhancements for spatial domain techniques - Agreement: companies encourage to further investigate SAN beam hopping in DL coverage enhancements for both FR1 and FR2 - clarify “transient time” meaning, if for beam switching delay or PA switch ON/OFF or frequency carrier. - Synchronisation aspects: - between ground gNB and satellite payload for transparent architecture; - between two or more transparent satellites serving the same Earth-fixed cell. FFS for Earth-moving cell. - NOTE: FFS if above issue could be treated as requirement or implementation specific. - Working hypothesis: PA always on, phase shift (pre-)reconfiguration to perform beam-hopping. - NOTE1: [RAN1 agreement] For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to zero at least if SAN phase array antenna is assumed. Values different from zero beam steering latency can be optionally reported by companies if any potential issue identified. - NOTE2: Other implementation with analog beam steering technologies are not precluded. Companies to further discuss if analog/digital antenna assumptions and for which FR1/FR2 implementations.

Discussion

## 2.2.DL coverage enhancements for FR1-NTN or FR2-NTN

During the last RAN4 meeting, there were some further discussions on beam hopping, at least the following two understandings were confirmed.

- Beam steering, SAN could adjust the analog phase shifter or digital beamforming weighting factors which is quite similar as legacy FR1/FR2 NR BS beam sweeping;

- Beam reshaping, this is somehow new compared with beam steering concept. Beam reshaping could be either done by reconfiguring the weighting factors for each antenna elements or sub-array or reconfiguring different antenna array size to change the beam pattern.

For beam switching delay for FR1-NTN or FR2-NTN for beam hopping, from our understanding, beam switching delay for FR2-NTN could be around 100ns and FR1-NTN and FR2-NTN.

Figure 1. the illustration of beam hopping

Proposal 1: regarding beam switching delay for beam hopping for beam sweeping, the beam switching delay would be around 100ns.

In addition, regarding the cell DTX for SAN, from our understanding, cell DTX could be conducted by beam hopping. In other words, we only need to focus on the transition period for beam hopping behavior regardless from beam sweeping or beam reshaping.

Conclusions

In this contribution, we shared some initial views on the RF requirements for regenerative NTN SAN in Rel-19 and proposals are made as following:

Proposal 1: regarding beam switching delay for beam hopping for beam sweeping, the beam switching delay would be around 100ns.

References

- RP-234078,New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales,CATT, Approved.

- R4-2406109, Way Forward for [110bis][315] NR_NTN_Ph3, Thales. Approved.

- R4-2410009, Way Forward for [111][315] NR_NTN_Ph3, Approved.