3GPP TSG-RAN WG4 Meeting#112                              R4-2412720

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.8.2.3

Source: 	ZTE Corporation

Title: 	Discussion on Rx requirements for NTN UE

Document for:	Discussion

Introduction

In RAN#103 meeting, the work item [RP-240857] on Enhanced requirements and test methodology for NR and IoT NTN was approved as one of Rel-19 RAN4 package. The NTN HPUE is one of objective for Rel-19 NTN evolution. In this contribution, we want to share some initial views on Rx RF requirements for Rel-19 NTN HPUE.

High power UE (HPUE) for NTN

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

Discussion on NTN HPUE Rx requirements

Table 2.1 UE RF requirements for Rel-19 NTN HPUE requirements

| NR-NTN UE Rx requirement | Proposals for Rel-19 NTN HPUE requirements |
| --- | --- |
| 7.3.2 Reference sensitivity | Regarding the RSD requirement for NTN HPUE, propose to postpone the discussions until the agreement reached for ACLR requirement for NTN HPUE. |
| 7.4 Max input level | no specification impact. |
| 7.5 Adjacent channel selectivity | no specification impact. |
| 7.6.2 In-band blocking | no specification impact. |
| 7.6.3 Out of band blocking | no specification impact. |
| 7.6.4 Narrow band blocking | no specification impact. |
| 7.7 Spurious response | no specification impact. |
| 7.8 Intermodulation characteristics | no specification impact. |
| 7.9 Spurious emissions | no specification impact. |

Proposal 1: To use the proposals in Table 2.1 for Rx requirements of Rel-19 NTN HPUE

Conclusions

In this contribution, we want to share some initial understandings on Rel-19 NTN HPUE Tx requirements and proposals are made in Table 2.1

References

- R4-2217473 WF on co-existence assumption for IoT over NTN, MTK, approved.

- RP-240857,New WID: Enhanced requirements and test methodology for NR and IoT NTN

- TR 38.863