3GPP TSG-RAN WG4 Meeting#112                              R4-2412721

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.19.1

Source: 	ZTE Corporation

Title: 	Discussion on system parameters for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. During the last RAN4 meeting, we have some initial discussions on work plan and general aspects for SBFD BS, however there are still lots of open issues not addressed yet. In this contribution, we would like to share further views on these remaining issues.

Discussion

- Issue 1-1-1: SBFD as band specific or general feature to all TDD bands

- SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations: - Option 1: Declaration based method - The band supported for SBFD shall be manufacturer declaration based - The channel bandwidth supported for SBFD shall be manufacturer declaration based - The supported channel bandwidth can be impacted by the subband/guard band size discussion - Option 2: General principle but with consideration(s) for a specific band - General principle: The channel bandwidth shall be larger than X MHz - FFS the value of X - X can be different for high and middle TDD bands - For a specific band, the following further restriction(s) can be considered: - Certain restriction can be provided by operator(s) for certain band - Other restrictions are not precluded.

ZTE: in last RAN4 meeting, we reached the consensus on SBFD as a feature which could be applicable for all TDD bands, however there are still some open issues left for further discussions, e.g. leave it to vendors’ manufacture declaration basis or to set some generic principle in the specification to guide the SBFD implementation in the specification. From our understanding, the second one is not necessary since the feasibility of SBFD operation within certain channel bandwidth is up to the vendor’s implementation or cost consideration and also rely on the operator’s commercial interest, it is very difficult to preclude any channel bandwidth to be supported for SBFD operation in the specification. For the option 1, we listed the detailed declaration as following for better understanding.

| Declaration | Characteristic |
| --- | --- |
| The supported bands for SBFD operation | Band A/B/C |
| The supported channel bandwidth for SBFD operation | Channel bandwidth 100MHz, 90MHz, 80MHz, ..50MHz etc |
| The supported DU configuration or DUD configuration | 100MHz: - The supported DU configuration or DUD configuration 90MHz: - The supported DU configuration or DUD configuration 80MHz: - The supported DU configuration or DUD configuration etc |

Proposal 1: adopt the Option 1 declaration based approach to declare the system parameters supported by the SBFD operation;

- Issue 1-2-2: How to handle guard band and subband configurations in specification

- Agreement: - Add clarification that only DUD and DU patterns are specified for the sub-band configuration. - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements - For a certain channel bandwidth which RAN4 agree to introduce RF requirements: - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements - FFS which UL/DL subbands configuration(s) will be defined in RAN4 - FFS Guard band size is declaration based and can be different for different BS classes - FFS the limitation on the maximum guard band - FFS possible range for UL/DL subband sizes - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except: - 1RB granularity (already introduced in RAN1)

ZTE: as we mentioned in the previous paragraph, all channel bandwidth should be defined for SBFD operation unless there are clear objections or low interest from the operators.

-

Regarding the DL sub-band configuration and guard band in the test configuration, this could be left up to the implementation since different BS class or different implementation might have guard band design, therefore we think that DL sub-band and the guard band is proposed up to the declaration.

Proposal 2:

at least the FRC of UL sub-band for SBFD sensitivity requirement should be defined and the following basic principle could be taken into account.

- The same DMRS pattern or configuration as legacy REFSENS requirements;

- MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity

- To define the ratio of UL transmission bandwidth configuration per BW according to the DL/UL traffic. e.g. 20% of transmission bandwidth configuration per BW for FRC design. For example, 100MHz, 30KHz, the number of PRB for FRC design is floor (273PRB*20%)=54PRB.

- RESENS =-174dBm/H+10*log(BW)+NF+IM+1dB;

Where

- NF follow the legacy FR1/FR2-1 BS assumption;

- SNR is assumed as -1dB to avoid the repeating simulation for different configuration since MCS 4 is fixed.

- IM is assumed as 2dB;

- Additional 1dB performance degradation

Proposal 3: DL sub-band and the guard band is proposed up to the declaration.

- Issue 2-1-1: BS RF Specification structure for SBFD requirements

- Options of how to introduce BS RF new requirements for SBFD-capable BS:

  - Option 1 (CATT): Creating new sub-clauses in TS 38.104 (similar as UE feature in TS38.101)

  - Option 2: Embedding the corresponding new RF requirements for SBFD in the corresponding section of TS 38.104

- Moderator Recommendation:

  - Discussion on this issue firstly by collecting pros and cons of both options.

ZTE: from our understanding,based on the legacy approach to capture the RF requirements for new features, it should be embedding the corresponding RF requirement for SBFD operation in the corresponding section of TS 38.104 instead of creating new section to avoid the duplicated context for the similar RF requirements. however lots of RF requirements for SBFD BS are still under the discussions, there are no urgency to make the decision for this issue, we are somehow fine to postpone the decision until we have full sets of RF requirement for SBFD BS.

Proposal 4: postpone the decision for Issue 2-1-1 until RAN4 reached the consensus on full sets of RF requirements of SBFD BS.

## Issue 2-3-2: Applicability of SBFD requirements to different feasible BS implementations

- The applicability of SBFD requirements to different feasible BS implementations:

  - Option 1 (Samsung): BS RF requirements for SBFD-capable BS shall be specified to enable all feasible BS implementations, including different kinds of solutions to address RX chain saturation and to mitigate interference.

- Moderator Recommendation:

  - Discussion on this option firstly in this meeting.

ZTE: in general, RAN4 specify the minimum RF requirement for certain features with reasonable coexistence performance or to ensure the reasonable performance within its wanted carriers. For the BS implementations, RAN4 specification will not put any restriction on that one. In addition, lots of new features specified in RAN4 with the corresponding RF requirement is also up to the vendor’s declaration without any mandatory support. Therefore from our understanding, it might be not necessary to mention any feasible BS implementation and RAN4 only need to ensure the reasonable performance regardless from coexistence performance or its link performance within the serving cells.

Proposal 5: RAN4 don’t need to mention any feasible BS implementation in the specification and RAN4 only need to ensure the reasonable performance regardless from coexistence performance or its link performance within the serving cells.

## Issue 2-7-1: Simulation for RX in-band blocking requirement

- There is one proposal from Ericsson to trigger the discussion on simulation for RX blocking requirement level:

  - Proposal 1 (Ericsson): Further simulation is necessary to determine the expected blocker levels due to other operators’ BSs during SBFD slots and to define the SBFD RX blocking requirement during the WI phase.

- Moderator Recommendation:

  - Discussion on this proposal in details, in the following aspects, e.g.:

    - What kinds of simulation required? System level or others.

    - Scenarios and other parameters?

    - Procedure to determine the level of in-band blocking?

    - How the co-channel CLI schemes to be considered in this evaluation?

    - Others.

ZTE: as we discussed during the SI phase, the interfering signal power level of in-band blocking requirement could be derived based on the system level simulation e.g. 99.99% CDF curve of blocking levels from aggressor system in DL direction. Since the in-band blocking signal is not relevant with co-channel CLI schemes even though co-channel CLI might be reused for adjacent channel CLI. However consider the worst assumption to define the minimum requirement for in-band blocking level for SBFD BS, then no CLI coordination between different carriers or operators should be assumed.

Proposal 6: RAN4 could derive the interference signal power level of in-band blocking requirement by system level simulation with 99.99% statistical CDF curve of blocking levels from aggressor system in DL direction and no CLI coordination among different carriers.

## Issue 2-8-1: CLI handling for co-channel/adjacent channel interference

- [Moderator] Based on the revised WID, the CLI handling schemes to be introduced are discussed in RAN1.

- Proposals:

  - Proposal 1 (Charter): We propose that companies planning to provide SBFD solutions should study this issue and offer a solution that will mitigate their degradation issue.

  - Proposal 2 (Charter): We propose that solutions identified to address adjacent channel CLI for a single operator with two adjacent carriers, if applicable to the case of adjacent channel interference between two operators, be adopted.

  - Proposal 3 (Samsung): The CLI handling schemes to be supported in RAN1 shall be considered in specifying the BS RF requirements, which are agreed to be based on RAN4 adjacent channel co-existence evaluation results.

- Moderator Recommendation:

  - Specifying CLI handling scheme(s) is clearly not within RAN4 scope:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3]: - Support gNB-to-gNB CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Support UE-to-UE CLI handling scheme(s) (the detailed schemes are to be down-selected from those in TR38.858 by RAN1#117) - Note: Without dedicated optimization for dynamic/flexible TDD.

  - If CLI handling scheme(s) is specified in other groups, RAN4 can discuss BS RF requirement by considering the newly introduced CLI handling scheme(s).

- Agreement: - The CLI handling impact on RF requirement: - Option 1: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline. - Option 2: FFS CLI handling impact on RF requirements

ZTE: considering the worst assumption to define the minimum requirement for in-band blocking level for SBFD BS to ensure the- operation robustness , then no CLI coordination between different carriers or operators should be assumed.

Proposal 7: RAN4 define the requirement for outside of wanted carrier based on no CLI coordination as baseline.

Conclusions

In this contribution, we want to share some initial views on general aspects for SBFD BS and proposals/observations are made as following:

Proposal 1: adopt the Option 1 declaration based approach to declare the system parameters supported by the SBFD operation;

Proposal 2:

at least the FRC of UL sub-band for SBFD sensitivity requirement should be defined and the following basic principle could be taken into account.

- The same DMRS pattern or configuration as legacy REFSENS requirements;

- MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity

- To define the ratio of UL transmission bandwidth configuration per BW according to the DL/UL traffic. e.g. 20% of transmission bandwidth configuration per BW for FRC design. For example, 100MHz, 30KHz, the number of PRB for FRC design is floor (273PRB*20%)=54PRB.

- RESENS =-174dBm/H+10*log(BW of above FRC)+NF+SNR+IM+[1dB];

Where

- NF follow the legacy FR1/FR2-1 BS assumption;

- SNR is assumed as -1dB to avoid the repeating simulation for different configuration since MCS 4 is fixed.

- IM is assumed as 2dB;

- Additional 1dB performance degradation

Proposal 3: DL sub-band and the guard band is proposed up to the declaration.

Proposal 4: postpone the decision for Issue 2-1-1 until RAN4 reached the consensus on full sets of RF requirements of SBFD BS.

Proposal 5: RAN4 don’t need to mention any feasible BS implementation in the specification and RAN4 only need to ensure the reasonable performance regardless from coexistence performance or its link performance within the serving cells.

Proposal 6: RAN4 could derive the interference signal power level of in-band blocking requirement by system level simulation with 99.99% statistical CDF curve of blocking levels from aggressor system in DL direction and no CLI coordination among different carriers.

Proposal 7: RAN4 define the requirement for outside of wanted carrier based on no CLI coordination as baseline.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2406107, Way Forward for [110bis][313] NR_duplex_evo

- R4-2409958, Way forward for [110bis][313] NR_duplex_evo, Approved