3GPP TSG-RAN WG4 Meeting#112                              R4-2412723

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.19.2.2

Source: 	ZTE Corporation

Title: 	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Discussion

## 2.1. Tx requirements

### 2.1.1. Co-location ACLR requirement

- Issue 2-3-1: The necessity of Co-location ACLR requirement - Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement.

From our understanding, the additional or tightening requirement when two SBFD BS operating at the adjacent frequency, then tightening ACLR requirement or OBUE requirement should be helpful to address the Bs2Bs CLI problem especially for Scenario 1, Case 3. However it should be noted that there are no much ACLR/OBUE requirement to be improved for AAS based SBFD BS due to the higher operating frequency and larger channel bandwidth and small transceiver’s PA linearity.

Proposal 1: propose to follow the legacy ACLR requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS.

### 2.1.2. OBUE requirement

- Issue 2-3-2: OBUE - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

During the last RAN4 meeting, there were some discussions on how to define OBUE requirement for SBFD capable BS in order to ensure the coexistence performance for SBFD BSs operating in adjacent channel shown as following. During the Rel-18 SBFD SI phase, the following coexistence scenario was not evaluated, however the CLI interference is quite similar as Case 3 where NR DL interfering SBFD UL. Especially for Scenario 1, Case 3, the severe performance degradation could be observed for SBFD UL due to the strong CLI from NR DL in the adjacent channel. Even with ACIR requirement tightening 8dB, the 5% throughput loss is still up to 43.5% and the mean throughput loss could be minimized down to 4.67%. However as we know that, for AAS based SBFD BS, due to the higher operating frequency and larger channel bandwidth and small transceiver’s PA linearity, there are no much ACLR/OBUE requirement to be improved. In other words, even though RAN4 is capable to agree more tightening requirement e.g. with 10dB tightening requirements than the legacy requirements, it is still not feasible to be achieved for SBFD capable BS.

Based on the above considerations, we propose to follow the legacy OBUE requirement for SBFD BS without defining any more tightening requirement for SBFD BS to address the CLI concern in the adjacent channel.

Proposal 2: propose to follow the legacy OBUE requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS to address the CLI concern in the adjacent channel.

Conclusions

In this contribution, we want to share some initial views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: propose to follow the legacy ACLR requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS.

Proposal 2: propose to follow the legacy OBUE requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS to address the CLI concern in the adjacent channel.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.