3GPP TSG-RAN WG4 Meeting #112							R4-2412727 Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.20.2.1

Source: 	OPPO

Title: 	on deployment scenarios and spectrum usage for A-IoT

Document for:	Approval

- Introduction

In RAN4#111 meeting, the co-existence scenario of ambient IoT has been fully discussed. Also some initial simulation results have been made through the calibration stage. Hence in this paper, we would like to further discuss the deployment scenarios and spectrum usage for A-IoT.

- Discussion

During the co-existence simulation calibration stage, some initial simulation of D2T2-A2 has been made. For the NR UE located outdoor only or 80% indoor, different simulation has been made.

Figure 1 D2T2-A2 co-existence scenario

Based on our simulation, the 95% CDF SINR with baseline and with inter-system interference has been proposed as below:

Table 1 SINR baseline and account for interference

|  | option 1 |  | option 2 |  |
| --- | --- | --- | --- | --- |
|  | baseline | Interference | baseline | interference |
| case14 | 44.56 | 44.56 | 14.91 | 14.91 |
| case16 | 43.35 | 42.59 | 9.12 | 9.12 |
| case18 | 44.62 | 17.85 | 14.92 | 9.83 |
| case20 | 43.33 | 20.54 | 9.12 | 7.86 |

For case 14 and case 16, it is all outdoor UEs. Case 14 is for victim of AIOT reader and case 16 is for victim of AIOT device. It can be clearly seen that the SINR for baseline (without inter-system interference) and for after inter-system interference are almost the same. However, if we look into case 18 and case 20 which case has 80% indoor UEs, the SINR degradation is quite large.

Observation 1: For outdoor UEs, the interference for SINR of AIOT reader and AIOT device are almost 0.

Observation 2: For 80% indoor UEs, the interference for SINR of AIOT reader and AIOT device are quite large.

In such case, we see different steps to move forward. Firstly, the indoor UE percentage can be further discussed. Secondly, there can be some mechanism between indoor AIOT BS and outdoor NR BS that can be FDMed with larger frequency separation or TDMed method so that the interference can be avoided. However, this might need other WGs progress and hence we can further allocate the question and send LS out. The last is try to use different spectrum usage. Currently, the NR UL interfere all the R2D and D2R link. Further consider a CW node who uses DL spectrum for CW as well as R2D and D2R, the interference can be avoided. However, this will also need further discussion on the out-of-topology deployment.

Proposal 1: To further investigate the indoor legacy UE deployment scenario.

3	Conclusions

In this contribution, we give further discussion on deployment scenarios and spectrum usage for A-IoT. The Observations and proposals are listed.

Observation 1: For outdoor UEs, the interference for SINR of AIOT reader and AIOT device are almost 0.

Observation 2: For 80% indoor UEs, the interference for SINR of AIOT reader and AIOT device are quite large.

Proposal 1: To further investigate the indoor legacy UE deployment scenario.

4 References