3GPP TSG-RAN WG4 Meeting #112							R4-2412728

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.5.2.3

Source: 	OPPO

Title: 	on intra-band non-collocated other aspects

Document for:	Approval

- Introduction

In RAN4#111 meeting, the number of CCs for EN-DC and NR CA of intra-band non-collocated UE has been discussed and the WF [1] has been agreed. In this paper, we give further analysis on the number of CCs for EN-DC and NR CA.

- Discussion

During the last RAN4#111 meeting, the agreement besides requirement and capabilities have been captured as below from the WF[1].

< Issue 2-3-1:  The number of B42 CCs for EN-DC >

Way Forward:

- Check the number with operators until next meeting again.

< Issue 2-3-2:  The number of NR CCs for EN-DC and NR-CA >

Way Forward:

- Check the number with operators until next meeting again.

< Issue 2-3-3:  Terminology for power imbalance requirement to take into account multiple B42 CCs >

Way Forward:

- Align to the agreed WF R4-2406628(Huawei) in the last meeting.

For EN-DC of type 4 UE, we believe similar agreement of number of LTE CCs can also apply.

Proposal 1: For EN-DC of type 4 UE, number of LTE CCs can be up to 4 and only co-located contiguous LTE CCs is considered.

For NR CA, this might need more operator’s input. From channel bandwidth perspective, current intra-band CA of band n77 and n78 has already supported up to 3 CCs as captured below:

| CA_n77D | n773 | 100 | 100 | 100 |  |  | 300 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | See n77 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 300 | 4 and 5 |
| CA_n78D | n783 | 100 | 100 | 100 |  |  | 300 | 0 |
|  |  | See n78 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 300 | 4 and 5 |

In such case that up to 3 maximum CCs of NR CA for one band is supported. However, this will lead to 300MHz max aggregated channel bandwidth which seems not supported by one operator in current status. Still up to 3 CCs can be supported and further determined by operator input.

Proposal 2: For NR CA of type 4 UE, number of NR CCs can be up to 3 and only co-located contiguous NR CCs is considered.

3	Conclusions

In this paper, we give further discussion on Type 4 UE other aspects.

Proposal 1: For EN-DC of type 4 UE, number of LTE CCs can be up to 4 and only co-located contiguous LTE CCs is considered.

Proposal 2: For NR CA of type 4 UE, number of NR CCs can be up to 3 and only co-located contiguous NR CCs is considered.

4 References

[1] R4-2410686 	WF on Rel-19 non-collocated UE RF requirements, KDDI