3GPP TSG-RAN WG4 Meeting #112							R4-2412729

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.5.2.2

Source: 	OPPO

Title: 	On the non-collocated UE behavior and capability

Document for:	Approval

- Introduction

In RAN4#111 meeting, the intra-band non-collocated UE behaviour and capability has been discussed and the WF [1] has been agreed. In this paper, we give further analysis on the UE capability

- Discussion

During the last RAN4#111 meeting, the agreement for UE capabilities has been captured as below from the WF[1].

< Issue 2-2-1:  New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Add new UE capabilities for Type 4a and Type 4b support indication.

  - Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

< Issue 2-2-2:  Whether to support Type 2 capability by UE having Type 4 capabilities >

Agreement:

- If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not.

  - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities.

  - If the issue is identified for these bullets, RAN4 will revisit them.

< Issue 2-2-3:  New BS Signalling to switch between Type 4a/4b and Type 1(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

< Issue 2-2-4:  New BS Signalling to switch between Type 4a/4b and Type 2 >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

< Issue 2-2-5: When to inform RAN2 the demand on new UE capability(s) and new BS signalling(s) >

Way Forward:

- Given the discussions on UE capabilities and network signalling, it is not hurry to send this.

< Issue 2-2-6:  UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signalling >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) and BS signalling for Type 4.

- And then, continue further discussion on BS signalling/UE behavior in the next meeting.

Firstly, we would like to discuss the type 4 UE capability. As listed in the WF, there are two options as one is to differentiate the type 4a and type 4b capability and the other one is to only define the type 4 capability. Going back to the type 2 capability, in TS 38.331, it is reported in the CA-ParametersNR.

– CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

CA-ParametersNR information element

CA-ParametersNR-v1800 ::= SEQUENCE {

intraBandNR-CA-non-collocated-r18 ENUMERATED {supported} OPTIONAL

}

Also captured in TS 38.306 the signalling is defined as a per BC reported capability.

Observation 1: The type 2 capability intraBandNR-CA-non-collocated-r18 is defined as a per BC reported signalling and it is reported in CA-ParametersNR.

In such case, if we want to define the type 4 capability, it is quite nature to reuse similar definition and reporting of type 2 capability to the type 4 capability. However, currently the Rel-18 capability and reporting mechanism works well has a very important baseline is that the TS 38.101-1 only define the UE behaviour with max-MIMO-layer equals to 2 and assuming the largest MIMO-layer per Band is 4.

Observation 2: TS 38.101-1 only define the UE behaviour with max-MIMO-layer equals to 2 and assuming the largest MIMO-layer per Band is 4.

This “hidden condition” does not apply to type 4 since the max-MIMO-layer for one of the CC should be 2 if it is a type 4a UE. And at the same time the largest MIMO-layer can be up to 6 or 8 for type 4a and type 4b respectively. This makes it quite important for NW to know if the UE supports 4+2 or 4+4 otherwise when it configure the maxNumberMIMO-LayersPDSCH based on the UE reported capability, there is for sure limitation for the configuration. Especially if a UE only support type 4a capability which means in one CC it can only support up to 2 layer MIMO while if the specific capability is not reported and then the NW configure the UE with 4 layer MIMO, this would make the UE has no idea how to react in the network.

In such case, it is quite important to the UE to clearly report its capability as type 4a or type 4b to the NW so that the NW can configure corresponding MIMO layers for the UE. Some company has worried about the capability reporting is currently too complicated, however, still only one bit can represent such capability. If the capability is not reported, then it means the UE cannot support non-collocated deployment. If its value is 0, then it means type 4a capability and if its value is 1 it means type 4b capability.

Observation 3: To report the type 4a/4b capability separately does not  increase the signalling bits.

With that, it is proposed to add one new UE capability signalling to report the type 4a/4b capability.

Proposal 1: To add one new UE capability signalling with one bit to report the type 4a/4b capability.

Secondly, compared to type 2 capability, the NW signalling is used to configure the UE working at type 2 non-collocated and it is configured in the CellGroupConfig IE. The detail IE description is captured as below:

– CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels

with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the

configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::= SEQUENCE {

nonCollocatedTypeNR-CA-r18 ENUMERATED { true } OPTIONAL, -- Need R

uplinkTxSwitchingMoreBands-r18 SetupRelease { UplinkTxSwitchingMoreBands-r18 } OPTIONAL -- Need M

]]

The default UE behaviour is type 2 so if the NW does not configure such nonCollocatedTypeNR-CA-r18 IE, then the UE will work in type 2 non-collocated while if the NW configures such IE, then the UE will work in type 1.

Similarly, a nonCollocatedTypeNR-CA-r19 IE can be introduced however a slight difference is the NW might need to clearly tells the UE its behaviour as type 4a or type 4b. The reason behind it is that a type 4b UE is natural to support type 4a UE. If a UE is type 4b capable, however, due to any other reason such as the channel estimation the NW might configure UE to work in lower number of MIMO-layers, then this NW signalling can help to configure UE correct behaviour.

Proposal 2: To define the NW signalling to differentiate type 4a/4b to configure UE working in proper status.

Regarding the UE behaviour of switching from type 1, type 2 and type 4a/4b, we believe with current UE capability report and NW signalling together with the newly designed type 4 capability signalling and NW signalling can clearly indicate the exact UE type and corresponding requirement set to apply. Below table 1 has shown the relationship. The first column “intraBandNR-CA-non-collocated-type4” indicates the newly designed UE capability. The 4th column “nonCollocatedTypeNR-CA-type4” indicates the newly designed NW signalling.

Table 1

| intraBandNR-CA-non-collocated-type4 | intraBandNR-CA-non-collocated-r18 | UE type can be configured | nonCollocatedTypeNR-CA-type4 | nonCollocatedTypeNR-CA-r18 | Requirement apply |
| --- | --- | --- | --- | --- | --- |
| a | Supported | 1，2，4 | a | Not provided | Type 4a |
|  |  |  | a | true | Type 4a |
|  |  |  | Not provided | Not provided | Type 2 |
|  |  |  | Not provided | true | Type 1 |
| a | Not reported | 1，4 | a | Network should not use this filed | Type 4a |
|  |  |  | Not provided |  | Type 1 |
| b | Supported | 1，2，4 | b | Not provided | Type 4b |
|  |  |  | b | true | Type 4b |
|  |  |  | Not provided | Not provided | Type 2 |
|  |  |  | Not provided | true | Type 1 |
| b | Not reported | 1，4 | b | Network should not use this filed | Type 4b |
|  |  |  | Not provided |  | Type 1 |
| Not reported | Supported | 1,2 | Network should not use this filed | true | Type 2 |
|  |  |  |  | Not provided | Type 1 |
| Not reported | Not reported | 1 | Network should not use this filed |  | Type 1 |

Proposal 3: To agree on table 1 of the relationship between type 2 and type 4a/4b UE capability and NW signalling with corresponding applied requirement set.

3	Conclusions

In this paper, we give further discussion on Type 4 UE behaviour and capability

Observation 1: The type 2 capability intraBandNR-CA-non-collocated-r18 is defined as a per BC reported signalling and it is reported in CA-ParametersNR.

Observation 2: TS 38.101-1 only define the UE behaviour with max-MIMO-layer equals to 2 and assuming the largest MIMO-layer per Band is 4.

Observation 3: To report the type 4a/4b capability separately does not  increase the signalling bits.

Proposal 1: To add one new UE capability signalling with one bit to report the type 4a/4b capability.

Proposal 2: To define the NW signalling to differentiate type 4a/4b to configure UE working in proper status.

Proposal 3: To agree on table 1 of the relationship between type 2 and type 4a/4b UE capability and NW signalling with corresponding applied requirement set.

4 References

[1] R4-2410686 	WF on Rel-19 non-collocated UE RF requirements, KDDI