3GPP TSG-RAN WG4 Meeting #112							R4-2412732

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.3.2.3

Source: 	OPPO

Title: 	on SL Intra-band non-contiguous CA RX requirements

Document for:	Approval

- Introduction

In RAN4#111 meeting, the RX requirements for intra-band non-contiguous SL CA has been discussed and a WF[1] has been agreed. Still there are some difference between the intra-band contiguous and non-contiguous SL CA, hence we would like to further clarify the RX requirements for intra-band non-contiguous SL CA.

- Discussion

During the last RAN4 meeting, the RX requirements for intra-band non-contiguous CA have been agreed to be updated correspondingly from contiguous SL CA. Below agreement is captured from WF[1].

Topic #3: UE RX RF requirement

Issue 3-1-1: RX requriements

- Proposals

  - Proposal 1 (Huawei):

    - The description of the following Rx requirements in the specification should be updated from that only for contiguous intra-band SL CA to also accommodate the requirements for non-contiguous intra-band SL CA

      - REFSENS

      - Maximum input level,

      - ACS locking

      - In-band/ out-of-band blocking

      - Spurious response

      - Wide band Intermodulation

Agreement:

- Use proposal 1 as the starting point.

For REFSENS requirement, currently the intra-band contiguous NR SL CA requirement in TS 38.101-1 is captured as below:

For intra-band contiguous NR SL CA operation, the reference sensitivity requirement specified in Table 7.3E.2-1 shall apply for each component carrier with all carriers active. The requirement is applied for each carrier reception when 2 carrier transmissions are activated at the same time.

Similar requirement should apply for intra-band non-contiguous CA.

Proposal 1: For intra-band non-contiguous CA REFSENS requirement, current requirement in TS 38.101-1 subclause 7.3E.2A.1 apply.

For maximum input level requirement, current requirement for intra-band non-contiguous CA in clause 7.4E.1A apply to bandwidth class B. Also the NR intra-band non-contiguous CA requirement are defined “with the uplink configuration in accordance with 7.3A.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in Table 7.4-1 and Table 7.4A.1-1 for one component carrier and two component carriers per sub-block, respectively.” , which is captured in TS 38.101-1 clause 7.4A.2. So the non-contiguous CA requirement apply for each sub-block. Similar approach shall be reused for intra-band non-contiguous CA for sidelink.

Proposal 2: For intra-band non-contiguous Sidelink CA maximum input level requirement, the the UE shall meet the requirements for each sub-block as specified in Table 7.4-1 in TS 38.101-1.

For ACS requirement, similarly, for intra-band non-contiguous CA, the requirement currently captured in clause 7.5A.2 apply for 2 or more Downlink sub-blocks with one or two carriers in one sub-block. For intra-band non-contiguous SL CA, there is only one carrier per each sub-block hence the simplified requirement can apply. It is proposed following requirement for ACS for intra-band non-contiguous SL CA:

Proposal 3: For ACS requirement, below text is proposed:

“the UE shall meet the requirements for each sub-block as specified in clauses 7.5E for one component carrier per sub-block. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all sidelink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5E-2 for one component carrier per sub-block. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer in accordance with the ACS requirement for each sub-block (Table 7.5E-1). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer is -25 dBm (Table 7.5E-3) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer like for Case 1.”

For the in-band blocking requirement, it is proposed below text as:

Proposal 4, for in-band blocking requirement,

the UE shall meet the requirements for each sub -block as specified in clause 7.6E.2 for one component carrier per sub-block. The requirements apply for in-gap and out-of-gap interferers while all sidelink carriers are active.

For the out-of-band blocking requirement, it is proposed below text as:

Proposal 5, for out-of-band blocking requirement,

the UE shall meet the requirements for each sub-block as specified in clauses 7.6E.3 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

For spurious response, it is proposed below text as:

Proposal 6, for spurious response requirement,

the UE shall meet the requirements for each sub-block as specified in clauses 7.7E for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

For wide band intermodulation, it is proposed below text as:

Proposal 7, for spurious response requirement,

the UE shall meet the requirements for each sub-block as specified in clauses 7.8E.2 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

3	Conclusions

In this contribution, we give further analysis on the intra-band NC CA RX requirements and the proposals are listed as below.

Proposal 1: For intra-band non-contiguous CA REFSENS requirement, current requirement in TS 38.101-1 subclause 7.3E.2A.1 apply.

Proposal 2: For intra-band non-contiguous Sidelink CA maximum input level requirement, the the UE shall meet the requirements for each sub-block as specified in Table 7.4-1 in TS 38.101-1.

Proposal 3: For ACS requirement, below text is proposed:

“the UE shall meet the requirements for each sub-block as specified in clauses 7.5E for one component carrier per sub-block. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all sidelink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5E-2 for one component carrier per sub-block. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer in accordance with the ACS requirement for each sub-block (Table 7.5E-1). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer is -25 dBm (Table 7.5E-3) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer like for Case 1.”

Proposal 4, for in-band blocking requirement,

the UE shall meet the requirements for each sub -block as specified in clause 7.6E.2 for one component carrier per sub-block. The requirements apply for in-gap and out-of-gap interferers while all sidelink carriers are active.

Proposal 5, for out-of-band blocking requirement,

the UE shall meet the requirements for each sub-block as specified in clauses 7.6E.3 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

Proposal 6, for spurious response requirement,

the UE shall meet the requirements for each sub-block as specified in clauses 7.7E for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

Proposal 7, for spurious response requirement,

the UE shall meet the requirements for each sub-block as specified in clauses 7.8E.2 for one component carrier per sub-block, respectively. The requirements apply with all sidelink carriers active.

4 References

[1] R4-2410586 	WF on NR_SL_ intraB_CA_ITS_part1, OPPO, Meta