3GPP TSG-RAN WG4 Meeting #112							R4-2412733

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.3.2.2

Source: 	OPPO

Title: 	Intra-band contiguous CA simulation results

Document for:	Approval

- Introduction

In RAN4#111 meeting, the intra-band contiguous CA MPR simulation assumption has been fully discussed. Similar to NR UL CA, different architecture with different number of LOs and PAs are considered. In this paper, we provide initial simulation results for intra-band contiguous CA.

- Discussion

2.1 MPR simulation configuration

In the WF [1] agreed in the last RAN4#111 meeting, the MPR corresponding agreements are captured as below:

Agreement

- Do evaluations based on assumptions of 1LO and 2LO

- Consider the following architecture for evaluation

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x26dBm PA + 1LO | Single–Tx |
| #1-2 | 2x23dBm PA + 1LO | dual-Tx or txDiversity |
| #2-2 | 2x23dBm PA + 2LO | dualPA-Architecture |

Issue 2-1-2: Frequency gap

Agreement:

- Consider the worst case for BW gap size between sub-block #1 and #2 for the MPR requirement.

- For evaluations, consider all the gap size.

Issue 2-1-4: Requirement for simulation

Agreement:

- For FCC regulation, work is based on current NS_52 first and wait for FCC update.

In our simulation, we have simulated the all the PA and LO architectures. For the intra-band non-contiguous, only 10+10MHz and 10+20MHz is applied for channel bandwidth. Also it has been agreed that same SCS for different carriers will be used to derive simulation results. Considering the GAP size, it has been agreed that worst case of GAP will be considered for simulation. Furthermore, for 1 LO architecture, it has been agreed that the max(CBW1, CBW2)≤GAP size ≤ CBW of CC1+CC2, so the GAP size used for evaluation is 10 and 20MHz for 10+10MHz CBW combination and 20 and 30MHz for 10+20MHz CBW combination respectively. For 2 LO architecture, there is no such limitation. Consider all of the above, below the BW configuration for simulation has been provided.

Below channel bandwidth and RB configuration are used as table 1, case 1 to 48 are for 1 LO architecture and case 49 to 96 is for 2 LO architecture. For each row, the difference is only for the gap size.

Table 1, channel bandwidth and RB configuration

| Ca se | LC RB1 | RBST ART1 | LC RB2 | RBST ART2 | SC S | CB W1 | CB W2 | GA P | Ca se | LC RB1 | RBST ART1 | LC RB2 | RBST ART2 | SC S | CB W1 | CB W2 | GA P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 0 | 10 | 42 | 15 | 10 | 10 | 20 | 49 | 10 | 0 | 10 | 42 | 15 | 10 | 10 | 50 |
| 2 | 10 | 18 | 10 | 22 | 15 | 10 | 10 | 20 | 50 | 10 | 18 | 10 | 22 | 15 | 10 | 10 | 50 |
| 3 | 10 | 19 | 10 | 22 | 15 | 10 | 10 | 20 | 51 | 10 | 19 | 10 | 22 | 15 | 10 | 10 | 50 |
| 4 | 10 | 27 | 10 | 14 | 15 | 10 | 10 | 20 | 52 | 10 | 27 | 10 | 14 | 15 | 10 | 10 | 50 |
| 5 | 10 | 33 | 10 | 7 | 15 | 10 | 10 | 20 | 53 | 10 | 33 | 10 | 7 | 15 | 10 | 10 | 50 |
| 6 | 10 | 34 | 10 | 7 | 15 | 10 | 10 | 20 | 54 | 10 | 34 | 10 | 7 | 15 | 10 | 10 | 50 |
| 7 | 10 | 42 | 10 | 1 | 15 | 10 | 10 | 20 | 55 | 10 | 42 | 10 | 1 | 15 | 10 | 10 | 50 |
| 8 | 25 | 0 | 25 | 27 | 15 | 10 | 10 | 20 | 56 | 25 | 0 | 25 | 27 | 15 | 10 | 10 | 50 |
| 9 | 25 | 19 | 25 | 8 | 15 | 10 | 10 | 20 | 57 | 25 | 19 | 25 | 8 | 15 | 10 | 10 | 50 |
| 10 | 25 | 20 | 25 | 8 | 15 | 10 | 10 | 20 | 58 | 25 | 20 | 25 | 8 | 15 | 10 | 10 | 50 |
| 11 | 25 | 27 | 25 | 1 | 15 | 10 | 10 | 20 | 59 | 25 | 27 | 25 | 1 | 15 | 10 | 10 | 50 |
| 12 | 30 | 0 | 30 | 22 | 15 | 10 | 10 | 20 | 60 | 30 | 0 | 30 | 22 | 15 | 10 | 10 | 50 |
| 13 | 30 | 19 | 30 | 3 | 15 | 10 | 10 | 20 | 61 | 30 | 19 | 30 | 3 | 15 | 10 | 10 | 50 |
| 14 | 30 | 19 | 30 | 2 | 15 | 10 | 10 | 20 | 62 | 30 | 19 | 30 | 2 | 15 | 10 | 10 | 50 |
| 15 | 30 | 22 | 30 | 1 | 15 | 10 | 10 | 20 | 63 | 30 | 22 | 30 | 1 | 15 | 10 | 10 | 50 |
| 16 | 50 | 1 | 50 | 0 | 15 | 10 | 10 | 20 | 64 | 50 | 1 | 50 | 0 | 15 | 10 | 10 | 50 |
| 17 | 10 | 0 | 20 | 86 | 15 | 10 | 20 | 30 | 65 | 10 | 0 | 20 | 86 | 15 | 10 | 20 | 40 |
| 18 | 10 | 18 | 20 | 44 | 15 | 10 | 20 | 30 | 66 | 10 | 18 | 20 | 44 | 15 | 10 | 20 | 40 |
| 19 | 10 | 19 | 20 | 44 | 15 | 10 | 20 | 30 | 67 | 10 | 19 | 20 | 44 | 15 | 10 | 20 | 40 |
| 20 | 10 | 27 | 20 | 28 | 15 | 10 | 20 | 30 | 68 | 10 | 27 | 20 | 28 | 15 | 10 | 20 | 40 |
| 21 | 10 | 33 | 20 | 14 | 15 | 10 | 20 | 30 | 69 | 10 | 33 | 20 | 14 | 15 | 10 | 20 | 40 |
| 22 | 10 | 34 | 20 | 14 | 15 | 10 | 20 | 30 | 70 | 10 | 34 | 20 | 14 | 15 | 10 | 20 | 40 |
| 23 | 10 | 42 | 20 | 2 | 15 | 10 | 20 | 30 | 71 | 10 | 42 | 20 | 2 | 15 | 10 | 20 | 40 |
| 24 | 25 | 0 | 50 | 56 | 15 | 10 | 20 | 30 | 72 | 25 | 0 | 50 | 56 | 15 | 10 | 20 | 40 |
| 25 | 25 | 19 | 50 | 16 | 15 | 10 | 20 | 30 | 73 | 25 | 19 | 50 | 16 | 15 | 10 | 20 | 40 |
| 26 | 25 | 20 | 50 | 16 | 15 | 10 | 20 | 30 | 74 | 25 | 20 | 50 | 16 | 15 | 10 | 20 | 40 |
| 27 | 25 | 27 | 50 | 2 | 15 | 10 | 20 | 30 | 75 | 25 | 27 | 50 | 2 | 15 | 10 | 20 | 40 |
| 28 | 30 | 0 | 60 | 44 | 15 | 10 | 20 | 30 | 76 | 30 | 0 | 60 | 44 | 15 | 10 | 20 | 40 |
| 29 | 30 | 19 | 60 | 6 | 15 | 10 | 20 | 30 | 77 | 30 | 19 | 60 | 6 | 15 | 10 | 20 | 40 |
| 30 | 30 | 19 | 60 | 4 | 15 | 10 | 20 | 30 | 78 | 30 | 19 | 60 | 4 | 15 | 10 | 20 | 40 |
| 31 | 30 | 22 | 60 | 2 | 15 | 10 | 20 | 30 | 79 | 30 | 22 | 60 | 2 | 15 | 10 | 20 | 40 |
| 32 | 50 | 1 | 100 | 0 | 15 | 10 | 20 | 30 | 80 | 50 | 1 | 100 | 0 | 15 | 10 | 20 | 40 |
| 33 | 5 | 0 | 10 | 41 | 30 | 10 | 20 | 30 | 81 | 5 | 0 | 10 | 41 | 30 | 10 | 20 | 40 |
| 34 | 5 | 9 | 10 | 22 | 30 | 10 | 20 | 30 | 82 | 5 | 9 | 10 | 22 | 30 | 10 | 20 | 40 |
| 35 | 5 | 10 | 10 | 22 | 30 | 10 | 20 | 30 | 83 | 5 | 10 | 10 | 22 | 30 | 10 | 20 | 40 |
| 36 | 5 | 13 | 10 | 14 | 30 | 10 | 20 | 30 | 84 | 5 | 13 | 10 | 14 | 30 | 10 | 20 | 40 |
| 37 | 5 | 16 | 10 | 7 | 30 | 10 | 20 | 30 | 85 | 5 | 16 | 10 | 7 | 30 | 10 | 20 | 40 |
| 38 | 5 | 17 | 10 | 7 | 30 | 10 | 20 | 30 | 86 | 5 | 17 | 10 | 7 | 30 | 10 | 20 | 40 |
| 39 | 5 | 19 | 10 | 1 | 30 | 10 | 20 | 30 | 87 | 5 | 19 | 10 | 1 | 30 | 10 | 20 | 40 |
| 40 | 12 | 0 | 25 | 26 | 30 | 10 | 20 | 30 | 88 | 12 | 0 | 25 | 26 | 30 | 10 | 20 | 40 |
| 41 | 12 | 10 | 25 | 8 | 30 | 10 | 20 | 30 | 89 | 12 | 10 | 25 | 8 | 30 | 10 | 20 | 40 |
| 42 | 12 | 10 | 25 | 8 | 30 | 10 | 20 | 30 | 90 | 12 | 10 | 25 | 8 | 30 | 10 | 20 | 40 |
| 43 | 12 | 12 | 25 | 1 | 30 | 10 | 20 | 30 | 91 | 12 | 12 | 25 | 1 | 30 | 10 | 20 | 40 |
| 44 | 15 | 0 | 30 | 21 | 30 | 10 | 20 | 30 | 92 | 15 | 0 | 30 | 21 | 30 | 10 | 20 | 40 |
| 45 | 15 | 8 | 30 | 3 | 30 | 10 | 20 | 30 | 93 | 15 | 8 | 30 | 3 | 30 | 10 | 20 | 40 |
| 46 | 15 | 9 | 30 | 2 | 30 | 10 | 20 | 30 | 94 | 15 | 9 | 30 | 2 | 30 | 10 | 20 | 40 |
| 47 | 15 | 9 | 30 | 1 | 30 | 10 | 20 | 30 | 95 | 15 | 9 | 30 | 1 | 30 | 10 | 20 | 40 |
| 48 | 24 | 0 | 50 | 0 | 30 | 10 | 20 | 30 | 96 | 24 | 0 | 50 | 0 | 30 | 10 | 20 | 40 |

For intra-band non-contiguous CA, both PC3 and PC2 will be defined in Rel-19 timeline. Below we would like to discuss them separately.

2.2 PC3 intra-band non-contiguous CA

For the GAP of 1 LO architecture that have been used for non-contiguous CA MPR simulation, during the simulation, we have found that also captured in the UL CA MPR simulation WF [] that as no further ACLR and spurious emission requirement exception, larger LO and image suppression should be considered in case the LO and image fail into the GAP and to be considered for ACLR and spurious emission. From our evaluation, for PC3 non-contiguous SL CA, to comply with appropriate ACLR and SEM margin, 34dBc image and 36dBc DC suppression is used for simulation.

Observation 1: To comply with appropriate ACLR and SEM margin, 34dBc image and 36dBc DC suppression is used for simulation for PC3 1LO case for intra-band non-contiguous Sidelink CA.

With that, the simulation result for 1LO 1PA PC3 with 34dBc image rejection and 36dBc LO suppression, results are shown below:

Figure 1 PC3 1PA 1LO NC CA MPR results

In such case, the proposed MPR is shown as below:

Proposal 1: For PC3 SL NC CA, for 1 PA 1LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 11; 0 ≤ B < 4.32

8; 4.32 ≤ B < 6.48

7.5; 6.48 ≤ B < 10.8

6; 10.8≤ B < 14.04

5.5; 14.04 ≤ B < 27

5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 13; 0 ≤ B < 4.32

10.5; 4.32 ≤ B < 6.48

10; 6.48 ≤ B < 10.8

9.5; 10.8≤ B < 14.04

8.5; 14.04 ≤ B < 27

8; 27 ≤ B

For the 2PA 2LO with 2PC5 PA case, the simulation result is shown as:

Proposal 2: For PC3 SL NC CA, for 2 PA 2 LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 9; 0 ≤ B < 4.32

6.5; 4.32 ≤ B < 6.48

7.5; 6.48 ≤ B < 10.8

6; 10.8≤ B < 14.04

6.5; 14.04 ≤ B < 27

5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 9; 0 ≤ B < 4.32

7; 4.32 ≤ B < 6.48

9.5; 6.48 ≤ B < 10.8

7; 10.8≤ B < 18.36

8.5; 18.36 ≤ B < 27

6.5; 27 ≤ B

2.3 PC2 intra-band non-contiguous CA

For intra-band NC sidelink CA, based on our simulation procedure, due to the more stringent ACLR requirement as well as the different PA non-linearity capability, the IQ image is set to 38dBc while the LO leakage suppression is set to 45dBc. With the above setting, further simulation is provided.

Figure 3 PC2 1PA 1LO NC CA MPR results

Proposal 3: For PC2 SL NC CA, for 1 PA 1LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 11.5; 0 ≤ B < 4.32

9.5; 4.32 ≤ B < 6.48

[14]; 6.48 ≤ B < 10.8

8; 10.8≤ B < 14.04

6; 14.04 ≤ B < 27

5.5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 15; 0 ≤ B < 4.32

11.5; 4.32 ≤ B < 6.48

12.5; 6.48 ≤ B < 10.8

12; 10.8≤ B < 14.04

10; 14.04 ≤ B < 27

8.5; 27 ≤ B

For the 2PA 2LO with 2PC3 PA case, the simulation result is shown as:

Figure 4 PC2 2PA 2LO NC CA MPR results

Proposal 4: For PC2 SL NC CA, for 2 23dBm PA 2LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 9.5; 0 ≤ B < 4.32

7; 4.32 ≤ B < 6.48

7.5; 6.48 ≤ B < 10.8

5.5; 10.8≤ B < 18.36

7; 18.36≤ B < 27

5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 12.5; 0 ≤ B < 4.32

8.5; 4.32 ≤ B < 6.48

11; 6.48 ≤ B < 10.8

7.5; 10.8≤ B < 18.36

10; 18.36 ≤ B < 27

7; 27 ≤ B

Although above simulation results have been provided, still we see much higher IQ image and LO suppression is used in the simulation due to the in-gap requirements to be fulfilled with the LO and image falling into the GAP area. The IQ image and LO suppression used is listed as below table:

Table 2 IQ image and LO suppression for SL NC CA simulation

|  | PC3 NC CA | Legacy PC3 SL (TR 38.886) | PC2 NC CA | Legacy PC2 SL(TR 38.785) |
| --- | --- | --- | --- | --- |
| IQ image | 34 | 25 | 38 | 25 |
| LO | 36 | 25 | 45 | 25 |

Although higher IQ image is used in SL-U simulation as 34dBc, the increase of LO might also need further investigate. Based on the PC2 NR UL NC CA, the WF [2] has recorded the simulation assumption of some of the companies and it is captured as below.

Required carrier and image suppression need for different SEM levels were studied in [1] and is summarized in following table

| In-gap emission limit | LO suppression requirement | IQ image suppression requirement |
| --- | --- | --- |
| SEM -13 dBm/MHz | 37 dB | 32 dB |
| SEM -25 dBm/MHz | 47 dB | 36 dB |
| Spurious emissions -30 dBm/MHz | 52 dB | 42 dB |

Compared with the table 2 numbers, the LO and IQ suppression are similar between SL NC CA and UL NC CA. In such case, it is proposed to agree on the LO and IQ image used for PC3 and PC2 Sidelink NC CA simulation.

Proposal 5: 34dBc image and 36dBc DC suppression is used for simulation for PC3 1LO case for intra-band non-contiguous Sidelink CA.

Proposal 6: 36dBc image and 45dBc DC suppression is used for simulation for PC2 1LO case for intra-band non-contiguous Sidelink CA.

3	Conclusions

In this contribution, we give further analysis on the intra-band NC CA MPR simulation and its corresponding results. The proposed MPR are listed as below.

Proposal 1: For PC3 SL NC CA, for 1 PA 1LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 11; 0 ≤ B < 4.32

8; 4.32 ≤ B < 6.48

7.5; 6.48 ≤ B < 10.8

6; 10.8≤ B < 14.04

5.5; 14.04 ≤ B < 27

5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 13; 0 ≤ B < 4.32

10.5; 4.32 ≤ B < 6.48

10; 6.48 ≤ B < 10.8

9.5; 10.8≤ B < 14.04

8.5; 14.04 ≤ B < 27

8; 27 ≤ B

Proposal 2: For PC3 SL NC CA, for 2 PA 2 LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 9; 0 ≤ B < 4.32

6.5; 4.32 ≤ B < 6.48

7.5; 6.48 ≤ B < 10.8

6; 10.8≤ B < 14.04

6.5; 14.04 ≤ B < 27

5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 9; 0 ≤ B < 4.32

7; 4.32 ≤ B < 6.48

9.5; 6.48 ≤ B < 10.8

7; 10.8≤ B < 18.36

8.5; 18.36 ≤ B < 27

6.5; 27 ≤ B

Proposal 3: For PC2 SL NC CA, for 1 PA 1LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 11.5; 0 ≤ B < 4.32

9.5; 4.32 ≤ B < 6.48

[14]; 6.48 ≤ B < 10.8

8; 10.8≤ B < 14.04

6; 14.04 ≤ B < 27

5.5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 15; 0 ≤ B < 4.32

11.5; 4.32 ≤ B < 6.48

12.5; 6.48 ≤ B < 10.8

12; 10.8≤ B < 14.04

10; 14.04 ≤ B < 27

8.5; 27 ≤ B

Proposal 4: For PC2 SL NC CA, for 2 23dBm PA 2LO architecture, the MPR to meet -13dBm/MHz and -30dBm/MHz are proposed as :

To meet the -13dBm/MHz requirement, the MPR is proposed as below:

MA = 9.5; 0 ≤ B < 4.32

7; 4.32 ≤ B < 6.48

7.5; 6.48 ≤ B < 10.8

5.5; 10.8≤ B < 18.36

7; 18.36≤ B < 27

5; 27 ≤ B

To meet the -30dBm/MHz requirement, the MPR is proposed as below:

MA = 12.5; 0 ≤ B < 4.32

8.5; 4.32 ≤ B < 6.48

11; 6.48 ≤ B < 10.8

7.5; 10.8≤ B < 18.36

10; 18.36 ≤ B < 27

7; 27 ≤ B

Proposal 5: 34dBc image and 36dBc DC suppression is used for simulation for PC3 1LO case for intra-band non-contiguous Sidelink CA.

Proposal 6: 36dBc image and 45dBc DC suppression is used for simulation for PC2 1LO case for intra-band non-contiguous Sidelink CA.

4 References

[1] R4-2410586 	WF on NR_SL_ intraB_CA_ITS_part1, OPPO, Meta

[2] R4-2119955 	WF on intra-band NC UL CA for FR1, Skyworks