3GPP TSG-RAN WG4 Meeting # 112	R4-240xxxx

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

Title: 	Views on beam steering for PMI test with ULA antenna configuration

Source: 	Huawei, HiSilicon, Ericsson

Agenda item:	8.16.2

Document for:	Discussion

Background

In last meeting, a CR for 38.101-4: Corrections on Rel-15 PMI test setup was submitted, the content of which is removing the beam steering for Rel-15 PMI test. Meanwhile, a CR for 38.101-4 was submitted, the content of which is removing the beam steering for RedCap PMI 1Rx test. However, some companies questioned about the rationality, this contribution provides our views.

- Discussions

Antenna configuration of ULA and beam steering have been configured simultaneously for Rel-17 RedCap PMI and Rel-15 FR2 PMI requirements; however, beam steering is only applicable for cross polarized antenna according to subclause B.2.3.2.3 of TS 38.101-4. There are two ways to address this issue: Option 1: Remove the configuration of beam steering for these two tests and RAN4 to make the consensus that ULA will not be used for PMI requirements in the future. Option 2: Create a new sub-clause to define beam steering approach for ULA antenna configuration.

Observation 1: TS 38.101-4 B.2.3 specifies the beam steering approach only for the cross-polarized antenna array at gNB.

Since the beam steering approach is used to make the beam direction of channel change, without this configuration, the optimal PMI will be fixed throughout the test, which can cause the problem that fixed precoder performs better than close loop MIMO (i.e., follow PMI) and UE may be easily to pass the PMI reporting test. Considering in the future it may be inevitable to introduce ULA for PMI test, hence it is necessary to introduce beam steering approach for ULA configuration.

To minimize the impact to the existing specification, the proposed steering approach for ULA configuration is shown as follows:

### B.2.3.1 MIMO Correlation Matrices using Uniform Linear Array (ULA) #### B.2.3.1.1 Definition of MIMO Correlation Matrices #### B.2.3.1.2 Correlation Matrices at High, Medium and Low Level #### B.2.3.1.3 Beam steering approach The beam steering approach defined in B.2.3.2.3 is applied, where the random channel matrix H is calculated by letting N2=1 and replacing Dθk,1,θk,2 by: Dθk,1,θk,2=Dθk,1N1

Proposal 1: Introduce the beam steering approach for ULA configuration.

For Rel-15 FR2 PMI test, considering low correlation is configured, beam steering configuration is pointless since channel is omini-directional. Therefore, beam steering configuration should be removed from test parameter table.

Proposal 2: For FR2 PMI test, remove the beam steering configuration.

For Rel-17 RedCap PMI test, since high correlation is configured, the new defined beam steering configuration for ULA should be applied to change the beam direction dynamically.

Proposal 3: For Rel-17 RedCap PMI test, apply the new defined beam steering configuration for ULA.

- Conclusion

In this contribution we provide our views on how to address the issue that whether and how to introduce beam steering approach for ULA antenna configuration. The proposals are:

Proposal 1: Introduce the beam steering approach for ULA configuration.

Proposal 2: For FR2 PMI test, remove the beam steering configuration.

Proposal 3: For Rel-17 RedCap PMI test, apply the new defined beam steering configuration for ULA.

- Reference

[1]    R4-2409022 [NR_newRAT-Perf] CR for 38.101-4 Corrections on PMI test setup. Huawei, Hisilicon, Ericsson

[2]    R4-2409026 [NR_redcap-Perf] CR for 38.101-4 Corrections on PMI test setup. Huawei, Hisilicon, Ericsson