3GPP TSG-RAN WG4 Meeting # 112	R4-2412754

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

Title: 	Discussions on advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	5.19.2

Document for:	Discussion

Background

This contribution provides our views on applicability rules implemented in the latest spec.

- Discussions

The applicability rules tests with modulation order detection in latest version of 38.101-4 are shown as follows:

| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML), when the co-scheduled UE information with DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled. | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2, Test 4-2) |  |
| --- | --- | --- | --- | --- |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2, Test 4-2) |  |

According to TS 38.306, there are two UE capabilities for modulation order detection:

MU-MIMO Interference Mitigation advanced receiver with modulation order detection R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX in FR1 when the co-scheduled UE information with DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS 38.212 [10] is signalled. UE supporting the feature is required to indicate the support of advReceiver-MU-MIMO-r18.

MU-MIMO Interference Mitigation advanced receiver with modulation order detection Enh R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX in FR1 when the co-scheduled UE information with DCI index 6 in Table 7.3.1.2.2-12 of TS 38.212 [10] is signalled. UE supporting the feature is required to indicate the support of advReceiver-MU-MIMO-r18.

It can be observed that current applicability for tests with modulation order detection rules don’t conform with UE capabilities definition in 38.306. Modulation order detection tests with 2Layers should be applicable to UE supporting first capability, i.e, Support processing up to 2layers, modulation order detection tests with 4Layers should be applicable to UE supporting second capability. I.e. Support processing up to 4layers.

Observation 1: Current applicability rules for tests with modulation order detection in 38.101-4 don’t conform with UE capabilities definition in 38.306.

Hence, we propose the following wording:

Proposal 1: Applicability rules for modulation order detection should be modified to the following:

| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML) receiver with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX when the co-scheduled UE information with DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled. | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2) |  |
| --- | --- | --- | --- | --- |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2) |  |
| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML) receiver with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX when the co-scheduled UE information with DCI index 6 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled. | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2, Test 4-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2, Test 4-2) |  |

- Conclusion

This contribution provides our views on remaining issues for advanced receiver for MU-MIMO. The proposals are:

Observation 1: Current applicability rules for tests with modulation order detection in 38.101-4 don’t conform with UE capabilities definition in 38.306.

Proposal 1: Applicability rules for modulation order detection should be modified to the following:

| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML) receiver with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX when the co-scheduled UE information with DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled. | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2) |  |
| --- | --- | --- | --- | --- |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2) |  |
| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML) receiver with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX when the co-scheduled UE information with DCI index 6 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled. | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2, Test 4-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2, Test 4-2) |  |

- Reference

[1]    TS 38.101-4. v18.6.0