3GPP TSG-RAN WG4 Meeting # 112	R4-2412755

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

Title: 	Simulation results for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	5.19.2

Document for:	Discussion

Background

This contribution provides our updated simulation results.

- Updated simulation results

The updated ideal simulation results are captured in Table 3-1:

Table 3-1: Summary of ideal simulation results for MU-MIMO

| Case Number (New) | Duplex Mode and SCS | CHBW |  | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | DCI Index | Ideal SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15KHz SCS | 10 MHz |  | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Medium | MCS 13 | QPSK | Index 1/2 | 13 |
| 2 |  |  |  |  | 2T4R |  |  |  |  |  |  | 12.1 |
| 3 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 15.1 |
| 4 | TDD 30KHz | 40 MHz |  | 1+1 | 2T2R |  | TDLC300-100 | ULA Medium | MCS 13 | QPSK |  | 13.4 |
| 5 |  |  |  |  | 2T4R |  |  |  |  |  |  | 12.5 |
| 6 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 15.4 |
|  | FDD 15KHz SCS | 10MHz |  | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 17 | 16QAM | Index 6 | 20.9 |
| 10 |  |  |  |  | 2T4R |  |  |  |  |  |  | 20.9 |
| 13 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 12.6 |
| 17 | TDD 30KHz SCS | 40 MHz |  | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 17 | 16QAM |  | 22.7 |
| 18 |  |  |  |  | 2T4R |  |  |  |  |  |  | 22.3 |
| 21 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 13 |

The updated impairment simulation results are captured in Table 3-2:

Table 3-2: Summary of impairment simulation results for MU-MIMO

| Case Number (New) | Duplex Mode and SCS | CHBW |  | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | DCI Index | Impairment SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15KHz SCS | 10 MHz |  | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Medium | MCS 13 | QPSK | Index 1/2 | 14.5 |
| 2 |  |  |  |  | 2T4R |  |  |  |  |  |  | 13.6 |
| 3 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 16.6 |
| 4 | TDD 30KHz | 40 MHz |  | 1+1 | 2T2R |  | TDLC300-100 | ULA Medium | MCS 13 | QPSK |  | 14.9 |
| 5 |  |  |  |  | 2T4R |  |  |  |  |  |  | 14 |
| 6 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM |  | 16.9 |
|  | FDD 15KHz SCS | 10MHz |  | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 17 | 16QAM | Index 6 | 23.4 |
| 10 |  |  |  |  | 2T4R |  |  |  |  |  |  | 23.4 |
| 13 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 15.6 |
| 17 | TDD 30KHz SCS | 40 MHz |  | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 17 | 16QAM |  | 25.2 |
| 18 |  |  |  |  | 2T4R |  |  |  |  |  |  | 24.8 |
| 21 |  |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 16 |

- Conclusion

This contribution provides our updated simulation results for advanced receiver for MU-MIMO.