3GPP TSG-RAN WG4 Meeting # 112	R4-2412757

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

Title: 	Discussions on eRedCap demodulation and CSI requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.18.2.1

Document for:	Discussion

Background

One remaining open issue is whether to define CQI requirements under fading channel. This contribution provides our views.

- Discussions

The updated simulation results for CQI test under fading channel are shown in Table 3-1:

Table 3-1: Simulation results for CQI under fading channel

| SNR(dB) | FDD |  |  |  |  |  |  |  | TDD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1Rx |  |  |  | 2RX |  |  |  | 1Rx |  |  |  | 2RX |  |  |  |
|  | Median CQI | BLER | Alpha | Gama | Median CQI | BLER | Alpha | Gama | Median CQI | BLER | Alpha | Gama | Median CQI | BLER | Alpha | Gama |
| 4 | 6 | 0.24 | 0.39 | 1.24 | 8 | 0.31 | 0.43 | 0.94 | 5 | 0.19 | 0.51 | 1.16 | 6 | 0.17 | 0.44 | 1.30 |
| 5 | 7 | 0.25 | 0.39 | 1.21 | 8 | 0.26 | 0.38 | 1.10 | 6 | 0.16 | 0.45 | 1.26 | 6 | 0.16 | 0.48 | 1.20 |
| 6 | 8 | 0.23 | 0.41 | 1.17 | 9 | 0.28 | 0.42 | 1 | 6 | 0.16 | 0.43 | 1.33 | 7 | 0.17 | 0.52 | 1.09 |
| 7 | 8 | 0.24 | 0.39 | 1.14 | 9 | 0.23 | 0.37 | 1.13 | 6 | 0.16 | 0.46 | 1.23 | 8 | 0.16 | 0.46 | 1.07 |
| 8 | 9 | 0.22 | 0.42 | 1.12 | 10 | 0.23 | 0.42 | 1.15 | 6 | 0.16 | 0.52 | 1.15 | 9 | 0.15 | 0.44 | 1.09 |
| 9 | 9 | 0.23 | 0.40 | 1.13 | 11 | 0.22 | 0.45 | 1.14 | 8 | 0.16 | 0.51 | 1.16 | 9 | 0.15 | 0.46 | 1.14 |
| 10 | 10 | 0.20 | 0.43 | 1.20 | 11 | 0.21 | 0.44 | 1.14 | 8 | 0.15 | 0.52 | 1.12 | 9 | 0.14 | 0.44 | 1.16 |
| 11 | 10 | 0.22 | 0.40 | 1.19 | 12 | 0.19 | 0.51 | 1.14 | 9 | 0.14 | 0.44 | 1.15 | 10 | 0.14 | 0.45 | 1.12 |
| 12 | 11 | 0.20 | 0.43 | 1.15 | 12 | 0.19 | 0.47 | 1.14 | 9 | 0.14 | 0.42 | 1.18 | 11 | 0.14 | 0.46 | 1.12 |
| 13 | 11 | 0.20 | 0.40 | 1.15 | 12 | 0.18 | 0.46 | 1.13 | 9 | 0.14 | 0.43 | 1.12 | 12 | 0.12 | 0.41 | 1.12 |
| 14 | 12 | 0.19 | 0.47 | 1.14 | 13 | 0.17 | 0.56 | 1.13 | 11 | 0.13 | 0.42 | 1.13 | 12 | 0.13 | 0.43 | 1.10 |
| 15 | 12 | 0.19 | 0.45 | 1.11 | 13 | 0.17 | 0.58 | 1.10 | 11 | 0.13 | 0.55 | 1.13 | 12 | 0.12 | 0.52 | 1.11 |
| 16 | 13 | 0.19 | 0.56 | 1.10 | 14 | 0.15 | 0.67 | 1.13 | 12 | 0.11 | 0.56 | 1.13 | 12 | 0.10 | 0.53 | 1.09 |
| 17 | 13 | 0.17 | 0.56 | 1.08 | 14 | 0.15 | 0.72 | 1.09 | 12 | 0.11 | 0.58 | 1.11 | 13 | 0.09 | 0.54 | 1.06 |

Based on our simulation results, it’s feasible to define CQI requirements under fading channel. The test SNR and test metric can be reused from Rel-18 RedCap test:

SNR=9/10dB and SNR=15/16dB for FDD/HD-FDD/TDD with 1Rx.

SNR=6/7dB for FDD/HD-FDD/TDD with 2Rx.

Requirements:

A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%.

Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05

Average BLER with follow CQI shall be greater than or equal to 0.02.

Proposal 1: RAN4 to define CQI requirements for eRedCap UE under fading channel. The test SNR and metric can be:

SNR=9/10dB and SNR=15/16dB for FDD/HD-FDD/TDD with 1Rx.

SNR=6/7dB for FDD/HD-FDD/TDD with 2Rx.

Requirements:

A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%.

Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05

Average BLER with follow CQI shall be greater than or equal to 0.02.

- Conclusion

In this contribution we provide our updated simulation results for eRedCap CQI requirements under fading channel. The proposal is:

Proposal 1: RAN4 to define CQI requirements for eRedCap UE under fading channel. The test SNR and metric can be:

SNR=9/10dB and SNR=15/16dB for FDD/HD-FDD/TDD with 1Rx.

SNR=6/7dB for FDD/HD-FDD/TDD with 2Rx.

Requirements:

A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%.

Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05

Average BLER with follow CQI shall be greater than or equal to 0.02.