3GPP TSG-RAN WG4 Meeting # 112	R4-2412762

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

Title: 	Views on spatial channel model

Source: 	Huawei, HiSilicon

Agenda item:	8.14.2

Document for:	Discussion

Background

A new study item [1] on spatial channel model for demodulation performance requirements was approved on RAN #104 meeting, which includes following objectives:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

This contribution provides our related views.

- Discussions

All demodulation requirements are defined with TDL model and spatial characteristics can be added by applying correlation matrixes to all taps, which is not realistic since more realistic channel always has different spatial component.

Observation1: The biggest difference between current channel model for RAN4 demodulation requirements and realistic channel model is that different tap always has different spatial component.

However, we don't think such difference impact the rationality of traditional RAN4 demod/CSI performance test since current TDL+ correlation matrix can also cause cross layer interference and imbalance SINR for each layer(with medium and high correlation), which is adequate to verify UE’s MIMO receiver performance. The motivation is that defining more realistic spatial channel model may accommodate more advanced requirements in future, e,g,6G such as AI and beam related test. Therefore, we propose the following:

Proposal 1:  RAN4 shall not define new requirements with new channel model for traditional demod/CSI test, only study the approach of channel modelling, simulation alignment uncertainty and test complexity.

Furthermore, channel capacity is one important metric to verify the reasonability of new channel. Spatial component provides higher channel correlation between each TX-RX branches, which may reduce the freedom and number of supported layers. Therefore, we propose the following:

Proposal 2: RAN4 should guarantee that the new channel model has appropriate channel capacity/freedom. I.e., The new channel model can support high rank and reasonable test SNR range.

CDL channel and extending TDL channel are listed as two candidates. Both approaches have advantages and disadvantages. CDL channel can directly generate channel coefficients referring to the actual transmission process of signal, but the channel capacity is unpredictable. TDL channel requires less input parameters and provides elegant and concise analytical expressions for MIMO channel spatial correlation matrixes but it is often questioned about the oversimplification of MIMO channel characteristics.

Extending TDL channel

As per TS38.901, the approach of TDL extension is indicated as follows:

There are two ways to derive per-tap correlation matrices, CDLs model based and system-level model based, the main difference is that channel characteristics such as delay, angular spread, AOA/AOD/DOA/DOD are fixed for CDL channel but random for system-level model. For simplicity, we prefer to use CDLs model based.

Proposal 3: RAN4 to use CDLs in combination with array assumptions indicated in 38.901 for TDL extension.

According to [2] ([17] noted in the above first yellow sentence), following assumptions are made:

For simplicity, channel modelling is analyzed on 2D plane. Multiple clusters are modelled, each cluster has AOD (Angle of departure), ASD (Angle spread of departure), AOA(Angle of arrival) and ASA(Angle spread of arrival).

BS antenna configuration: Two dual-polarized slant +45° and -45° elements are spatially separated by the distance d, where d is 0.5λ.

BS antenna pattern:

Aθ=-min12θθ3dB2,Am

- Where is defined as the angle between the direction of interest and the bore sight of the antenna,  is the 3dB beam width in degrees, and Am is the maximum attenuation. For a 3 sector scenario  is 70 degrees, 20dB, maximum directional gain of an antenna element GE,max =8dBi

UE antenna configuration: two dual-polarized slant +90° and -90° elements are spatially separated by the distance d, where d is 0.5λ.

UE antenna pattern: omni-directional antenna.

Figure 2-1: Proposed TDL based channel modelling

The correlation matrix for each cluster can be derived from following steps:

Step1: Calculate antenna correlation between BS side for one polarization.

For ith cluster, correlation coefficient of antenna m and n is defined as following:

ρBS,m,ni=φAODi-12ASDφAODi+12ASDej2πλm-ndcosφA(φ)PAS(φ)dφφAODi-12ASDφAODi+12ASDA(φ)PAS(φ)dφ

Where φAODi is angle of departure of ith cluster. ASD is angle spread, A(φ) is antenna gain at  φ, PAS(φ) is power angular spectrum for each cluster. Then we can derive correlation matrix Ai for ith cluster at BS side:

Am,ni=ρBS,m,ni

Step2: Calculate antenna correlation between UE side for one polarization.

Similar as BS side, for ith cluster, correlation coefficient of antenna m and n can be derived as following:

ρUE,m,ni=φAOAi-12ASAφAOAi+12ASAej2πλm-ndcosφA(φ)PAS(φ)dφφAOAi-12ASAφAOAi+12ASAA(φ)PAS(φ)dφ

Where φAOAi is angle of arrival of ith cluster. ASA is angle spread, A(φ) is antenna gain at  φ, PAS(φ) is power angular spectrum for each cluster. Then we can derive correlation matrix Bi for ith cluster at: UE side:

Bm,ni=ρUE,m,ni

Step3: Calculate polarization matrix

The channel polarization matrix is given as following:

S=SvvSvhShvShh

Where Svv=Shh=1, Svh=Shv=10-XPR20,  XPR can be selected from typical values defined in CDL parameter table in 38.827.

The polarization matrixes at the BS and UE are the following respectively

PBS=cosθ1,BScosθ2,BSsinθ1,BSsinθ2,BS

PUE=cosθ1,UEcosθ2,UEsinθ1,BSsinθ2,UE

Where θ1,BS, θ2,BS are the polarization of the antennas of BS and θ1,UE, θ2,UE are the polarization of the antennas of UE. The total channel is the matrix product of BS polarization, channel polarization and UE polarization for the downlink case:

Then polarization correlation matrix is given as follows:

Step4: Calculate the total channel correlation matrix

Channel correlation matrix for ith cluster is given as follows, which is same approach as clause B.2.3.2.1 in 38.101-4

Rspat,i=PAi⨂Γ⨂BiPT

Noted that all above analysis are based on 2D, which can easily be extended to 3D by applying another angular variable.

AOA, AOD, XPR can be reused from AOA(Azimuth angle of Arrival) and AOD(Azimuth of Departure) of CDL table defined in 38.827/38.901, or can be redefined.

PAS can be replaced by reusing table 7.5-3 of 38.901(copied as Table 2-1) or can be redefined by using Laplace distribution or uniform.

Table 2-1 Ray offset angles within a cluster, given for rms angle spread normalized to 1

| Ray number m | Basis vector of offset angles m |
| --- | --- |
| 1,2 | ± 0.0447 |
| 3,4 | ± 0.1413 |
| 5,6 | ± 0.2492 |
| 7,8 | ± 0.3715 |
| 9,10 | ± 0.5129 |
| 11,12 | ± 0.6797 |
| 13,14 | ± 0.8844 |
| 15,16 | ± 1.1481 |
| 17,18 | ± 1.5195 |
| 19,20 | ± 2.1551 |

For information, Laplace distribution is defined as following:

Pφ,σ,φ=N0-2φ-φσ

φ is AOD/AOA of the given cluster, σ is RMS angle spread. Finally, No is the normalization constant:

N0-1=-π+φπ+φexp-2φ-φσdφ

- Proposal 4: Use assumptions and steps in clause 2.1 for TDL extension modelling.

CDL channel

It is agreed to use spatial channel model of 38.827 as start point, which has less random variables compared to 38.901. Following parameters are fixed or converged on some typical candidate values:

Coupling pattern of ray angles are specified as per Table 7.2-6.

BS Antenna Parameters (Mg,Ng,Me,Ne,P,dH,dv) are specified as per Table 7.2-7

Random initial phase are specified as per table 7.2-8

UE velocity and travelling direction are specified based on different channel model.

These parameters can be reused for demodulation test as baseline. However, some parameters and procedures are not specified, which are needed to be further discussed:

Position for UE and BS antennas

- Firstly, RAN4 should define the coordinate of BS and UE antenna elements in the LCS (Local coordinate system) and define the angular rotation variable of LCS to GCS.(αUE, βUE, γUE, αBS, βBS, γBS) . For simplicity, LCS can be assumed to be aligned with GCS to avoid complex coordinate system conversion operation.

Beamforming

- RAN4 should specify the beamforming, i,e, weight vector for each antenna element:38.827 has specified following weight vector for each antenna element:

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe

rtx,max is a spherical unit vector denoting the target beam direction. This parameter highly impacts the performance. For example, if beam direction is aligned with the strongest cluster, the performance may be better, if it is aligned with weakest cluster, the performance may be worse. RAN4 should study and find an appropriate beam for demod test.

Map CSI ports to antenna elements

RAN4 should specify how to map CSI ports to antenna elements. 2/4/8 ports are common configurations for demodulation test and more ports are used for PMI test.  RAN4 should specify how to map CSI ports to antenna elements considering different number of CSI ports may be used for demod test.

UE antenna configurations and radiation pattern

- RAN4 should specify UE antenna configurations and antenna radiation pattern. UE antenna pattern is up to UE implementation for OTA test. Conducted test is considered for FR1 for demod test, so UE antenna configuration and antenna radiation pattern could be assumed.

Proposal 5: RAN4 to use following parameters in 38.827 as baseline for demod tested specific CDL channel model

Coupling pattern of ray angles in Table 7.2-6.

BS Antenna Parameters (Mg, Ng, Me, Ne, P, dH, dv) in Table 7.2-7

Random initial phase in table 7.2-8

UE velocity and travelling direction based on different channel model.

Proposal 6: RAN4 to further discuss following parameters/procedures:

Local coordinate systems for UE and BS

Antenna elements coordinates for UE and BS

Beamforming (Weight vector for each antenna element)

How map CSI ports to antenna elements. (2/4/8/16/32 ports)

UE antenna configurations and radiation pattern

Whether/How to simplify the CDL channel in 38.827

- It is mentioned that all channel models used for demod test only have 12 taps with 5ns time resolution. However, most CDL channel models defined in 38.827 have 23 taps with higher time resolution. CDL channel should also be simplified like did for TDL channel, combining multiple taps is the key operation to reduce the number of taps, which seems difficult for CDL channel, where each cluster has not only power but also angular information, which can be treated as “vector”, so it seems unreasonable to directly add the power of two taps, furthermore, how to derive the angular of new tap is needed to be studied.

- Proposal 7: RAN4 should discuss whether to simplify the CDL channels in 38.827 for the purpose of demodulation test like did for TDL channel. If yes, RAN4 should discuss the approach mentioning that different cluster have not only power but also angular information.

- Conclusion

- In this paper, we provide our views on spatial channel models for demodulation requirements. The proposals are:

Observation1: The biggest difference between current channel model for RAN4 demodulation requirements and realistic channel model is that different tap always has different spatial component.

Proposal 1:  RAN4 shall not define new requirements with new channel model for traditional demod/CSI test, only study the approach of channel modelling, simulation alignment uncertainty and test complexity.

Proposal 2: RAN4 should guarantee that the new channel model has appropriate channel capacity/freedom. I.e., The new channel model can support high rank and reasonable test SNR range.

Proposal 3: RAN4 to use CDLs in combination with array assumptions indicated in 38.901 for TDL extension.

Proposal 4: Use assumptions and steps in clause 2.1 for TDL extension modelling.

Proposal 5: RAN4 to use following parameters in 38.827 as baseline for demod tested specific CDL channel model

Coupling pattern of ray angles in Table 7.2-6.

BS Antenna Parameters (Mg, Ng, Me, Ne, P, dH, dv) in Table 7.2-7

Random initial phase in table 7.2-8

UE velocity and travelling direction based on different channel model.

Proposal 6: RAN4 to further discuss following parameters/procedures:

Local coordinate systems for UE and BS

Antenna elements coordinates for UE and BS

Beamforming (Weight vector for each antenna element)

How map CSI ports to antenna elements. (2/4/8/16/32 ports)

UE antenna configurations and radiation pattern

- Proposal 7: RAN4 should discuss whether to simplify the CDL channels in 38.827 for the purpose of demodulation test like did for TDL channel. If yes, RAN4 should discuss the approach mentioning that different cluster have not only power but also angular information.

- Reference

[1]    RP-241610 New SID: Study on spatial channel model for demodulation 	performance requirements. Nokia
[2]    Asplund, H., Medbo, J., Göransson, B., Karlsson, J., Sköld, J.: "A simplified approach to applying the 3GPP spatial channel model", in Proc. of PIMRC 2006.