3GPP TSG-RAN WG4 Meeting # 112	R4-2412764

# Maastricht, Netherlands, 19th August 2024 – 23rd August 2024

Title: 	Views on BS MMSE-IRC requirements with interference

Source: 	Huawei, HiSilicon

Agenda item:	8.16.3

Document for:	Discussion

Background

A new work item on NR demodulation performance: Phase 5[1] was approved in RANP#104 meeting. One objective is to define BS requirements with inter cell interference.

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

This contribution provides our views

- Discussions

Interference model

The motivation of this WI is to verify BS MMSE-IRC requirements under inter-cell interference, we don’t think receiver algorithm will be changed as interference profiles changing, the test purpose can be achieved if the gain over baseline receiver (MMSE-MRC) is sufficient by using current LTE interference profiles, however, if there is no performance gain, RAN4 can consider other interference profiles.

Proposal 1: RAN4 to reuse the current LTE interference model for evaluation to check the gain over baseline receiver (MMSE-MRC). RAN4 consider other interference profiles only if the gain is not sufficient.

LTE interference profile:

Homogeneous: DIP1=-1.11dB, DIP2=-10.91dB

Heterogeneous: DIP=-0.43Db, DIP2=-13.78dB

Meanwhile, for simplicity, besides the scenarios noted in the objectives in the WI, serving PUSCH and interference PUSCH should have same wave type (CP-OFDM), PUSCH mapping type, bandwidth and DMRS port index.

Proposal 2: For simplicity, serving PUSCH and interference PUSCH should have same wave type, PUSCH mapping type and DMRS port index.

Algorithm

TR.36.884 specifies that interference covariance matrix is derived from DMRS based estimation with granularity of per PRB per TTI:

The MMSE-IRC receiver weight matrix is usually defined as where and denote the estimated channel matrix and covariance matrix, respectively. To obtain the MMSE-IRC receiver weight matrix, the covariance matrix including the sources of inter-cell interference needs to be estimated. The covariance matrix is estimated at DM-RS REs by following equations: , , where the DM-RS symbols are used to estimate the covariance matrix, and Nsp is the number of sampling DM-RS REs. The estimation of interference covariance matrix is performed at per PRB and per TTI basis.

We prefer to unify the algorithm for simulation alignment by using such assumption.

Waveform

Considering DFT-S-OFDM is rare in the real deployment, we propose to only consider CP-OFDM

Antenna configuration

For antenna configuration, we prefer to reuse LTE parameters: 1T2R, 1T4R, 1T8R.

MCS and propagation

MCS2,16,20 can be considered as start point for study and RAN4 to select one for requirements definition.

Propagation conditions

For study purpose, we propose to reuse Rel-15 parameters: TDLB100-400 low for MCS2, TDLC300-100 low for MCS16 and TDLA30-10 low for MCS20. The propagation condition of interference PUSCH can be TDLC300-100 low to cover the worst case. For the requirements definition, we propose to only choose one combination of MCS and propagation conditions.

Bandwidth

Considering BS always perform interference covariance matrixes estimation with 1 RB granularity, bandwidth doesn’t impact MMSE-IRC performance. Considering the target scenario is cell edge, the number of scheduled RB is very small, we propose to select 5MHz for 15kHz SCS and 10MHz for  30kHz for requirements definition.

Proposal 3: RAN4 to consider following parameters as start point:

Antenna configuration: 1T2R, 1T4R and 1T8R for serving PUSCH and interference PUSCH

Waveform:

CP-OFDM

MCS:

CP-OFDM: 2,16,20 for study purpose, choose one for requirements definition.

Propagation conditions:

Serving PUSCH: TDLB100-400 low for MCS2, TDLC300-100 low for MCS16 and TDLA30-10 low for MCS20 for study purpose, choose one for requirements definition.

Interference PUSCH: TDLC300-100 low

SCS: 15kHz and 30kHz

Mapping type: Type A and B

Bandwidth for serving and interference PUSCH:

15kHz SCS:  5MHz

30kHz SCS: 10MHz

Receiver Type:

- MMSE-IRC receiver: DMRS based interference covariance matrix estimation with granularity of per RB per TTI

- MMSE-MRC receiver (For study purpose)

- Conclusion

- This contribution provides our views on BS MMSE-IRC requirements with interference. The proposals are:

Proposal 1: RAN4 to reuse the current LTE interference model for evaluation to check the gain over baseline receiver (MMSE-MRC). RAN4 consider other interference profiles only if the gain is not sufficient.

LTE interference profile:

Homogeneous: DIP1=-1.11dB, DIP2=-10.91dB

Heterogeneous: DIP=-0.43dB, DIP2=-13.78dB

Proposal 2: For simplicity, serving PUSCH and interference PUSCH should have same wave type, PUSCH mapping type and DMRS port index.

Proposal 3: RAN4 to consider following parameters as start point:

Antenna configuration: 1T2R, 1T4R and 1T8R for serving PUSCH and interference PUSCH

Waveform:

CP-OFDM (1st priority),

DFT-S-OFDM (2nd priority)

MCS:

CP-OFDM: 2,16,20

DFT-S-OFDM: MCS2

Propagation conditions:

Serving PUSCH: TDLB100-400 low for MCS2, TDLC300-100 low for MCS16 and TDLA30-10 low for MCS20.

Interference PUSCH: TDLC300-100 low

SCS: 15kHz and 30kHz

Mapping type: Type A and B

Bandwidth for serving and interference PUSCH:

15kHz SCS:  5MHz, 10MHz, 20MHz.

30kHz SCS: 10MHz, 20MHz,40MHz,100MHz

Receiver Type:

- MMSE-IRC receiver: DMRS based interference covariance matrix estimation with granularity of per RB per TTI

- MMSE-MRC receiver

- Reference

[1]    Revised WID on NR demodulation performance: Phase 5. China Telecom, NTT DOCOMO

[2]    TR 36.884