3GPP TSG-RAN WG4 Meeting #112	  R4-2412765

Maastricht, Netherlands, 19 - 23 August, 2024

Agenda Item:	8.17.1

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects for AIML

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #111 [1], we continue to discuss the remaining issues related to the general test framework.

# AI test principle

TR 38.843 [2]: The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition.

- Discussion on ‘what the minimum level performance for each identified scenario and/or configuration is’

For a feature where RAN4 has non-AI requirements, DUT should not only meet but also exceed legacy requirements according to the intention of introducing AI.

Proposal 1: RAN4 test principle of AI is that DUT should not only meet but also exceed legacy non-AI requirements in AI test.

However, in some cases, legacy test environment may not be applicable for AI test. Potential new or enhanced test environment/conditions are under discussion per use case.  For example, in AI/ML CSI compression, CDL or UMa are proposed by some companies to be involved. In AI/ML BM, larger number of beams is proposed.

Observation 1: For some cases, to verify AI performance, new test set up is under discussion.

To define AI requirements in cases where new test set up is introduced, two options are identified.

- Option 1: Define non-AI requirements under new test set up subject to AI. Use the non-AI requirement to define AI requirement, where the AI requirement has to be higher than that of non-AI.

- Option 2: Define absolute requirement of AI without considering the comparison with non-AI.

Both solutions have open issues. For Option 1, the feasibility of aligning non-AI performance baseline under new test environment subject to AI needs to be investigated. Though it may be feasible, heavy workload is expected. For Option 2, without considering the comparison to non-AI, how to reflect the test principle.

Proposal 2: If new test set up is introduced for AI test, two options are identified for defining AI performance requirement.

- Option 1: Define non-AI requirements under new test set up subject to AI. Use the non-AI requirement to define AI requirement, where the AI requirement has to be higher than that of non-AI.

  - FFS: Feasibility of aligning non-AI performance baseline under new test set up

- Option 2: Define absolute requirement of AI without considering the comparison with non-AI.

  - FFS: how to reflect the AI test principle that DUT should not only meet but also exceed legacy non-AI requirements in AI test

- Discussion on ‘what the significant degradation for other scenarios and/or configurations is’

The other scenarios and/or configurations are the scenarios and/or configurations that are not reported as applicable by UE according to RAN2 latest progress. In this case, the performance under other scenarios and/or configurations can be ensured by RAN4 legacy non-AI test. There is no need to verify the AI performance under non-applicable scenarios and/or configurations, since the non-applicable AI functionality/model will not be activated in real network.

Proposal 3: For generalization test, other scenarios and/or configurations are the scenarios and/or configurations that are not reported as applicable by UE according to RAN2 latest progress.

Proposal 4: For generalization test, performance for other scenarios and/or configurations can be ensured by RAN4 legacy non-AI test.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 test principle of AI is that DUT should not only meet but also exceed legacy non-AI requirements in AI test.

Observation 1: For some cases, to verify AI performance, new test set up is under discussion.

Proposal 2: If new test set up is introduced for AI test, two options are identified for defining AI performance requirement.

- Option 1: Define non-AI requirements under new test set up subject to AI. Use the non-AI requirement to define AI requirement, where the AI requirement has to be higher than that of non-AI.

  - FFS: Feasibility of aligning non-AI performance baseline under new test set up

- Option 2: Define absolute requirement of AI without considering the comparison with non-AI.

  - FFS: how to reflect the AI test principle that DUT should not only meet but also exceed legacy non-AI requirements in AI test

Proposal 3: For generalization test, other scenarios and/or configurations are the scenarios and/or configurations that are not reported as applicable by UE according to RAN2 latest progress.

Proposal 4: For generalization test, performance for other scenarios and/or configurations can be ensured by RAN4 legacy non-AI test.

# References

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #111, May 20th – 24th, 2024.

- RP-233133, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, 3GPP TR 38.843, V2.0.0, December 11-15, 2023.