3GPP TSG-RAN WG4 Meeting #112	  R4-2412766

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.17.2

Source:	Huawei, HiSilicon

Title:	Discussion on testability and interoperability issues for beam management

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #111 [1], we continue to discuss the remaining issues related to the test for AIML enabled beam management.

# Test setup for AI-BM

It is observed that the performance of the model may be impacted by a set of factors, including the gNB-side beampattern (which is gNB-side proprietary information), the channel conditions (e.g., the scatter in the physical environment), the receiving gain at UE (i.e., different UE implementation, UE rotation). Therefore, whether and how to define a common training/testing dataset is an open issue.

If the training dataset is not specified, how to ensure the consistency between training and testing needs to be studied. One possible solution is to let the DUT to collect training dataset and complete the model training, followed by performing model test. However, according to R18 study outcome, the training procedure is not considered in RAN4 test due to many reasons, such as test cost/time.

Even if RAN4 revisits the training procedure, how to ensure the following principle which identified in R18 is also an open issue:

The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field. [2]

This is due to the reason that the parameters considered in the test set up which limit the model generalization may be totally different from real deployment. In this case, a DUT has an AI model to pass RAN4 test does not mean it can work well in the field.

Proposal 1: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to ensure that a UE can pass the test but perform poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

After achieving clear answers of the aforementioned questions, we can start to discuss how to define the setup, where the exiting FR2 OTA test set up should be considered as the baseline. Any enhancements on top of existing fancy enough framework should be justified.

Proposal 2: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to ensure that a UE can pass the test but perform poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

Proposal 2: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

# References

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #111, May 20th – 24th, 2024.

- RP-233133, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, 3GPP TR 38.843, V2.0.0, December 11-15, 2023.