3GPP TSG-RAN WG4 Meeting #112	R4-2412775

# Maastricht, Netherlands, 19 - 23 Aug, 2024

Title: 	Discussion on PTRS configuration for UE demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	4.6

Document for:	Discussion

Discussion

In Table 7.2.1 of TS 38.101-4, the PTRS is configured with default value K = 1 and L = 2 with both frequencyDensity and timeDensity absent for all FR2 PDSCH demodulation requirements. This PTRS configuration is considered for RMC determination (including overhead, channel bits calculation) and requirements derivation.

At the same time, the frequencyDensity and timeDensity in phaseTrackingRS is not configured in RAN5 test specification TS 38.508-1. Based on clause 5.1.6.3 in TS 38.213 as extracted below, the UE shall assume PT-RS is not present if MCS < 10 (QPSK) when both frequencyDensity and timeDensity in phaseTrackingRS are not configured. So, there is mismatching between PTRS configuration and RMC determination / requirements derivation for all FR2 QPSK PDSCH demodulation requirements.

| Derivation Path: Table 4.6.3-50 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos1 |  |  |
| phaseTrackingRS | Not present |  | DEMOD_FR1 |
| phaseTrackingRS SEQUENCE { |  |  | DEMOD_FR2 |
| epre-Ratio | 0 |  |  |
| resourceElementOffset | Offset10 |  |  |
| } |  |  |  |
| } |  |  |  |

There is mismatching between PTRS configuration and RMC determination / requirements derivation for all FR2 QPSK PDSCH demodulation requirements.

Actually, evaluations in many WIs shows that the phase noise impact is negligible under QPSK and PTRS is not so necessary under QPSK. Also based on our simulation results, there is negligible performance difference between PTRS present and not present using Example 2 phase noise model at 40GHz. Therefore, to solve such mismatching problem, we propose to only modify the RMC with PTRS not present for all FR2 QPSK PDSCH demodulation requirements and keep the corresponding performance requirements unchanged.

There is negligible performance difference between PTRS present and not present using Example 2 phase noise model at 40GHz.

Only modify the RMC with PTRS not present for all FR2 QPSK PDSCH demodulation requirements and keep the corresponding performance requirements unchanged.

Proposals

In this contribution, we discuss on UE demodulation requirements for NR NTN enhancements. Our observations and proposals are:

- There is mismatching between PTRS configuration and RMC determination / requirements derivation for all FR2 QPSK PDSCH demodulation requirements.

There is negligible performance difference between PTRS present and not present using Example 2 phase noise model at 40GHz.

- Only modify the RMC with PTRS not present for all FR2 QPSK PDSCH demodulation requirements and keep the corresponding performance requirements unchanged.

Reference

TS 38.101-4, User Equipment (UE) radio transmission and reception, Part 4: Performance requirements

TS 38.508-1, User Equipment (UE) conformance specification, Part 1: Common test environment

TS 38.213, Physical layer procedures for control