3GPP TSG-RAN WG4 Meeting #112	R4-2412783

# Maastricht, Netherlands, 19 - 23 Aug, 2024

Title: 	Discussion on NTN testing for NGSO for demodulation performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.8.4

Document for:	Discussion

Background

During RAN#104 meeting, Revised WID [1] on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN was approved. In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements.

Discussion

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios

In previous evaluation and requirements definition, LEO-600 scenario is more typical, so we propose to only consider LEO-600 scenario for NTN testing for NGSO.

Only consider LEO-600 scenario for NTN testing for NGSO.

For simplification of the channel model derivation, assuming satellite orbit is circular orbit, establish 2-D coordinate system centered on the earth center and assume satellite position is fixed and located on the y axis (i.e. the UE is moving relative to the satellite), as shown in Figure 2-1.

Figure 2-1 Geometrical of satellite and UE

The instant satellite speed v is given as:

v=μR+h

The initial angle β of vector geocentric to UE relative to the x axis is given as:

β=π-α-arcsin⁡R∙cosαR+h

The satellite position s is given as:

s=j∙R+h

The UE position u is given as:

u=R∙ej∙β+v∙tR+h

The Doppler fd is given as:

fd=vc∙fc∙realu-su-s

The delay T is given as:

T=|u-s|c

where μ=398600.4418×109m3s2 is standard gravitational parameter, R=6371008.7714m is earth average radius, h=600000m is satellite altitude for LEO-600, α is initial angle of vector UE to satellite relative to vector UE to clockwise side horizon, t is the time, c=299792458ms is the speed of light, fc=2×109Hz is the carrier frequency.

The Doppler trajectory and the delay trajectory for all visible range of the satellite to the UE are shown in Figure 2-2 and Figure 2-3 respectively (i.e. from α=180° to α=0°).

α=30°

Figure 2-2 Doppler trajectory for all visible range for a satellite to the UE

α=30°

Figure 2-3 Delay trajectory for all visible range for a satellite to the UE

We propose to consider above channel model for NTN testing for NGSO. However, it is not necessary to test such long time. A small period (such as 5s) of time should be selected to perform the test starting from the typical parameters (such as when α=30° corresponding to above figures).

Consider above channel model for NTN testing for NGSO. A small period (such as 5s) of time should be selected to perform the test starting from the typical parameters (such as when α=30° corresponding to above figures).

For the test procedure, based on TS38.509, it is feasible for TE to inform UE position by using UE Positioning test mode procedures. Also based on UE receiving ephemeris from TE by SIB periodically, UE can perform the Doppler and delay compensate. The detailed test procedure is shown as following:

- Enter UE Positioning test mode, inform UE position that is randomly selected by TE from a pre-defined position set (including latitude and longitude). The UE position is static during the whole test.

- Based on above UE position, TE calculates initial satellite position at α=30° and generates ephemeris information that is accordance with above channel model.

- TE sends the ephemeris information to the UE via SIB19 at the starting of the test, and add Doppler and delay to the transmitting signal based on above channel model.

- Check whether UE can meet the requirements.

Consider above test procedure for NGSO test.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Only consider LEO-600 scenario for NTN testing for NGSO.

- Consider following channel model for NTN testing for NGSO. A small period (such as 5s) of time should be selected to perform the test starting from the typical parameters (such as when α=30° corresponding to above figures).

The instant satellite speed v is given as: v=μR+h The initial angle β of vector geocentric to UE relative to the x axis is given as: β=π-α-arcsin⁡R∙cosαR+h The satellite position s is given as: s=j∙R+h The UE position u is given as: u=R∙ej∙β+v∙tR+h The Doppler fd is given as: fd=vc∙fc∙realu-su-s The delay T is given as: T=¦u-s¦c where μ=398600.4418×109m3s2 is standard gravitational parameter, R=6371008.7714m is earth average radius, h=600000m is satellite altitude for LEO-600, α is initial angle of vector UE to satellite relative to vector UE to clockwise side horizon, t is the time, c=299792458ms is the speed of light, fc=2×109Hz is the carrier frequency.

Consider above test procedure for NGSO test.

- Enter UE Positioning test mode, inform UE position that is randomly selected by TE from a pre-defined position set (including latitude and longitude). The UE position is static during the whole test.

- Based on above UE position, TE calculates initial satellite position at α=30° and generates ephemeris information that is accordance with above channel model.

- TE sends the ephemeris information to the UE via SIB19 at the starting of the test, and add Doppler and delay to the transmitting signal based on above channel model.

- Check whether UE can meet the requirements.

Reference

RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, RAN#104, Samsung