3GPP TSG-RAN WG4 Meeting # 112	R4-2412784

# Maastricht, Netherlands, August 19th – 23rd, 2024

Title: 	Discussion on introduction of new FR2 UE power class

Source: 	Huawei, HiSilicon

Agenda item:	4.8

Document for:	Discussion

- Background

As per the discussion during last RAN4#111 meeting [1], most companies prefer to specify a new power class for the target use cases instead of extending PC7 to non-RedCap UE. With consideration of the views in the RAN4#111, we proposed a new WI in TSG RAN#104. However, the WI was not approved or even not treated in the RAN. This contribution shares our views on the urgency of the introduction, as well as the related performance requirements that need to be defined for this new power class.

- Discussion

Urgency of the introduction

In China, 26GHz spectrum (24.75-27.5GHz) is issued for innovation to promote microwave development, with mandatory support of 200MHz channel bandwidth for a single carrier and 2 x 200MHz DL CA, where mandatory support 200MHz/CC is also required from NR Rel-15. Hence, PC7 cannot address these conditions. In addition, the local industry standards based on the 3GPP specifications led by CCSA target for the completion by the end of 2024 to facilitate the FR2 industry development. It is noted that further update to the standards is allowed at least two more years later. Hence, it is essential to accelerate this new FR2 power class introduction in order to enable the first industry standards for FR2 to include the new PC in China.

Observation 1: It’s essential to introduce the new power class into 3GPP by the end of 2024, given that local industry standard targets for the completion by the end of 2024 to facilitate the FR2 industry development.

We believe that the acceleration will promote the spread of FR2 market in China as well as lead to the expansion of the FR2 industry development around the world, given that the proposed new FR2 power class with low complexity with clear targeted use cases will shorten the time to market of such FR2 devices, and thus help to speed up FR2 network deployment.

Impact of the new power class on TS 38.101-2

For the target new use cases, the UE device has limited form factor, it is challenging for it to be equipped with multiple panels and a large number of antenna elements on the FR2 bands, such as wearable devices serving as a data hub for others, and small form factor devices for FWA purposes to be used under well designed FR2 network deployment. Hence, it is reasonable and feasible to refer to the requirements for a power class with a similar UE device architecture assumption that is UE Power class 7, i.e., single panel, dual polarization, 2x1 array [2], so some requirements defined for PC7 that are closely related to the UE RF device architecture can be applied for this new power class, such as maximum output power and etc., Link budget analysis is performed as given in the Annex part to evaluate the feasibility to use the PC7 requirements for this new FR2 PC.

Proposal 1: Some PC7 requirements can be applied for this new FR2 power class.

However, the target use cases require the UE device to support larger bandwidth than 100MHz to support higher data rate for service such as real-time ultra-high definition video call and tracking, live streaming and virtual reality interaction in some hotspot scenarios. Mandatory support of 200MHz per CC is required from NR Rel-15 as specified for UE capability channelBWs-DL/ channelBWs-UL that the third / rightmost bit (for 200MHz) shall be set to 1 [3]. RedCap is an exception that only supports up to 100MHz channel bandwidth for FR2. This is also clearly specified in local standardization in China. It’s also noted that the targeted bands are the same as those for Power Class 7, i.e., n257, n258 and n261. Link budget analysis is performed as given in the Annex part to demonstrate the requirements for scenario and service with larger throughput and channel bandwidth.

Thus, new necessary RF requirements shall cover 200MHz and 400 MHz channel bandwidth for single carrier operation.

Proposal 2: New RF requirements shall cover 200MHz and 400MHz channel bandwidth for single carrier operation. Note that targeted bands are n257, n258 and n261.

To meet the higher throughput requirements and better performance, the new power class is allowed to support CA and UL-MIMO that are also not applicable for RedCap, so the corresponding RF requirements for CA and UL-MIMO need to be specified.

Proposal 3: Specify new RF requirements for CA and UL-MIMO for this new FR2 power class.

By considering all aspects for this new FR2 power class (e.g. PC8) as discussed above, we summarize all the requirements to be specified for this new power class as below in Table 1:

Table 1: The summary of RF requirements for the new FR2 PC

| Section | Requirements for PC8 | Comments |
| --- | --- | --- |
| 6.2.1.0 | General | Assumption of lower power non-RedCap UE types for new PC8 |
| 6.2.1.8 | UE maximum output power for power class 8 | The MOP specified in section 6.2.1.7 for PC7 applies. |
| 6.2.2.8 | UE maximum output power reduction for power class 8 | MPR specified in section 6.2.2.3 for PC3 FR2-1 applies. |
| 6.2.3.3.8 | A-MPR for NS_202 for power class 8 | A-MPR for NS_202 specified in clause 6.2.3.3.3 for PC3 applies. |
| 6.2.3.4.8 | A-MPR for NS_203 for power class 8 | A-MPR for NS_203 specified in clause 6.2.3.4.3 for PC3 applies. |
| 6.2A.2.8 | Maximum output power reduction for power class 8 | MPR for PC3 specified in sub-clause 6.2A.2.4.1 for intra-band contiguous UL CA applies. |
| 6.2A.3.3.7 | A-MPR for CA_NS_202 for power class 8 | For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 for PC3 applies. |
| 6.2A.3.4.7 | A-MPR for CA_NS_203 for power class 8 | For intra-band contiguous CA, AMPR specified in sub-clause 6.2A.3.4.3 for PC3 applies. |
| 6.2D.1.7 | UE maximum output power for UL MIMO for power class 8 | The MOP specified in section 6.2.1.7 for PC7 applies. |
| 6.2D.2.7 | UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 8 | MPR for PC3 specified in section 6.2.2.3 applies. |
| 6.2D.3.7 | UE maximum output power reduction with additional requirements for UL MIMO for power class 8 | A-MPR for PC3 specified in section 6.2.3 applies. |
| 6.3.1.5 | Minimum output power for power class 8 | Specify requirements for 200MHz and 400MHz CBW on top of requirements in clause 6.3.1.4 for PC7 |
| 6.3A.1.4 | Minimum output power for CA for power class 8 | Min output power specified in clause 6.3A.1.3 for intra-band contiguous CA for PC3 with up to 400MHz applies. |
| 6.3D.1.4 | Minimum output power for UL MIMO for power class 8 | Specify corresponding requirements based on new clause 6.3.1.5 |
| 6.4.2.1 | Error vector magnitude | EVM for PC3 in FR2-1 applies. |
| 6.4.2.2.9 | Carrier leakage for power class 8 | the relative carrier leakage power specified in subclause 6.4.2.2.4 for PC3 applies. |
| 6.4.2.3.9 | In-band emissions for power class 8 | The average of the in-band emission specified in subclause 6.4.2.3.4 for PC3 applies. |
| 6.4A.2.2.8 | Carrier leakage for CA for power class 8 | For intra-band contiguous carrier aggregation, the carrier leakage requirement specified in clause 6.4A.2.2.4 for PC3 in FR2-1 applies. |
| 6.4A.2.3.8 | Inband emissions for CA for power class 8 | For intra-band contiguous carrier aggregation, the average of the in-band emission specified in clause 6.4A.2.3.4 for PC3 applies. |
| 6.6.9 | Beam correspondence for power class 8 | Requirements specified in clause 6.6.8 for PC7 applies. |
| 7.3.2.8 | Reference sensitivity power level for power class 8 | Specify requirements for 200MHz and 400MHz CBW on top of requirements in clause 7.3.2.7 for PC7 |
| 7.3.4.8 | EIS spherical coverage for power class 8 | Specify requirements for 200MHz and 400MHz CBW on top of requirements in clause 7.3.4.7 for PC7 |

For the detailed updates to the specification for introduction of this new FR2 power class, the company CR is submitted [4] for reference.

Proposal 4: Introduce the new power class based on the proposals 1, 2 and 3 with specification of the requirements as shown in Table 1.

- Conclusion

As per our analysis above, our observations and proposals are summarized as following:

Observation 1: It’s essential to introduce the new power class into 3GPP by the end of 2024, given that local industry standard targets for the completion by the end of 2024 to facilitate the FR2 industry development.

Proposal 1: Some PC7 requirements can be applied for this new FR2 power class.

Proposal 2: New RF requirements shall cover 200MHz and 400MHz channel bandwidth for single carrier operation. Note that targeted bands are n257, n258 and n261.

Proposal 3: Specify new RF requirements for CA and UL-MIMO for this new FR2 power class.

Proposal 4: Introduce the new power class based on the proposals 1, 2 and 3 with specification of the requirements as shown in Table 1.

- Annex

PDSCH

For outdoor hotspot scenarios, NLOS channel is assumed. As per the following configuration parameters, at least 200MHz channel bandwidth is needed for 8K ultra-high resolution autostereoscopic 3D displays with DL 300Mbps throughput required and at least 400MHz channel bandwidth with ISD=300m or 200MHz channel bandwidth with ISD=200m is needed for 8K cloud XR service, DL 500Mbps max throughput. So some service with higher DL throughput, larger channel bandwidth than 100MHz is required.

Table 1: Configuration Parameters

| Parameters | Value |
| --- | --- |
| Scenario | UMa |
| BS antenna heights (m) | 25 |
| UT antenna heights (m) | 1.5 |
| h_E | 1 |
| SCS | 120 |
| Carrier frequency (GHz) | 26 |
| Occupied channel bandwidth for PDSCH Per CC (MHz) | 200, 400 |
| Number of CC | 1 |
| occupied PRB Per CC | 132, 264 |
| ISD(m) | 300, 200 |
| layer | 2 |
| TDD pattern | DDDSU with S = 10:2:2 |
| Symbols with PDCCH | 1 |
| DM-RS configuration | Type 1: 1+1 |
| Thermal noise density (dBm/Hz) | -174 |
| Interference margin(dB) | 1 |
| Noise figure (dB) | 10 |
| UE receiver antenna gain (dB) | 3 |
| Shadow fading margin for data channel (dB) | 6 |
| Propagation condition | TDLC300-7, 3km/h velocity |

PUSCH

Same as above for PDSCH, link budget analysis for both NLOS and LOS is performed for outdoor hotspot scenario and indoor stadium or exhibition center. For some service, such as 5G VoNR+ and live streaming with 4K resolution, 20Mbps throughput is needed. Currently for most services, smaller UL throughput than 20Mbps is required, based on the following configuration parameters analysis, UE 16.4dB EIRP is enough to meet those UL throughput.

Table 1: Configuration Parameters

| Parameters | Value |
| --- | --- |
| Scenario | UMa |
| BS antenna heights (m) | 25 |
| UT antenna heights (m) | 1.5 |
| h_E | 1 |
| SCS | 120 |
| Carrier frequency (GHz) | 26 |
| Occupied channel bandwidth for PDSCH Per CC (MHz) | 200, 400 |
| Number of CC | 1 |
| occupied PRB Per CC | 132, 264 |
| ISD(m) | 300, 200 |
| layer | 1 |
| TDD pattern | DDDSU with S = 10:2:2 |
| Symbols with PDCCH | 1 |
| DM-RS configuration | Type 1: 1+1 |
| EIRP (dBm) | 16.4 |
| Thermal noise density (dBm/Hz) | -174 |
| Interference margin(dB) | 1 |
| Noise figure (dB) | 7 |
| Shadow fading margin for data channel (dB) | 6 |
| Propagation condition | TDLC300-7, 3km/h velocity |

- Reference

[1] R4-2408997 Discussion on extension FR2 UE power class 7 to non-RedCap UE, Huawei, HiSilicon

[2] R4-2206544 WF on the RedCap RF, RAN4#102-e, Ericsson

[3] TS 38.306 NR; User Equipment (UE) radio access capabilities

[4 R4-240xxxx CR on introduction of new FR2 power class, Huawei, HiSilicon