# 3GPP TSG-RAN WG4 Meeting # 112                                   R4-2412790

# Maastricht, Netherlands, 19 – 23 August, 2024

# Title: 	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.16.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices, which was approved in [1], wherein the objectives related to 8Rx for CPE/FWA /vehicles are listed as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we give some initial discussions on the UE performance requirements for 8Rx CPE/FWA/vehicle/industrial device, which include inter-cell interference cancellation and intra-cell inter-user interference cancellation.

# 2 Discussion

As we all know, co-channel inter-cell and intra-cell inter-user interference cancellation performance requirements were introduced in Rel-17. And in Rel-18, RAN4 introduced 8Rx demodulation requirements for PDSCH and CSI performance. With 8Rx, larger performance gain can be verified, due to larger spatial domain freedom compared to 2/4Rx. As a consequence, from demodulation perspective, there are more benefits to define UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices under interference scenarios.

  - Sub-topic 2-1 Inter-cell interference cancellation scenario

Regarding the inter cell interference cancellation scenario, since RAN4 discussed receiver structure in Rel-17, and MMSE-IRC receiver type was captured in [2], since we don’t need to discuss more details anymore. Also the WID noticed that Rel-17 interference model could be reused. From the TS 38.101-4[3], INR1= 7.77dB and INR2=2.29dB were assumed for the first strong interference cell and the second strong interference cell, or INR=7.58dB only for the interference cell.

Proposal 1. Rel-17 INR values can be as a start point, e.g. INR1= 7.77dB and INR2=2.29dB or INR=7.58dB.

Regarding the other simulation parameters,e.g. rank, MCS, antenna configuration, we have the following discussion.

With 8Rx for CPE/FWA /vehicles, which is a high processing capability. The number of layers could be up to 8 layers. In Rel-18 8Rx demodulation requirements, RAN4 introduced 2 layers, 4 layers and 8 layers. And for 2/4 Rx inter cell interference requirements, only considered 1 layer transmission. Since high processing capability, more transmission layers could be considered, e.g. 2 layers and 4 layers form NW scheduling. However, based on our investigation, 8 layers is not a typical scenario in actual scheduling compared to 6layers. Since we prefer to consider 2 layers, 4 layers and 6 layers as starting point.

Proposal 2. Propose to consider 2layers, 4layers and 6layers for inter cell scenario.

Regarding modulation order, RAN4 introduced MCS 19 and MCS 17 for rank 2 and rank 4 for PDSCH demodulation requirements in Rel-18. From capability perspective, we also would like to consider high modulation order. Even if high modulation order is more sensitive to interference caused by more receiver antennas.

Proposal 3. Propose to consider MCS 19 and MCS 17 for rank 2 and rank 4 for PDSCH demodulation requirements.

With CPE/FWA/vehicle devices, which no limitation on the devises size, we believe low correlation could be introduced as least for performance.

Proposal 4. Propose to consider TDLA/ TDLC low correlation as starting point.

And for other parameters, e.g. duplex model, bandwidth and subcarrier spacing , we have following proposal.

Proposal 5. Propose to consider FDD/TDD, 10M/15kHz and 40M/30kHz for demodulation requirements.

More further CQI requirements, INR = 10.04dB was defined in TS38.101-4 for inter-cell interference under fading channel conditions. On the other side, static channel was used for the interference model and in case for white Gaussian noise model Cell 2 is not present.

Compared to legacy 2/4Rx requirement, there is no difference for CSI processing. With 8Rx capability, rank 4 could be considered as starting point.

Proposal 6. Rel-17 INR values can be reused, e.g. INR= 10.04dB.

Proposal 7. Propose to consider rank 4 to define CQI requirements under fading channel conditions.

  - Sub-topic 2-2 Intra-cell inter users interference cancellation scenario

Regarding the intra-cell inter users interference cancellation scenario, since RAN4 defined requirements for 2/4Rx in Rel-17 for MMSE-IRC receiver. Since RAN4 can follow the approach of Rel-17 as starting point, e.g. duplex model, bandwidth and subcarrier spacing, etc.

Proposal 8. RAN4 can follow the approach of Rel-17 as start point, e.g. duplex model, bandwidth and subcarrier spacing, etc.

From our understanding, following simulation parameters may need to be considered again, e.g. rank, MCS, antenna configuration and propagation condition. With 8Rx of CPE/FWA /vehicles, we believe that rank 4+4 can be introduced, which is different from legacy requirements and also can verify the processing capability of 8Rx.

Regarding modulation order, we prefer considering MCS 13 and MCS 17 as starting point.

Observation1. Rank 4+4 can be introduced, which is different from legacy requirements and also can verify the processing capability of 8Rx.

Proposal 9. Propose to consider rank 2+2 and rank 4+4 to define requirements.

Proposal 10. Propose to consider MCS 13 and MCS 17 as starting point.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for MU-MIMO demodulation requirements , The conclusions are:

Observation1. Rank 4+4 can be introduced, which is different from legacy requirements and also can verify the processing capability of 8Rx.

Proposal 1. Rel-17 INR values can be as a start point, e.g. INR1= 7.77dB and INR2=2.29dB or INR=7.58dB.

Proposal 2. Propose to consider 2layers, 4layers and 6layers for inter cell scenario.

Proposal 3. Propose to consider MCS 19 and MCS 17 for rank 2 and rank 4 for PDSCH demodulation requirements.

Proposal 4. Propose to consider TDLA/ TDLC low correlation as starting point.

Proposal 5. Propose to consider FDD/TDD, 10M/15kHz and 40M/30kHz for demodulation requirements.

Proposal 6. Rel-17 INR values can be reused, e.g. INR= 10.04dB.

Proposal 7. Propose to consider rank 4 to define CQI requirements under fading channel conditions.

Proposal 8. RAN4 can follow the approach of Rel-17 as start point, e.g. duplex model, bandwidth and subcarrier spacing, etc.

Proposal 9. Propose to consider rank 2+2 and rank 4+4 to define requirements.

Proposal 10. Propose to consider MCS 13 and MCS 17 as starting point.

# 4 References

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements.

- TR. 38.833. V1.2.0.

- TS 38.101-4. V18.2.0.