# 3GPP TSG-RAN WG4 Meeting # 112                                   R4-2412792

# Maastricht, Netherlands, 19 – 23 August, 2024

# Title: 	Discussion on testability and interoperability issues for AI-CSI

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	8.17.4

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

During the last meeting, some conclusions has been reached for CSI compression and CSI prediction[2]. In the following section, we will provide the detailed discussions for CSI compression and prediction.

# 2 Discussion

Option 3 for 2-side model

Agreements about the parameters used in the simulations for checking the feasibility of testing options for CSI compression and prediction: Agreement: Parameters agreed are just for the feasibility study of testing options. If/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

At the last meeting, RAN4 discussed how to align the simulation results between different companies for option 3. RAN4 tried to align the AI model structure, parameters and other related parameters., After email discussion, RAN4 agreed on all AI mode parameters for simulation evaluation. Hence, we provide our simulation results based on the assumptions from RAN4. The detailed assumptions as shown in Appendix. The test metric of SGCS as shown in following.

Table 1. Simulation results for rank1

| Model structure |  | CNN |  | eType II |
| --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | / |
|  |  | UMa |  | UMa |
| 64 bits | UMa | 0.7294 |  | 0.6803 |
| FLOPS and size |  |  | FLOPS: Encoder: 46.966M Decoder: 739.965M Parameters Num: 1.928M | / |

Table 2. Simulation results for rank 2

| Model structure |  | CNN |  | eType II (118bits) |  |
| --- | --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | / |  |
|  |  | UMa |  | UMa |  |
| 64 bits+64bits | UMa | Rank 1 | 0.7228 | Rank 1 | 0.6870 |
|  |  | Rank 2 | 0.5865 | Rank 2 | 0.5538 |
| FLOPS and size |  | FLOPS: Encoder: 46.966M Decoder: 739.965M Parameters Num: 1.928M |  | / |  |

We provide the simulation results for rank 1 and rank 2. Also we give the legacy eType II codebook feedback results. Simulation results demonstrate that the AI CSI compression indeed achieves performance improvement compared to legacy eType II codebook. On the other hand, for the rank 2 scenario, the first layer outperforms the second layer results.

Next, we intend to explore the aspects that follow the alignment of results. As expected in RAN4, we aspire to the alignment of results among different companies. The first obvious issue is that the required span among different companies. It is evident that RAN4 never study the span for SGCS or other test metrics besides SNR value in RAN4 testing conformance. If RAN4 intends to achieve conclusions that can be aligned among different companies, it should first establish that the extent of the span that can be agreed among different companies. Based on our observations, when considering low overhead feedback, the variation in the number of bits has a significant impact on the outcome; conversely, with high overhead feedback, changes in bit count barely influence the outcome.

Moreover, regarding the demodulation requirements for AI CSI compression, even RAN4 agreed on using CNN model for alignment evaluation. However, this is not the final requirements. From RAN4 demodulation requirements perspective, conformance testing is based on minimum requirements, additionally requirements are based on implementation. We noticed that different companies provided different models in last meeting, even the same models still have different parameters configuration, which gives rise to different outcomes in theoretically. Unfortunately, RAN4 has not evaluated the extent of differences in results from different models or parameter configurations. From our understanding, RAN4 requires more evaluations to validate the impacts of such differences on the outcomes.

On the other side, model structures rely on companies implementation. It is well-known that AI is often perceived as a block box, making it challenging to analyze theoretically through formulas or principles. In the past, for other demodulation requirements, RAN4 has established strict guidelines to govern process from Release 15. For AI- driven work item, even more stringent regulations are imperative.

Observation 1. AI CSI compression indeed achieves performance improvement compared to legacy eType II codebook from simulation results.

Observation 2. RAN4 built a strict guidelines to govern process for Release 15 for demodulation requirements.

Observation 3. If RAN4 would like to study feasibility for CSI compression, new regulations should be considered for AI-based KPI.

Observation 4. RAN4 requires more evaluations to validate the impacts of such differences on the outcomes.

Option 4 for 2- side model

At the previous meeting, RAN4 deliberated on potential candidate options for option 4, namely the means of achieving interoperability between DUT vendors and TE vendors. In TS 38.843, we defined partial knowledge in RAN4 specification for decoder to achieve interoperability. With regard to this partial knowledge of the decoder, it can bring more flexible approaches for TE vendors. To ensure interoperability, we need to define more knowledge between DUT and TE side. All candidate options as shown in following table.

- Option 1: training data/aggregated data - Option 2: reference encoder - Option 3: reference encoder+decoder

For above options, we want to give some analysis form our side. Firstly, if RAN4 standardizes a training data, which can be derived from reference encoder/decoder according generation procedure. And reference encoder/ decoder is also full defined in RAN4 specification. DUT vendor and TE vendor could use the training data to train AI/ML model, e.g. encoder and decoder models. A TE vendor could use training data and partial knowledge to train their own encoder and decoder. A DUT vendor also could use training data and partial knowledge to design their own encoder and decoder. DUT vendor and TE vendor can guarantee the better performance on their side.

As mentioned before, RAN4’s motivation is to achieve interoperability, which means RAN4 must guarantee that any encoder paired with any TE decoder should result in the same performance. In other words, TE decoder should be paired with any DUT decoder during the test.

However, the test decoder with partial knowledge, considering more implementations aspects during its design phase. Even all TE vendors use same model structure, it is a challenging to expect the same performance during test. Also it is difficult to ensure that DUT vendor and TE vendor have same model parameters during training their own decoder and test decoder.

On the other hand, the construction of training data is intimately related to the implementation of the physical layer, such as channel estimation, and system simulation design. Even RAN4 has agreed some system level assumptions to generate training data. Ensuring consistency among the training data provided by various companies remains a challenge task unless RAN4 demonstrates that distinct training data can also lead to convergence in the final results. Actually, we expect to reach a conclusion on this issue in the next meeting, as RAN4 unanimously agreed to use CNN model for result calibration. If the outcomes from different companies cannot be aligned, this would imply training data discrepancies in partial knowledge are irreparable.

Figure 3. Training data based for option 4 test decoder

Base on above analysis, we acknowledge that the validity of the training data still requires validation in the partial knowledge for option 4.

And regarding option 4 with aggregated data, which based on the contributions of all companies and capturing all aggregated datasets in the specification. Firstly, RAN4 can achieve some agreements for part of parameters, all companies can design encoder/ decoder pair to contribute the training data. DUT and TE vendors can use the aggregated datasets to train their own encoder/decoder model. Since partial knowledge is captured in RAN4 specification, TE vendor can use partial knowledge and aggregated datasets to train the test decoder, even if partial knowledge provide more flexibility to TE vendor, but the aggregated datasets include all DUT vendor’s training data, which depends on its own encoder model structure and parameters. Compared to training data, we believe that the aggregated datasets has more possibility to achieve interoperability and testability.

Figure 4. Aggregated datasets based for option 4 test decoder

Next, we want to share our view from reference perspective. In last meeting, reference encoder and decoder were agreed to use in RAN4 discussion as least for simulation alignment / requirements derivation for option 3. Reference decoder is the test decoder for option 3. From test perspective, we think reference encoder/decoder is still needed for requirements derivation, which can be assumed as baseline model structure. As we all known, option 4 is allowed to provide more flexibility to TE vendor or DT vendor. If RAN4 defined the reference encoder/decoder, the minimum performance requirements can be guaranteed. Even DUT or TE vendor can perform updates based on partial knowledge, it also ensure that the performance meets the minimum requirements. Since the reference encoder/ decoder is necessary to ensure that the test results meet or surpass the minimum requirements.

On the other hand, reference model can guarantee the feasibility of implementation from partial knowledge. At least, RAN4 is not expect that the performance degradation due to flexibility. The worst case is that the reference decoder is the test decoder itself.

If RAN4 only standardizes reference encoder, it is a challenging to guarantee the interoperability and testability during test. DUT vendor and TE vendor still lacks sufficient information to guarantee the decoder design consistency. Hence, latent space may be needed to be considered. Regarding latent space, we prefer to consider the latent before quantization. From our understanding, the latent before quantization represents the effective feature information extracted by AI compression, which also determines the efficacy of feature representation for input reconstruction. The varying sizes of effective features also determine the performance of reconstruction. Once the size of the effective features values is determined, the final performance of reconstruction is also determined. Regardless of the quantization method employed. From decoder perspective, the design of the AI decompression module is also based on the extracted effective features. Consequently, we think that the latent space is necessary for ensuring the implementation of decoder. Even if test decoder is partial knowledge, reference encoder and latent space could ensure the interoperability between DUT and TE vendors.

Proposal 4. To consider standardizing the reference model or reference encoder and latent space for option 4.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for AI-CSI , The conclusions are:

Observation 1. AI CSI compression indeed achieves performance improvement compared to legacy eType II codebook from simulation results.

Observation 2. RAN4 built a strict guidelines to govern process for Release 15 for demodulation requirements.

Observation 3. If RAN4 would like to study feasibility for CSI compression, new regulations should be considered for AI-based KPI.

Observation 4. RAN4 requires more evaluations to validate the impacts of such differences on the outcomes.

Proposal 4. To consider standardizing the reference model or reference encoder and latent space for option 4.

# 4 References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R4-2401570, WF on AI/ML. Qualcomm Incorporated.

- 3GPP TS38.101-4.

# 5 Appendix

- System simulation assumption

Table 5-1. System simulation assumptions for training data

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | ideal channel estimation |

- AI model structure