3GPP TSG-RAN WG4 Meeting #112	R4-2412799

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on RRM impact for NR_FR1_lessthan_5MHz_BW_Ph2

Agenda item:	8.4.3

Document for:	Discussion

Introduction

In Rel-18 RAN4 has been working on the Work item NR_FR1_lessthan_5MHz_BW. The work item in Rel-18 only included single carrier operation. RAN plenary has agreed on a new Rel-19 WI related to NR_FR1_lessthan_5MHz_BW which introduces CA on top of the Rel-18 solution [1].

Justification:

In Release-18, RAN4 specify requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz RAN4, including in bands n100, n106, n26, n28 and n85 Carier Aggregation of a 3MHz carrier with one or more 3MHz or wider bandwidth carriers will increase the total available network capacity and hence the value of the 3MHz carrier. In fact, it in some cases may enable the use of spectrum blocks that otherwise would not be used at all for 5G. Dual Connectivity of a 3MHz carrier with one or more 3MHz or wider bandwidth carriers will increase radio coverage and cell hand-over performance. For railways the latter is particularly relevant for handovers for example between tunnel entries or exits, and the open field network coverage areas.

In this paper we give some initial views on the WI ‘NR channel BW less than 5MHz for FR1 Phase 2’ related to expected RRM impact.

Discussion

The WID [1] includes following objectives:

RP-240832: - Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

One part of the work for RRM will be to define any necessary new RRM requirement related to introduction of Carrier Aggregation and Dual Connectivity of a 3MHz carrier with one or more 3MHz or wider bandwidth carriers.

Hence, we see that Rel-19 would need to include:

- RRM requirements for inter-band CA for combinations introduced in RF part

- RRM requirements for inter-band DC for combinations introduced in RF part

These combinations could include a 3MHz carrier and:

- a 3MHz carrier

- more a 3MHz carriers

- a wider bandwidth carrier

- more wider bandwidth carriers

The exact combinations will be decided by RF session. However, from RRM point of view the main work is at least including defining the delay requirements related to such 3MHz carrier used for CA or DC.

Based on the Rel-18 work and discussions, we do not expect any other changes at the moment.

RAN4 to work on defining the SCell and PSCell delay requirements related to such 3MHz carrier used for CA or DC.

Hence, we expect that the RRM work will impact requirements listed in section 8. More precisely we expect that following sections may need to be updated:

- 8.3 SCell Activation and Deactivation Delay

- 8.9 PSCell Addition and Release Delay

- 8.9A Conditional PSCell Addition Delay

- 8.11 PSCell Change

- 8.17 SCG Activation and Deactivation Delay

Besides this, there might be a need for some minor clarification in the specification.

It is not fully clear if there is a need for changes to:

- 8.9C Subsequent Conditional PSCell Addition Delay.

- 8.11B Conditional PSCell Change

- 8.11E Subsequent Conditional PSCell Change

RAN4 can discuss this further.

Besides this, the WID of course also requests RAN4 to specify necessary performance requirements.

RAN4 to discuss the expected RRM scope of a device supporting the NR channel BW less than 5MHz for FR1 Phase 2 feature. At least the following sections can be considered:

- 8.3 SCell Activation and Deactivation Delay

- 8.9 PSCell Addition and Release Delay

- 8.9A Conditional PSCell Addition Delay

- 8.11 PSCell Change

- 8.17 SCG Activation and Deactivation Delay

Conclusion

In this paper we give some initial views on the WI ‘NR channel BW less than 5MHz for FR1 Phase 2’ related to expected RRM impact.

- RAN4 to work on defining the SCell and PSCell delay requirements related to such 3MHz carrier used for CA or DC.

We also provide an initial list of potentially impacted sections. Additionally, RAN4 would need to discuss the expected RRM scope of a device supporting the NR channel BW less than 5MHz for FR1 Phase 2 feature in order to make correct scoping of which requirements RAN4 should be working on.

RAN4 to discuss the expected RRM scope of a device supporting the NR channel BW less than 5MHz for FR1 Phase 2 feature. At least the following sections can be considered:

- 8.3 SCell Activation and Deactivation Delay

- 8.9 PSCell Addition and Release Delay

- 8.9A Conditional PSCell Addition Delay

- 8.11 PSCell Change

- 8.17 SCG Activation and Deactivation Delay

- References

- RP-240832, New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN, vivo