3GPP TSG-RAN WG4 Meeting #112	R4-2412800

Maastricht, Netherland, Aug 19 – 23, 2024

Source:	Nokia

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Agenda item:	8.15.3

Document for:	Discussion

Introduction

In RAN#103, the Rel-19 WI on RRM phase 5 enhancements for Fast SCell activation for UE supporting Rel-18 EMR is approved in RAN#103 [1].

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this paper, the requirements for fast SCell activation are discussed.

Discussion

Scope of the work

In RAN4#111 meeting [2], a work plan was discussed to provide timeline of completion of fast SCell activation for UE supporting R-18 EMR as summarized below.

| RAN4 Meetings | Core requirements | Performance requirements |
| --- | --- | --- |
| RAN4#112 | Feasibility to use the EMR report Feasible scenarios identification | - |
| RAN4#112bis & 113 | Delay requirements Side conditions and signaling enhancements | - |
| RAN4#114 | CRs on requirements and side conditions | - |
| RAN4#114bis | Maintenance phase | Test case identification |
| RAN4#115 | Maintenance phase | Test case agreement and work split |
| RAN4#116 | Maintenance phase | CRs on test cases |

Fast Scell activation delay target

During the Rel-18 eEMR work, we presented in the paper R4-2302247  system-level evaluations showing the benefits of faster SCell establishment for both network and UE. To summarize, faster resume/setup enables user data transmission with lower latency and higher throughput, enhances network capability of performing load balancing, and contributes to lower UE energy consumption. Since the bandwidth parts are usually larger in FR2 than FR1, the achievable gain of a faster SCell/PScell setup is considerable. Furthermore, with a shorter setup delay we observe a reduction in UE energy consumption due to faster offload and release of the connection.

Fast DC/CA resume/setup enables data DC/CA usage with lower latency, higher throughput, enhances load balancing and enables lower UE energy consumption.

The Figure 1 recaps the simulation setup where we considered the delay from SCell when the PCell was capable of transmitting data. This means that any delay from this point onwards causes, in many cases, that the data is transmitted via PCell instead of SCell. If the setup delay of the Scell is not very fast, the benefits of this early data transmission is diminishing.

Figure 1: SCell Setup delay – Note that RRC delay is not included in the simulation scenarios

The following table illustrates the gain from throughput when the offered load is 40 Mbps. We can observe that for this type of traffic load, there is significant benefit if the Scell setup is less than 100ms.

Table 1: Gain in throughput from achieving shorter SCell setup delay when offered load is 40 Mbps

| Figure 2: 95 percentile user throughput –Users in good radio conditions benefit from the extra capacity | Figure 3: 5 percentile user throughput – Users in bad radio conditions benefit from lower SCell setup time, offloading will be improved |
| --- | --- |
| Figure 4: Mean user throughput | Conclusions: - UEs in good radio conditions benefit from the extra capacity - UEs in bad radio conditions benefit from lower SCell setup time - There is significant performance gain for both network and UEs target delay should be less than 100ms - Network offloading effect is significant |
| Figure 5: Network clearly benefits for faster SCell setup time | Figure 6: Average file transmission delay increases as the SCell setup delay increases |

Load does not make a major difference for the best users (95 prc) as the scenario is dominated by fast access to high throughput. SCell setup delay has significant effect on the peak user throughputs. For worst 5 prc users, load starts to dominate but both users and the network of a cell benefit from the offloading effect. SCell setup delay has significant, positive, effect on the coverage user throughputs.

RAN4 system-level simulation results show that low SCell setup delays have significant, positive, impact on UEs with different radio link conditions and network load. RAN4 shall specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay).

Fast SCell activation for UE supporting Rel-18 EMR should aim to achieve less than 100ms activation delay. RAN4 to discuss the options how this is achieved.

Rel-18 eEMR

The following captures the rel-18 EMR for the case where UE is not performing measurements during and / or after the connection setup. The following text includes some of the corrections we proposed in R4-2412428  based on the approved CRs and latest RAN2 status.

### 4.7.3 Measurement Report Requirements A UE shall perform validity check and report valid measurement results: - if the UE is supporting measValidationReportEMR-r18 and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, - if the UE is supporting measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

The release 18 framework allows validity check to be performed, but it also allows network not to configure validityDuration, which means that the actual validation is left somewhat up to UE implementation. From SCell setup point of view, both options may lead to successful SCell setup and they should be considered in the scope of this WI.

Validity check is only performed for configured corresponding carriers if the measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured.

If the UE has not been configured with measReselectionValidityDuration-r18, the UE may still have valid measurement results that will result in successful SCell establishment, and which meet the measurement reporting requirements.

Rel-19 Fast SCell WI supports the case where  measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured and are not configured

On the other hand, the ValidityDuration-r18 allows the validity duration to be configured as very long time (e.g. 100s). This means that the measurement results can be valid, even if the UE has not performed any measurement results, and using those measurements may cause SCell setup to fail as the valid measurements do not correspond to real radio conditions.

RAN4 to discuss how to enable Fast SCell activation when the validity duration is configured with high values.

One of the challenges of current reporting framework is that the network may only pull the idle-mode report with UE information request / response framework. This means that when the Scell is activated based on idle-mode (either EMR or reselection) reporting, the Scell can be only activated after UE transmits information response message.

Discuss if improvements to idle/inactive-mode reporting framework are needed, and possibly send LS to RAN2.

In RAN4 #109 meeting, after an extensive discussion over multiple meetings, RAN4 managed to reach an agreement on considering the case where the UE would perform additional measurements starting from RRC connection setup procedure. RAN4 endorsed a CR R4-2321635 in the meeting, but due to the required efforts from other groups, it was removed in plenary.

Due to limited time, RAN4 agreed CR R4-2321635 was removed in RAN plenary.

## 4.x Measurement reporting for fast CA/DC setup ### 4.x.x Introduction A UE supporting [solution based on enhanced measurement], the UE may perform additional measurement starting from RRC connection setup/resume procedure. Measurement configuration for fast CA/DC setup is provided [for the carrier by higher layers]. The UE may perform additional measurement after paging reception for RRC setup/resume or after first RACH preamble transmission for RRC setup/resume. #### 4.x.x Measurement reporting Measurements reported by UE supporting [solution based on enhanced measurement] shall meet the accuracy requirements defined in [10.x]. The UE shall follow the reporting configuration by the higher layers.

RAN4 to discuss if the UE may perform additional measurement starting from RRC connection setup/resume procedure

Scell Activation Delay

Once the UE receives the SCell activation command, it must perform the necessary actions and transmit a valid CSI report within a specified delay. This delay, within which the UE must activate the SCell, is influenced by several factors, including HARQ delay (the timing between DL data transmission and acknowledgment), RRC procedure delay, CSI reporting delay, activation time (the SCell activation delay in milliseconds), and (in some cases) an additional constant value.

Activation time is governed by various parameters and scenarios, as detailed in TS38.133 section 8.3, for SCells in EN-DC, standalone NR carrier aggregation, NE-DC, or NR-DC configurations. As an example, for deactivated Scell, when the PCell and the target SCell are configured as FR1-FR2-1 CA activation delay can be specified as below;

- If the target SCell is known to UE, when semi-persistent CSI-RS is used for CSI reporting, then activation delay (Tactivation_time) equals: - 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), - If target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). Where TSMTC_MAX: is the SMTC periodicity of SCell being activated (bounded to a minimum value of 10ms). Trs: is the SMTC periodicity of the SCell being activated. TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC. TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC. TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state. TL1-RSRP, measure is L1-RSRP measurement delay. TL1-RSRP, report is delay of acquiring CSI reporting resources. Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to SCell activation command for known case or first valid L1-RSRP reporting for unknown case. Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to SCell activation command for known case or first valid L1-RSRP reporting for unknown case. Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to SCell activation command for known case and first valid L1-RSRP reporting for unknown case. TRRC_delay is the RRC procedure delay as specified in TS38.331. Details of the exact above parameter settings, and how to calculate the activation delay can be found in TS38.133 Section 8.3.

Upon examining the activation delay components outlined above, it becomes evident that there are substantial differences in activation delay (in order of hundred milli seconds) between scenarios with known and unknown conditions.

There are significant differences in Scell activation delay between known and unknown conditions.

For Scell activation, known and unknown conditions for Scell are then defined as summarized in below [TS 38.133]:

In FR1, SCell is known if it has been meeting the following conditions, and otherwise SCell is unknown: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions in TS38.133. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in 38.133 clause 9.2 and 9.3. In FR2 bands, the SCell is known if it has been meeting the following conditions, and otherwise SCell is unknown: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell specified identification conditions, and the TCI state is selected based on one of the latest reported SSB indexes.

RAN4 study and if needed, specify known and unknown conditions for Scell activation based on Rel-18 eEMR measurement report.

Conclusion

The paper the following Observations and Proposals were made:

Observation 1: Fast DC/CA resume/setup enables data DC/CA usage with lower latency, higher throughput, enhances load balancing and enables lower UE energy consumption.

Proposal 1: RAN4 system-level simulation results show that low SCell setup delays have significant, positive, impact on UEs with different radio link conditions and network load. RAN4 shall specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay).

Proposal 2: Fast SCell activation for UE supporting Rel-18 EMR should aim to achieve less than 100ms activation delay. RAN4 to discuss the options how this is achieved.

Observation 2: Validity check is only performed for configured corresponding carriers if the measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured.

Observation 3: If the UE has not been configured with measReselectionValidityDuration-r18, the UE may still have valid measurement results that will result in successful SCell establishment, and which meet the measurement reporting requirements.

Proposal 3: Rel-19 Fast SCell WI supports the case where  measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured and are not configured

Proposal 4: RAN4 to discuss how to enable Fast SCell activation when the validity duration is configured with high values.

Proposal 5: Discuss if improvements to idle/inactive-mode reporting framework are needed, and possibly send LS to RAN2.

Observation 4: Due to limited time, RAN4 agreed CR R4-2321635 was removed in RAN plenary.

Proposal 6: RAN4 to discuss if the UE may perform additional measurement starting from RRC connection setup/resume procedure

Observation 5: There are significant differences in Scell activation delay between known and unknown conditions.

Proposal 7: RAN4 study and if needed, specify known and unknown conditions for Scell activation based on Rel-18 eEMR measurement report.

References

- RP-240830, “New WID: WI resulting from discussion on RRM enhancements,” vivo, RAN#103, Maastricht, Netherlands, Mar. 18th – 22nd, 2024.

- R4-2408529, “Work plan for NR Radio Resource Management (RRM) Phase 5,” Ericsson, RAN4#111, Fukuoka, Japan, May 20 – 24, 2024.