3GPP TSG-RAN WG4 Meeting #112	R4-2412801

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on LP-WUS RRM simulation assumptions and results

Agenda item:	8.22.4.1

Document for:	Discussion

Introduction

In this contribution we discuss about simulation assumptions and LP-RSRP measurements, and provide some preliminary simulation results. The core paper is presented in R4-2412802.

Related to simulations, the following agreements were made.

Issue 2-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode Agreement: - The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study. - Aspects not considered in RAN1 (if any) can be further discussed in RAN4. Issue 1-4-2: LP-SS periodicity for evaluation Agreement: - As starting point for RAN4 simulation purpose, uses 320 ms as the LP-SS periodicity in IDLE/Inactive mode, and other values based on RAN1/2 discussion are not precluded. - Not consider the MR DRX configuration in the simulation.

Discussion

For the simulation purposes, we think it is beneficial to align parameters with RAN1 as far as reasonably possible. RAN1 is still working on various fundamental aspects of LP-WUS and LP-SS design. To benefit from the RAN1 simulations as a baseline for RAN4 work, we would expect the RAN4 simulation assumptions to follow.

LP-RSRP measurement metric for LP-WUR

Traditionally, RSRP is used as the measurement metric to determine the cell coverage/range. In the LP-WUS context, it is one of the possible reliable metric to determine the RRM relaxation thresholds. To study the LP-RSRP estimation accuracy, a time correlated fading of 1.28s is used with 1e6 samples for the LP-RSRP evaluation under TDL-C 300ns channel. In this evaluation, we aimed to determine the value of X, which represents the number of measurements needed within 1.28s that are spaced at equal interval. The following table shows the percentile distribution of LP-RSRP averaging under different SNR operating point. We have determined LP-RSRP based on the signal energy received during the ON duration corrected with the (noise and interference) energy received in OFF duration of Manchester encoded LP-SS, i.e., EON-EOFF.  Table 1 outlines the LP-RSRP error distribution with different operating SNR condition. The “Delta” column highlights half the deviation between 90%ile and 10%ile value. As can be seen that the LP-RSRP deviation is approximately within ±2.5dB error margin for values with X having 320ms or less.

From the table we can notice that the impact of fading on the LP-RSRP estimation accuracy improves as the number of samples used for averaging, i.e., X, increases from 1→16. Furthermore, the LP-RSRP estimation is robust against the SNR as there is only a little variation across when looking at each row. The impact of M on the LP-RSRP estimation accuracy is also not that drastic to gain any obvious attention. However, as the value of X increase from 1→4, the LP-RSRP estimation accuracy improves noticeably, whereas increasing from 4→16, the gain is only marginal. This ensures that the effect of fading can be normalized by taking at least X≥4 samples that are spread across 1.28sec interval.

Table 1. Measured LP-RSRP percentile values for Manchester encoded OOK with M∈{2,4}  and X∈1,2,4,8,16.

| M | X | interval |  | SNR 0dB |  |  | Delta |  | SNR 6dB |  |  | Delta |  | SNR -3dB |  |  |  | Delta |  |  |  | SNR -9dB |  |  |  | Delta |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ms) |  | 10%ile | 50%ile | 90%ile |  |  | 10%ile | 50%ile | 90%ile |  |  | 10%ile | 50%ile | 90%ile |  |  |  |  | 10%ile |  | 50%ile | 90%ile |  |  |
| 2 | 16 | 80 |  | -2,15 | -0,39 | 1,28 | 1,715 |  | -2,1 | -0,36 | 1,29 | 1,695 |  | -2,22 | -0,45 | 1,24 | 1,73 |  |  |  | -2,41 |  | -0,6 | 1,1 | 1,755 |  |
| 2 | 8 | 160 |  | -2,31 | -0,44 | 1,36 | 1,835 |  | -2,25 | -0,4 | 1,37 | 1,81 |  | -2,38 | -0,48 | 1,33 | 1,855 |  |  |  | -2,59 |  | -0,65 | 1,18 | 1,885 |  |
| 2 | 4 | 320 |  | -3,26 | -0,58 | 1,72 | 2,49 |  | -3,17 | -0,54 | 1,75 | 2,46 |  | -3,32 | -0,64 | 1,71 | 2,515 |  |  |  | -3,56 |  | -0,82 | 1,6 | 2,58 |  |
| 2 | 2 | 640 |  | -4,81 | -0,9 | 2,35 | 3,58 |  | -4,7 | -0,87 | 2,36 | 3,53 |  | -4,93 | -0,96 | 2,32 | 3,625 |  |  |  | -5,18 |  | -1,14 | 2,18 | 3,68 |  |
| 2 | 1 | 1280 |  | -7,16 | -1,58 | 3,03 | 5,095 |  | -7,02 | -1,53 | 3,05 | 5,035 |  | -7,24 | -1,65 | 3,02 | 5,13 |  |  |  | -7,73 |  | -1,88 | 2,89 | 5,31 |  |
| 4 | 16 | 80 |  | -2,27 | -0,47 | 1,22 | 1,745 |  | -2,17 | -0,41 | 1,27 | 1,72 |  | -2,38 | -0,56 | 1,17 | 1,775 |  |  |  | -2,63 |  | -0,78 | 0,98 | 1,805 |  |
| 4 | 8 | 160 |  | -2,42 | -0,51 | 1,3 | 1,86 |  | -2,31 | -0,44 | 1,34 | 1,825 |  | -2,53 | -0,58 | 1,25 | 1,89 |  |  |  | -2,83 |  | -0,8 | 1,08 | 1,955 |  |
| 4 | 4 | 320 |  | -3,38 | -0,66 | 1,69 | 2,535 |  | -3,26 | -0,59 | 1,72 | 2,49 |  | -3,53 | -0,75 | 1,63 | 2,58 |  |  |  | -3,8 |  | -0,98 | 1,46 | 2,63 |  |
| 4 | 2 | 640 |  | -5,01 | -0,99 | 2,3 | 3,655 |  | -4,81 | -0,91 | 2,33 | 3,57 |  | -5,2 | -1,07 | 2,25 | 3,725 |  |  |  | -5,57 |  | -1,33 | 2,12 | 3,845 |  |
| 4 | 1 | 1280 |  | -7,5 | -1,69 | 2,97 | 5,235 |  | -7,22 | -1,61 | 3,02 | 5,12 |  | -7,67 | -1,81 | 2,97 | 5,32 |  |  |  | -8,64 |  | -2,04 | 2,79 | 5,715 |  |

The objective is to determine the number of distinct LP-SS based measurement samples that are spaced at equal interval within a period of 1.28s, to normalize the effect of channel fading. As the fading varies slowly in case of static UEs, it is evaluated over a period of 1.28s, which is several times greater than the coherence time of 3kmph UE speed. Thus, the estimated LP-RSRP is a true estimate of path loss, and thus normalizing the effect of fading channel. The following figure illustrates the LP-RSRP estimation error with ideal path loss. It can be seen that LR may need more than four measurements using LP-SS that are spread across 1.28s to obtain a reliable LP-RSRP estimate that characterizes the actual path loss.

Fig. 1 LP-RSRP evaluation over 1.28s using X sample averages on LP-SS in TDL-C 300ns.

The effect of OOK modulation order M is not visible in the LP-RSRP estimation accuracy. Thus, using the above evaluations, we may need four measurements that are equally spaced between 1.28s to obtain a reliable LP-RSRP estimation that limits the error ≤3dB in fading conditions.

Depending on the operating SNR, a LP-RSRP presentative of the pathloss can be obtained using X ≥4 LP-SS samples that are spread over 1.28s at equal interval.

A minimum of X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Our simulations indicate that X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Discussion about coverage

In this section, we will discuss briefly on the required coverage of LP-WUS/LP-SS depending on the past agreements. In the previous RAN1#116/RAN1#116bis, it has been agreed that companies report the target SNR required to match MSG3 coverage.

Agreement For RAN1 evaluation purpose, the SNR to achieve the coverage of PUSCH for message3 is determined for OOK-based LP-WUR and OFDM-based LP-WUR, respectively. - Companies are encouraged to report the SNR, together with the associated assumptions as listed in the table below. Bandwidth for LP-WUS signal (MHz); NF for LP-WUR (dB); Gain of antenna element (dBi) assumed for LP-WUR: e.g., -3 dBi for redcap UE and e.g., 0dBi for non-redcap UE; # of Tx chains for LP-WUS/LP-SS transmission, e.g., 2 Note: The number of Tx chains for LP-WUS/LP-SS transmission is assumed the same as the number of RX chains for MSG3 reception; MIL value of MSG3: taking redcap UE /non-redcap UE @dense urban 2.6GHz; The SNR (dB) to achieve the coverage of PUSCH for message3. Companyname-01. Conclusion: For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

In the table below we provide the corresponding values with PUSCH MSG3 MIL value based on either on our analysis and to the average MIL from Rel-17 for non-RedCap UE:

| LP-WUS BW (MHz) | LR NF (dB) | LR Antenna gain (dBi) | # of DL TX chains | MIL value of MSG3@dense urban 2.6GHz [dB] | SNR required to achieve MSG3 coverage [dB] |
| --- | --- | --- | --- | --- | --- |
| 3.96 | 12 | -3 | 1 | 144.2A | 6.12 |
| 3.96 | 12 | -3 | 1 | 153.51B | -3.19 |
| Note A: Based on company specific evaluation for RedCap UE. Note B: Based on average of Rel-17 Coverage enhancement for non-RedCap UEs |  |  |  |  |  |

It is good to note that when company specific antenna correction factors are used in both UL and DL, it is accounted equally, i.e. could be “compensated”. When the MIL based on companies’ evaluations is used, this may lead to overall increased variance among the SNR values.

Simulations

Simulation assumptions

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | FR1: 2.6 GHz/700MHz |
| System bandwidth | 20/[100] MHz; |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 1 rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30KHz |
| Measurement period (in number of measurement samples) | LP-SS (signal based) OOK based: [5, other number of samples could be studied upon a need] (SSB based) - OFDM based: [5] (other samples if necessary) |
| LP-SS/SSB burst periodicity | LP-SS based: 320 ms OFDM based: 320 ms |
| LP-SS block BW | 132 subcarriers for SCS=30kHz; Note: May need be updated based on RAN1 conclusion |
| Actual LP-SS block transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Regarding the cell specific parameters, it is not clear for us if the cell 1 and cell 2 in simulation parameters refer to a same simulation scenario, or whether the cell 1 and cell 2 are simulated independently from each other. From LP-WUR simulation point of view, our understanding is that the LP-WUR only supports serving cell measurements, and there is no support for neighbouring cell measurements. In the case of MR reselection simulation / test case, there may be a need for another cell, but for LP-WUR it is not clear why a second cell is needed.

Discuss what is the meaning of Cell 1 and Cell 2 in the simulated scenario as the LP-WUR only supports serving cell measurements.

For LP-SS simulations with OOK4, we think that the OOK with M=8 can be removed from the simulation assumptions as there is no RAN1 agreement to support it.

Discuss if M=8 can be removed from the RAN4 simulation assumptions until / if RAN1 agrees to support it.

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| [Overlaid OFDM sequence(s) for LP-SS] Note: up to RAN1 further decision |  |  |  |
| LP-SS (OOK based LP-WUR receiver) |  | OOK-1; OOK-4 with M = 2,4,[8] Note: 8 is up to company’s preference. [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 or NR signal is used for Cell 2 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM [5] ppm OOK: [5] or [10 20] ppm Note: RAN1 may have further conclusion at future meeting | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: company report |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | [CP/2] |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | [3 ms] |
| SNR | dB | Set 1: SNR = [-2.7] Set 2: SNR = [-2.4] | Set 1: SNR = [-11.7] (Relative [9 dB] lower compared with Cell 1) Set 2: SNR = [-8.4] (Relative [6 dB] lower compared with Cell 1) |
| Es/IoT (calculated from SNR) | dB | [-3] | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | FR1: 3 km/h |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |

Consider the simulation assumptions provided in this paper.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Our simulations indicate that X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Proposal 2: Discuss what is the meaning of Cell 1 and Cell 2 in the simulated scenario as the LP-WUR only supports serving cell measurements.

Proposal 3: Discuss if M=8 can be removed from the RAN4 simulation assumptions until / if RAN1 agrees to support it.

Proposal 4: Consider the simulation assumptions provided in this paper.